Copyright 2010 AIA
MasterSpec Full Length
05/10

PRODUCT MASTERSPEC LICENSED BY ARCOM TO CLARKDIETRICH BUILDING SYSTEMS.


This Product MasterSpec Section is licensed by ARCOM for limited distribution by ClarkDietrich Building Systems ("Licensee").  The license is subject to revocation by ARCOM subject to the Product MasterSpec License Agreement between ARCOM and Licensee.

A clean version (revisions accepted) and an underline-and-strikeout version of this Section, both of which modify the original MasterSpec text, are distributed by Licensee to assist in specifying Licensee's products.   Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, ARCOM or The American Institute of Architects (AIA).  Neither AIA nor ARCOM are liable in any way for such revisions or for the use of this Product MasterSpec Section by Licensee or any end user to which Licensee distributes this Product MasterSpec Section.  A qualified design professional should review and edit the document to suit project requirements.

For more information, contact ClarkDietrich Building Systems, 9100 Centre Pointe Dr., Suite 210, West Chester, OH 45069; phone: (513) 870-1100; fax: (513) 870-1300; Website: www.clarkdietrich.com; e-mail: info@clarkdietrich.com.  For information about MasterSpec contact ARCOM at (800) 424-5080 or visit www.arcomnet.com.

Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 054000 - COLD-FORMED METAL FRAMING
Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. Section Includes:

1. Exterior load-bearing wall framing.

2. Interior load-bearing wall framing.

3. Exterior non-load-bearing wall framing.

4. Floor joist framing.

5. Roof rafter framing.

6. Ceiling joist framing.

B. Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

1. Section 054400 "Cold-Formed Metal Trusses" for roof trusses.

2. Section 055000 "Metal Fabrications" for masonry shelf angles and connections.

3. Section 092116.23 "Gypsum Board Shaft Wall Assemblies" for interior non-load-bearing, metal-stud-framed, shaft-wall assemblies.

4. Section 092216 "Non-Structural Metal Framing" for interior non-load-bearing, metal-stud framing and ceiling-suspension assemblies.

1.3 PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

A. Preinstallation Conference:  Conduct conference at Project site.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

1.4 ACTION SUBMITTALS

A. Product Data:  For each type of cold-formed steel framing product and accessory.

B. LEED Submittals:

1. Product Data for Credit MR 4.1[ and Credit MR 4.2]:  For products having recycled content, documentation indicating percentages by weight of postconsumer and preconsumer recycled content.  Include statement indicating cost for each product having recycled content.

2. Product Data for Credit MR 2.1[ and Credit MR 2.2]:  For products diverted from disposal in landfills and incinerators, and where recycled resources are directed back to the manufacturing process.  Include a statement indicating percentage of materials diverted and recycled and the costs associated with each.

3. Product Data for Credit MR 5:  For products where product manufacturing is within a 500 mile radius of the jobsite and the point of extraction of the raw materials.  Include a statement indicating the location and distances for the manufacturing plant and the point of extraction of raw materials in relation to the jobsite location.

ClarkDietrich Engineering Services (CDES) is the largest light-gauge framing engineering company in the United States.  CDES is licensed to perform structural design work in all 50 states and Canada, and provides shop drawings, technical assistance and design support for projects built anywhere in the world.  Specifier may wish to consider requiring the contractor to utilize this service.  If this option is to be a Project requirement, retain optional language in paragraph below and coordinate contract requirements in Division 01.

C. Shop Drawings:  [Provide shop drawings prepared by cold-formed metal framing manufacturer.]

1. Include layout, spacings, sizes, thicknesses, and types of cold-formed steel framing; fabrication; and fastening and anchorage details, including mechanical fasteners.

2. Indicate reinforcing channels, opening framing, supplemental framing, strapping, bracing, bridging, splices, accessories, connection details, and attachment to adjoining work.

3. For cold-formed metal framing indicated to comply with design loads, include structural analysis and detailed shop drawings signed and sealed by a qualified professional engineer, who shall be licensed in the State in which the Project is located and responsible for their preparation.

1.5 INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

A. Qualification Data:  For [professional engineer] [testing agency].

Retain "Welding certificates" Paragraph below if retaining "Welding Qualifications" Paragraph in "Quality Assurance" Article.

B. Welding certificates.

C. Product Test Reports:  From a qualified testing agency, unless otherwise stated, indicating that each of the following complies with the requirements, based on evaluation of comprehensive tests for current products:

1. Steel sheet.

2. Expansion anchors.

3. Power-actuated anchors.

4. Mechanical fasteners.

Insert specific model code organization in "Research Reports" Paragraph below or revise if report must be from another source.

D. Research / Evaluation Reports:  For cold-formed steel framing.

1.

Metal stud manufacturer to have a 3rd party evaluation report for its products that are reviewed to the local building code or its model code (IBC 2006 and AISI NASPEC 2001 with 2004 amendments or IBC 2009 and AISI S100).

1.6 QUALITY ASSURANCE

A. Manufacturer Qualifications:  Member in good standing of Steel Framing Industry Association (SFIA).

1.

Products to be certified under an independent third part inspection program administered by an agency accredited by IAS to ICC-ES AC98 IAS Accreditation Criteria for Inspection Agencies.

B. Engineering Responsibility:  Preparation of Shop Drawings, design calculations, and other structural data by a qualified professional engineer.

C. Professional Engineer Qualifications:  A professional engineer who is legally qualified to practice in jurisdiction where project is located and who is experienced in providing engineering services of the kind indicated.  Engineering services are defined as those performed for installations of cold-formed metal framing that are similar to those indicated for this project in material, design, and extent.

Retain "Testing Agency Qualifications" Paragraph below if Contractor or manufacturer selects testing agency or if Contractor is required to provide services of a qualified testing agency in "Field Quality Control" Article.  Qualification requirements are in addition to those specified in Section 014000 "Quality Requirements," which also defines "NRTL" (nationally recognized testing laboratory).

D. Testing Agency Qualifications:  An independent testing agency, acceptable to authorities having jurisdiction, qualified according to ASTM E 329 for testing indicated.

Usually retain mill certifications or test reports from a qualified testing agency below.  Insert option of test reports from in-house testing with calibrated test equipment if permitted.  Insert option for testing ductility if required.  See "Mill Certification" Paragraph in "Materials" Article in the Evaluations for more information.

E. Product Tests:  Mill certificates or data from a qualified independent testing agency[, or in-house testing with calibrated test equipment] indicating steel sheet complies with requirements, including base-metal thickness, yield strength, tensile strength, total elongation, chemical requirements, and metallic-coating thickness.

Retain "Welding Qualifications" Paragraph below if shop or field welding is required.  If retaining, also retain "Welding certificates" Paragraph in "Informational Submittals" Article.

F. Welding Qualifications:  Qualify procedures and personnel according to the following:

1. AWS D1.1/D1.1M, "Structural Welding Code - Steel".

2. AWS D1.3/D1.3M, "Structural Welding Code - Sheet Steel".

G. Fire-Test-Response Characteristics:  Where indicated, provide cold-formed metal framing identical to that of assemblies tested for fire resistance per ASTM E 119 by, and displaying a classification label from, a testing and inspecting agency acceptable to authorities having jurisdiction.

H. AISI Specifications and Standards: Comply with:

1. AISI S100 “North American Specification for the Design of Cold-Formed Steel Structural Members”.

2. AISI S200 “North American Standard for Cold-Formed Steel Framing – General Provisions”.

3. AISI S201 “North American Standard for Cold-Formed Steel Framing – Product Standard”.

4. AISI S211 “North American Standard for Cold-Formed Steel Framing – Wall Stud Design”.

5. AISI S212 “North American Standard for Cold-Formed Steel Framing – Header Design”.

6. AISI S213 “North American Standard for Cold-Formed Steel Framing – Lateral Design”. 
7. AISI “Code of Standard Practice for Cold-Formed Steel Structural Framing”.
1.7 DELIVERY, STORAGE, AND HANDLING

A. Protect and store cold-formed steel framing from corrosion, moisture staining, deformation, and other damage during delivery, storage, and handling as required in AISI's "Code of Standard Practice".

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products.  For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 MANUFACTURERS

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Basis-of-Design Product:  Subject to compliance with requirements, provide cold-formed metal framing products manufactured by ClarkDietrich Building Systems specified in other Part 2 articles, or comparable products from members of the SFIA.
1. <Insert manufacturer's name>.

2.2 PERFORMANCE REQUIREMENTS

Retain "Delegated Design," "Structural Performance," and "Cold-Formed Steel Framing Design Standards" paragraphs below if Contractor is required to assume responsibility for design.

Coordinate with other Part 2 articles, deleting prescriptive requirements such as steel thickness and minimum yield strength unless imposing minimum design restrictions.  Insert other performance and design criteria below to suit Project or add to Drawings.  AIA Document A201 requires Owner or Architect to specify performance and design criteria to be satisfied.

A. Delegated Design:  Engage a qualified professional engineer, as defined in Section 014000 "Quality Requirements," to design cold-formed steel framing.

B. Structural Performance:  Provide cold-formed steel framing capable of withstanding design loads within limits and under conditions indicated.

1. Design Loads:  [As indicated] <Insert design loads>.

a. Dead Loads:  [Weights of materials and construction] <Insert applicable dead loads>.

b. Live Loads:  <Insert applicable live loads>.

c. Roof Loads:  <Insert applicable roof loads>.

d. Snow Loads:  <Insert applicable snow loads>.

e. Wind Loads:  <Insert applicable wind loads>.

f. Seismic Loads:  <Insert applicable seismic loads>. 
g. Building Code Requirements:  <Insert specific requirements of code authority having jurisdiction>.

2. Deflection Limits:  Design framing systems to withstand design loads without deflections greater than the following:

Component deflection limits in first six subparagraphs below are examples only.  Retain deflection limits from applicable subparagraphs or insert other limits as appropriate for wall, floor, and ceiling finish materials.

a. Exterior Load-Bearing Wall Framing:  Horizontal deflection of [1/240] [1/360] [1/600] [1/720] <Insert ratio> of the wall height.

b. Interior Load-Bearing Wall Framing:  Horizontal deflection of [1/240] [1/360] <Insert ratio> of the wall height under a horizontal load of 5 lbf/sq. ft. (239 Pa).

c. Exterior Non-Load-Bearing Framing:  Horizontal deflection of [1/240] [1/360] [1/600] [1/720] <Insert ratio> of the wall height.

d. Floor Joist Framing:  Vertical deflection of 1/480 for live loads and 1/360  for total loads of the span.

e. Roof Rafter Framing:  Vertical deflection of [1/240] [1/360] of the horizontally projected span.

f. Ceiling Joist Framing:  Vertical deflection of [1/240] [1/360] of the span.

g. <Insert deflection limits of any special construction>.

3. Design framing systems to provide for movement of framing members without damage or overstressing, sheathing failure, connection failure, undue strain on fasteners and anchors, or other detrimental effects when subject to a maximum ambient temperature change of 120 deg F (67 deg C).

Retain first subparagraph below for non-load-bearing and load-bearing walls if design responsibility for cold-formed framing is assigned to Contractor.  Indicate locations on Drawings if different movement is anticipated for different building elements.  If preferred, change deflection limits to ratios such as L/300 for floors and L/200 for roofs.

4. Design framing system to maintain clearances at openings, to allow for construction tolerances, and to accommodate live load deflection of primary building structure as follows:

a. Upward and downward movement of [1/2 inch (13 mm)] [3/4 inch (19 mm)] [1 inch (25 mm)] [1-1/2 inches (38 mm)].

Usually retain subparagraph below for exterior non-load-bearing wall framing, particularly for brick-veneer backup framing.

5. Design exterior non-load-bearing wall framing to accommodate horizontal deflection without regard for contribution of sheathing materials.

C. Cold-Formed Steel Framing Design Standards:  Design according to AISI's S100 “North American Specification for the Design of Cold-Formed Steel Structural Members”.

Retain subparagraphs below, as applicable, if Project includes floor and roof systems, wall studs, headers for load-bearing walls, or lateral design requirements.

1. AISI S200 “North American Standard for Cold-Formed Steel Framing – General Provisions”.

2. AISI S201 “North American Standard for Cold-Formed Steel Framing – Product Standard”.

3. AISI S211 “North American Standard for Cold-Formed Steel Framing – Wall Stud Design”.

4. AISI S212 “North American Standard for Cold-Formed Steel Framing – Header Design”.

5. AISI S213 “North American Standard for Cold-Formed Steel Framing – Lateral Design”.

6. AISI “Code of Standard Practice for Cold-Formed Steel Structural Framing”.
2.3 COLD-FORMED STEEL FRAMING, GENERAL

Retain this article to supplement specific framing articles that follow.

Retain "Recycled Content of Steel Products" Paragraph below if required for LEED Credit MR 4.  USGBC allows a default value of 25 percent to be used for steel, without documentation; higher percentages can be claimed if they are supported by appropriate documentation.  The Steel Recycling Institute indicates that steel sheet typically has 23 percent postconsumer recycled content and 1.5 percent preconsumer recycled content.

A. Recycled Content of Steel Products:  Provide products with an average recycled content of steel products so postconsumer recycled content plus one-half of preconsumer recycled content not less than [25] <Insert number> percent.

B. Steel Sheet:  ASTM A 1003/A 1003M, Structural Grade, Type H, metallic coated, of grade and coating weight as follows:

Retain minimum grade requirements from options in "Grade" Subparagraph below.  If more than one grade option is required, indicate location of each on Drawings.

1. Grade:  [ST33H (ST230H)] [ST50H (ST340H)] [As required by structural performance] <Insert grade>.

Minimum coating requirement for Structural Grade, Type H steel is G60 (Z180).  Retain first option in "Coating" Subparagraph below if ASTM A 1003/A 1003M's listing of minimum coating thicknesses is required.  This minimum coating designation assumes normal exposure conditions and construction practices.  When more severe exposure conditions are probable, for example, in coastal areas, consider specifying a heavier coating.  BIA recommends G90 (Z275) coating for stud backup applications.  Verify availability of heavier-coated steel.  If more than one coating option is required, indicate location of each on Drawings.

2. Coating:  [G60 (Z180), A60 (ZF180), AZ50 (AZ150), or GF30 (ZGF90)] [G90 (Z275)] <Insert coating weight>.

Retain "Steel Sheet for (Vertical Deflection) (Drift) Clips" Paragraph below if applicable.

C. Steel Sheet for [Vertical Deflection] [Drift] Clips:  ASTM A 1003/A 1003M, A 653/A 653M, structural steel, zinc coated, of grade and coating as follows:

Retain grade requirements from options in "Grade" Subparagraph below.

1. Grade:  [50 (340)] [As required by structural performance].

2. Coating:  G90 (Z275).

2.4 LOAD-BEARING WALL FRAMING

Retain this article if load-bearing steel studs are required.  Distinguish location of axial load-bearing framing from exterior non-load-bearing wall framing here or on Drawings.  Indicate stud and track web depth on Drawings or insert here.

A. Steel Studs:  Manufacturer's standard C-shaped steel studs, of web depths indicated, punched, with stiffened flanges, and as follows:

1. Basis-of-Design Product:  ClarkDietrich Building Systems.

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.  If retaining different thicknesses for different components, indicate locations here or in framing articles below.

2. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)].

Flange widths vary with application.  If sheathing or masonry ties are required, consider minimum flange width of 1-5/8 inches (41 mm).  Sequence corresponds to common flange width designators 137, 162, 200, and 250.

3. Flange Width:  [1-3/8 inches (35 mm)] [1-5/8 inches (41 mm)] [2 inches (51 mm)] [2-1/2 inches (63 mm)].

Retain "Section Properties" Subparagraph below if not assigning design responsibility to Contractor.  If retaining, indicate whether design is based on gross or effective section properties.

4. Section Properties:  <Insert minimum allowable calculated section modulus, moment of inertia, and allowable moment>.

B. Steel Track:  Manufacturer's standard U-shaped steel track, of web depths indicated, unpunched, with straight flanges, and as follows:

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.

1. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)] [Matching steel studs].

2. Flange Width:  [1-1/4 inches (32 mm)] <Insert dimension if manufacturer's standard width is insufficient>.

C. Headers and Jambs - Heavy-Duty Stud:  Manufacturer's proprietary shape used to form header beams and jambs, columns or posts, of web depths indicated, unpunched, with stiffened flanges and as follows:

1. Basis-of-Design Product:  ClarkDietrich Building Systems; [Heavy Duty Stud (HDS)] [and] [Header Bracket (HDSC)] [ProX Header] and [RedHeader RO].
Select one steel thickness from first subparagraph below or revise to suit Project.  Sequence corresponds to new common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.

2. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)] [Matching steel studs].

Retain one or both subparagraphs below to suit Project requirements.

3. Web and Flange Widths, Type HDS:  [3-5/8 by 3 by 1-1/16 by 3/4 inch (92.1 by 76.2 by 27.0 by 19.1 mm)] [6 by 3 by 2-1/4 by 3/4 inch (152 by 76.2 by 57.2 by 19.1 mm)].

4. Web and Flange Widths, Type HDSC:  [3-1/2 by 3-1/16 by 2 inches (88.9 by 77.8 by 50.8 mm)] [5-7/8 by 3-1/16 by 2 inches (149 by 77.8 by 50.8 mm)].

D. Steel Box or Back-to-Back Headers:  Manufacturer's standard C-shapes used to form header beams, of web depths indicated, punched, with stiffened flanges, and as follows:

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.  If retaining different thicknesses for different locations, indicate locations here or in the framing articles below.

1. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)].

Flange widths vary with application; coordinate with wall width.

2. Flange Width:  [1-3/8 inches (35 mm)] [1-5/8 inches (41 mm)] [2 inches (51 mm)] [2-1/2 inches (63 mm)].

Retain "Section Properties" Subparagraph below if not assigning design responsibility to Contractor.  If retaining, indicate whether design is based on gross or effective section properties.

3. Section Properties:  <Insert minimum allowable calculated section modulus, moment of inertia, and allowable moment>.

E. Steel L Headers:  Manufacturer's standard L-shapes used to form header beams, of web depths indicated, and as follows:

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 43, 54, 68, and 97 and obsolete 18-, 16-, 14-, and 12-gage designations.  If retaining different thicknesses for different locations, indicate locations here or in framing articles below.

1. Minimum Base-Metal Thickness:  [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)].

2. Leg Width:  1-1/2 inches (38 mm) by Leg Length [6 inches (152 mm)] [8 inches (203 mm)] [10 inches (254 mm)].

Retain "Section Properties" Subparagraph below if not assigning design responsibility to Contractor.  If retaining, indicate whether design is based on gross or effective section properties.

3. Section Properties:  <Insert minimum allowable calculated section modulus, moment of inertia, and allowable moment>.

F. Opening Framing:

ClarkDietrich Building Systems, RedHeader RO System is designed to provide an opening alternative system for conventional interior framing and/or exterior curtain wall built-up header and jamb framing by reducing labor and components up to 50 percent. 

1. Exterior systems are subject to requirements of code evaluation reports and/or shop drawings with engineering calculations signed and sealed by a registered PE (Professional Engineer). 

2. Minimum Material Thickness: As required by design. 

3.
Minimum Flange Width: As required by design.

2.5 EXTERIOR NON-LOAD-BEARING WALL FRAMING

Retain this article if exterior non-load-bearing wall framing, sometimes called "curtain-wall" framing by manufacturers, is required.  Indicate stud and track web depth on Drawings or insert here.

A. Steel Studs:  Manufacturer's standard C-shaped steel studs, of web depths indicated, punched, with stiffened flanges, and as follows:

1. Basis-of-Design Product:  ClarkDietrich Building Systems.

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.

2. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)].

Flange widths vary with application.  If sheathing or masonry ties are required, consider minimum flange width of 1-5/8 inches (41 mm).  Sequence corresponds to common flange width designators 137, 162, 200, and 250.

3. Flange Width:  [1-3/8 inches (35 mm)] [1-5/8 inches (41 mm)] [2 inches (51 mm)] [2-1/2 inches (63 mm)].

Retain "Section Properties" Subparagraph below if not assigning design responsibility to Contractor.  If retaining, indicate whether design is based on gross or effective section properties.

4. Section Properties:  <Insert minimum allowable calculated section modulus, moment of inertia, and allowable moment>.

B. Steel Track:  Manufacturer's standard U-shaped steel track, of web depths indicated, unpunched, with unstiffened flanges, and as follows:

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.

1. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)] [Matching steel studs].

2. Flange Width:  [1-1/4 inches (32 mm)] <Insert dimension if manufacturer's standard width is insufficient>.

Retain "Vertical Deflection Clips," "Single Deflection Track," or "Double Deflection Tracks" Paragraph below for components to cope with vertical deflection of the primary structure.

Retain "Vertical Deflection Clips" Paragraph below if required; revise to add displacement if deflection of primary structure is not indicated or if "Performance Requirements" Article is deleted.

C. Headers and Jambs - Heavy-Duty Stud:  Manufacturer's proprietary shape used to form header beams and jambs, columns or posts, of web depths indicated, unpunched, with stiffened flanges and as follows:

1. Basis-of-Design Product:  ClarkDietrich Building Systems; [HDS Heavy Duty Stud] [and] [HDSC Header Bracket] [ProX Header] [RedHeader RO].
Select one steel thickness from first subparagraph below or revise to suit Project.  Sequence corresponds to new common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.

2. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)] [Matching steel studs].

3. Web and Flange Widths, Type HDS:  [3-5/8 by 3 by 1-1/16 by 3/4 inch (92.1 by 76.2 by 27.0 by 19.1 mm)] [6 by 3 by 2-1/4 by 3/4 inch (152 by 76.2 by 57.2 by 19.1 mm)].

4. Web and Flange Widths, Type HDSC:  [3-1/2 by 3-1/16 by 2 inches (88.9 by 77.8 by 50.8 mm)] [5-7/8 by 3-1/16 by 2 inches (149 by 77.8 by 50.8 mm)].

D. Vertical Deflection Clips:  Manufacturer's standard [bypass] [head] clips, capable of accommodating upward and downward vertical displacement of primary structure through positive mechanical attachment to stud web and capable of resisting forces imposed by the wall system.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Basis-of-Design Product:  Subject to compliance with requirements, provide ClarkDietrich Building Systems; [FCSC] [FS12] [FS15] [FS24] [QC] [SD], or a comparable product by one of the following:

a. Simpson Strong-Tie.

b. Steel Network, Inc. (The).

c. <Insert manufacturer's name>.

2. Minimum Base-Metal Thickness:  [Fast Clip Slide Clip; 0.0677 inch (1.72 mm)] [Fast Strut Clip; 0.0677 inch (1.72 mm)] [Quick Clip; 0.1180 inch (3 mm)] [Slide Clip; 0.0677 inch (1.72 mm)] [Slide Clip; 0.0966 inch (2.45 mm)].

E. Slotted Deflection Track:  Manufacturer’s single, deep-leg, U-shaped steel track; punched with vertical slots in both legs.  Studs should be positively attached to deep-leg track using vertical slots while allowing free vertical movement.  Legs designed to support horizontal and lateral loads and transfer them to the primary structure, as follows:

1. Basis-of-Design Product:  ClarkDietrich Building Systems; [MaxTrak] [SLP-TRK] Slotted Deflection Track.
2. Standard Leg:  2-1/2 inches (63.5 mm).

3. Standard Vertical Slot in Leg:  [1-1/2 inches (38.1 mm)] [2-1/2 inches (63.5 mm)].
4. Minimum Yield Strength and Thickness:  [33 ksi: 0.0329 inch (0.84 mm)] [33 ksi; 0.0428 inch (1.09 mm)] [50 ksi; 0.0538 inch (1.37 mm)] [50 ksi; 0.0677 inch (1.72 mm)].

Retain "Single Deflection Track" Paragraph below if required.  Revise description, as applicable, if limiting types of single-leg track.

F. Single Deflection Track:  Manufacturer's single, deep-leg, U-shaped steel track; unpunched, with unstiffened flanges, of web depth to contain studs while allowing free vertical movement, with flanges designed to support horizontal  and lateral loads and transfer them to the primary structure.  Install a continuous row of bridging, composed of 1-1/2 inch (38 mm) cold-formed channel secured to each stud with clip angle or Spazzer 5400, at upper-most knockout, not more than 12 inches (305 mm) from top of wall.

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 43, 54, 68, and 97 and obsolete 18-, 16-, 14-, and 12-gage designations.  Deflection track flange is usually designed to be thicker than corresponding studs to resist transverse loading.  Coordinate flange width with deflection of primary structure to ensure that structure does not bear on framing.

1. Basis-of-Design: ClarkDietrich Building Systems or a comparable product by one of the members of the SFIA.
a. <Insert manufacturer's name>.

2. Minimum Base-Metal Thickness:  [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)].

3. Flange Width:  [1 inch (25 mm) plus the design gap for one-story structures] [and] [1 inch (25 mm) plus twice the design gap for other applications] <Insert dimension>.

G. U-Channel:  Manufacturer’s standard length.

1. Basis-of-Design Product: ClarkDietrich Building Systems; U-Channel.

2. Size:  1-1/2 inches (38.1 mm).

3. Minimum Base-Metal Thickness:  0.0538 inch (1.37 mm).

H. Bridging and Spacer Bar:

1. Basis-of Design-Product:  ClarkDietrich Building Systems; TradeReady Spazzer 5400 (SPZS) [Spazzer Bar Guard (SPBG)].

2. Minimum Base-Metal Thickness:  0.0538 inch (1.37 mm).

3. Size:  1-1/4 by 1-1/4 by 50 inches (32 by 32 by 1270 mm) long, pre-notched at 12, 16 and 24 inches (305, 406, and 610 mm) centers.

I. Double Deflection Tracks:  Manufacturer's double, deep-leg, U-shaped steel tracks, consisting of nested inner and outer tracks; unpunched, with unstiffened flanges.

Retain one steel thickness from "Outer Track" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 43, 54, 68, and 97 and obsolete 18-, 16-, 14-, and 12-gage designations.  Deflection track flange is usually designed to be thicker than corresponding studs to resist transverse loading.  Coordinate flange width with deflection of primary structure to ensure that structure does not bear on framing.

1. Outer Track:  Of web depth to allow free vertical movement of inner track, with flanges designed to support horizontal and lateral loads and transfer them to the primary structure, and as follows:

a. Minimum Base-Metal Thickness:  [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)].

b. Flange Width:  [1 inch (25 mm) plus the design gap for one-story structures] [and] [1 inch (25 mm) plus twice the design gap for other applications] <Insert dimension>.

2. Inner Track:  Of web depth indicated, and as follows:

a. Minimum Base-Metal Thickness:  [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)].

b. Flange Width:  <Insert dimension equal to sum of outer deflection track flange width plus 1 inch (25 mm)>.

Retain "Drift Clips" Paragraph below if drift clips are required to accommodate horizontal and vertical deflection of the primary structure.

J. Drift Clips:  Manufacturer's standard bypass or head clips, capable of isolating wall stud from upward and downward vertical displacement and lateral drift of primary structure.

2.6 FLOOR JOIST FRAMING

Retain this article if steel floor joists are required.  If joist and joist-track depth are not indicated, revise paragraphs below and insert depth required.

A. Steel Floor Joists:  Manufacturer's proprietary cold-formed galvanized steel joists, of web depths indicated, featuring large extruded holes and as follows:

1. Basis-of-Design Product:  ClarkDietrich Building Systems; TradeReady Steel Joists.

2. Minimum Base-Metal Thickness:  [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0966 inch (2.45 mm)].

B. Steel Joist Rim Track:  Manufacturer’s standard U-shaped steel joist track; punched with clip angles at required joist spacing, of web depths indicated; with stiffened webs and as follows:

1. Minimum Base-Metal Thickness:  [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)] [Matching steel joists].

2. Flange Width:  1-1/4 inches (31.8 mm) top, 2-1/2 inches (63.4 mm) bottom minimum.

Retain paragraph below if steel joists are required.  If joist and joist-track depth are not indicated, revise paragraphs below and add depth required.

C. Steel Joists:  Manufacturer's standard C-shaped steel joists, of web depths indicated, [unpunched,] [punched,] with stiffened flanges, and as follows:

1. Basis-of-Design Product:  ClarkDietrich Building Systems: [As indicated on Drawings] <Insert product description>.

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.

2. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)].

Retain one flange width from "Flange Width" Subparagraph below.  Flange widths may vary with application.  Sequence corresponds to common flange width designators 162, 200, and 250.

3. Flange Width:  [1-5/8 inches (41 mm)] [2 inches (51 mm)] [2-1/2 inches (63 mm)], minimum.

Retain "Section Properties" Subparagraph below if not assigning design responsibility to Contractor.  If retaining, indicate whether design is based on gross or effective section properties.

4. Section Properties:  <Insert minimum allowable calculated section modulus, moment of inertia, and allowable moment>.

Revise "Steel Joist Track" Paragraph below to include descriptions of other related components, such as headers or other built-up members, at openings or other terminations.

D. Steel Joist Track:  Manufacturer's standard U-shaped steel joist track, of web depths indicated, unpunched, with unstiffened flanges, and as follows:

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.

1. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)] [Matching steel joists].

2. Flange Width:  [1-1/4 inches (32 mm)] <Insert dimension if manufacturer's standard flange width is different>.

2.7 ROOF-RAFTER FRAMING

Retain this article if steel rafters are required.  If rafter web depth is not indicated, revise "Steel Rafters" Paragraph below and insert depth required.

A. Steel Rafters:  Manufacturer's standard C-shaped steel sections, of web depths indicated, with stiffened flanges, and as follows:

1. Basis-of-Design Product:  ClarkDietrich Building Systems.

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.

2. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)].

Retain one flange width from "Flange Width" Subparagraph below.  Flange widths may vary with application.  Sequence corresponds to common flange width designators 162, 200, and 250.

3. Flange Width:  [1-5/8 inches (41 mm)] [2 inches (51 mm)] [2-1/2 inches (63 mm)], minimum.

Retain "Section Properties" Subparagraph below if not assigning design responsibility to Contractor.  If retaining, indicate whether design is based on gross or effective section properties.

4. Section Properties:  <Insert minimum allowable calculated section modulus, moment of inertia, and allowable moment>.

B. Built-up Members:  Built-up members of manufacturer's standard C-shaped steel section with stiffened flanges, nested into a U-shaped steel section joist track, with unstiffened flanges; [unpunched] [punched with large service holes] of web depths indicated; and as follows:

1. Minimum Base-Steel Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)] [Matching steel rafters].

2.

Flange Width:  [1-5/8 inches (41 mm)] [2 inches (51 mm)] [2-1/2 inches (63 mm)], minimum.

2.8 CEILING JOIST FRAMING

Retain this article if steel joists are required.  If joist and joist-track depth is not indicated, revise "Steel Ceiling Joists" Paragraph below and insert depth required.

A. Steel Ceiling Joists:  Manufacturer's [standard C-shaped steel sections] [proprietary joist system], of web depths indicated, [unpunched,] [punched with enlarged service holes,] with stiffened flanges, and as follows:

1. Basis-of-Design Product:  ClarkDietrich Building Systems.

Retain one steel thickness from "Minimum Base-Metal Thickness" Subparagraph below or revise to suit Project.  Sequence corresponds to common thickness designators 33, 43, 54, 68, and 97 and obsolete 20-, 18-, 16-, 14-, and 12-gage designations.

2. Minimum Base-Metal Thickness:  [0.0329 inch (0.84 mm)] [0.0428 inch (1.09 mm)] [0.0538 inch (1.37 mm)] [0.0677 inch (1.72 mm)] [0.0966 inch (2.45 mm)].

Retain one flange width from "Flange Width" Subparagraph below.  Flange widths may vary with application.  Sequence corresponds to common flange width designators 162, 200, and 250.

3. Flange Width:  [1-5/8 inches (41 mm)] [1-3/4 inches (44.5 mm)] [2 inches (51 mm)] [2-1/2 inches (63 mm)], [1-1/4 inches (31.8 mm) top, 1-5/8 inches (63.4 mm) bottom] minimum.

Retain "Section Properties" Subparagraph below if not assigning design responsibility to Contractor.  If retaining, indicate whether design is based on gross or effective section properties.

4. Section Properties:  <Insert minimum allowable calculated section modulus, moment of inertia, and allowable moment>.

2.9 FRAMING ACCESSORIES

Revise minimum yield strength of accessories as required.

A. Fabricate steel-framing accessories from steel sheet, ASTM A 1003/A 1003M, Structural Grade, Type H, metallic coated, of same grade and coating weight used for framing members.

B. Provide accessories of manufacturer's standard thickness and configuration, unless otherwise indicated, as follows:

Revise list below to suit Project.

1. Supplementary framing.

2. Bracing, bridging, and solid blocking.

a. Basis-of-Design Product:  ClarkDietrich Building Systems; [Spazzer 5400 Bridging Bar (SPZS)] [Spazzer Bar Guard (SPBG)].

3. Web stiffeners.

a. Basis-of-Design Product:  ClarkDietrich Building Systems; QTWS.

4. Anchor clips.

5. End clips.

6. Foundation clips.

7. Gusset plates.

8. Stud kickers and knee braces.

9. Joist hangers and end closures.

10. Hole reinforcing plates.

11. Backer plates.

2.10 ANCHORS, CLIPS, AND FASTENERS

Delete "Steel Shapes and Clips" Paragraph below if rolled steel shapes and clips are not required or are specified in another Section.

A. Steel Shapes and Clips:  ASTM A 36/A 36M, zinc coated by hot-dip process according to ASTM A 123/A 123M.

Retain grade of anchor bolt, head type, and type of protective coating from "Anchor Bolts" Paragraph below.  Revise if using high-strength, low-alloy anchor bolts.

B. Anchor Bolts:  ASTM F 1554, [Grade 36] [Grade 55], threaded carbon-steel [hex-headed bolts] [headless, hooked bolts] [headless bolts, with encased end threaded,] and carbon-steel nuts; and flat, hardened-steel washers; zinc coated by [hot-dip process according to ASTM A 153/A 153M, Class C] [mechanically deposition according to ASTM B 695, Class 50].

Retain "Expansion Anchors" Paragraph below if expansion anchors are acceptable.  Verify safety factor with Project's structural engineer.  Revise as required or insert specific load requirements and names of acceptable products.

C. Expansion Anchors:  Fabricated from corrosion-resistant materials, with capability to sustain, without failure, a load equal to 5 times design load, as determined by testing per ASTM E 488 conducted by a qualified testing agency.

Retain "Power-Actuated Anchors" Paragraph below if power-actuated fasteners are acceptable.  Verify safety factor with Project's structural engineer.

D. Power-Actuated Anchors:  Fastener system of type suitable for application indicated, fabricated from corrosion-resistant materials, with capability to sustain, without failure, a load equal to 10 times design load, as determined by testing per ASTM E 1190 conducted by a qualified testing agency.

E. Mechanical Fasteners:  ASTM C 1513, corrosion-resistant-coated, self-drilling, self-tapping, steel drill screws.

1. Head Type:  Low-profile head beneath sheathing, manufacturer's standard elsewhere.

F. Welding Electrodes:  Comply with AWS standards.

2.11 MISCELLANEOUS MATERIALS

A. Galvanizing Repair Paint:  [SSPC-Paint 20 or DOD-P-21035] [ASTM A 780].

Retain appropriate grout from "Cement Grout" and "Nonmetallic, Nonshrink Grout" paragraphs below if concrete or masonry substrates require leveling before setting track or prefabricated assemblies.

B. Cement Grout:  Portland cement, ASTM C 150, Type I; and clean, natural sand, ASTM C 404.  Mix at ratio of 1 part cement to 2-1/2 parts sand, by volume, with minimum water required for placement and hydration.

C. Nonmetallic, Nonshrink Grout:  Premixed, nonmetallic, noncorrosive, nonstaining grout containing selected silica sands, portland cement, shrinkage-compensating agents, and plasticizing and water-reducing agents, complying with ASTM C 1107/C 1107M, with fluid consistency and 30-minute working time.

D. Shims:  Load bearing, high-density multimonomer plastic, nonleaching.

E. Sealer Gaskets:  Closed-cell neoprene foam, 1/4 inch (6.4 mm) thick, selected from manufacturer's standard widths to match width of bottom track or rim track members.

2.12 FABRICATION

Retain this article for examples of shop or field fabrication including panelized load-bearing and non-load-bearing walls.

A. Fabricate cold-formed steel framing and accessories plumb, square, and true to line, and with connections securely fastened, according to referenced AISI's specifications and standards, manufacturer's written instructions, and requirements in this Section.

1. Fabricate framing assemblies using jigs or templates.

2. Cut framing members by sawing or shearing; do not torch cut.

3. Fasten cold-formed steel framing members by welding, screw fastening, clinch fastening, pneumatic pin fastening, or riveting as standard with fabricator.  Wire tying of framing members is not permitted.

a. Comply with AWS D1.3/D1.3M requirements and procedures for welding, appearance and quality of welds, and methods used in correcting welding work.

b. Locate mechanical fasteners and install according to Shop Drawings, with screw penetrating joined members by no fewer than three exposed screw threads.

Retain subparagraph below if fabricated assemblies include other materials or revise to suit Project.

4. Fasten other materials to cold-formed steel framing by welding, bolting, or screw fastening, according to Shop Drawings.

B. Reinforce, stiffen, and brace framing assemblies to withstand handling, delivery, and erection stresses.  Lift fabricated assemblies to prevent damage or permanent distortion.

Revise "Fabrication Tolerances" Paragraph below to suit Project.

C. Fabrication Tolerances:  Fabricate assemblies level, plumb, and true to line to a maximum allowable tolerance variation of 1/8 inch in 10 feet (1:960) and as follows:

1. Spacing:  Space individual framing members no more than plus or minus 1/8 inch (3 mm) from plan location.  Cumulative error shall not exceed minimum fastening requirements of sheathing or other finishing materials.

2. Squareness:  Fabricate each cold-formed steel framing assembly to a maximum out-of-square tolerance of 1/8 inch (3 mm).

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine supporting substrates and abutting structural framing for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

B. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 PREPARATION

Retain first two paragraphs below if sprayed fire-resistive materials are required.

A. Before sprayed fire-resistive materials are applied, attach continuous angles, supplementary framing, or tracks to structural members indicated to receive sprayed fire-resistive materials.

B. After applying sprayed fire-resistive materials, remove only as much of these materials as needed to complete installation of cold-formed framing without reducing thickness of fire-resistive materials below that are required to obtain fire-resistance rating indicated.  Protect remaining fire-resistive materials from damage.

Retain first paragraph below if bottom track of load-bearing walls requires a uniform-bearing surface on concrete or masonry construction.

C. Install load bearing shims or grout between the underside of wall bottom track or rim track and the top of foundation wall or slab at stud or joist locations to ensure a uniform bearing surface on supporting concrete or masonry construction.

Retain paragraph below if sealing the gap between framing and concrete or masonry construction.

D. Install sealer gaskets at the underside of wall bottom track or rim track and at the top of foundation wall or slab at stud or joist locations.

3.3 INSTALLATION, GENERAL

Retain this article with each specific framing classification in articles elsewhere below.

A. Install cold-formed framing in accordance with ASTM C1007 and AISI S200 "North American Standard for Cold-Formed Steel Framing - General Provisions".

B. Cold-formed steel framing may be shop or field fabricated for installation, or it may be field assembled.

C. Install cold-formed steel framing according to AISI's "Standard for Cold-Formed Steel Framing - General Provisions" and to manufacturer's written instructions unless more stringent requirements are indicated.

Retain first paragraph below for shop- or field-fabricated wall panels.

D. Install shop- or field-fabricated, cold-formed framing and securely anchor to supporting structure.

1. Screw, bolt, or weld wall panels at horizontal and vertical junctures to produce flush, even, true-to-line joints with maximum variation in plane and true position between fabricated panels not exceeding 1/16 inch (1.6 mm).

E. Install cold-formed steel framing and accessories plumb, square, and true to line, and with connections securely fastened.

1. Cut framing members by sawing or shearing; do not torch cut.

2. Fasten cold-formed steel framing members by welding, screw fastening, clinch fastening, or riveting.  Wire tying of framing members is not permitted.

a. Comply with AWS D1.3/D1.3M requirements and procedures for welding, appearance and quality of welds, and methods used in correcting welding work.

b. Locate mechanical fasteners and install according to Shop Drawings, and complying with requirements for spacing, edge distances, and screw penetration.

F. Install framing members in one-piece lengths unless splice connections are indicated for track or tension members.

G. Install temporary bracing and supports to secure framing and support loads comparable in intensity to those for which structure was designed.  Maintain braces and supports in place, undisturbed, until entire integrated supporting structure has been completed and permanent connections to framing are secured.

Include details on Drawings showing expansion-joint construction and locations.

H. Do not bridge building expansion and control joints with cold-formed steel framing.  Independently frame both sides of joints.

Retain first paragraph below if insulation for exterior framing is required.

I. Install insulation, specified in Section 072100 "Thermal Insulation," in built-up exterior framing members, such as headers, sills, boxed joists, and multiple studs at openings, that are inaccessible on completion of framing work.

J. Fasten hole reinforcing plate over web penetrations that exceed size of manufacturer's standard punched openings.

Revise "Erection Tolerances" Paragraph below to suit Project.  Coordinate with limitations of subsequent finish materials.

K. Erection Tolerances:  Install cold-formed steel framing level, plumb, and true to line to a maximum allowable tolerance variation of 1/8 inch in 10 feet (1:960) and as follows:

1. Space individual framing members no more than plus or minus 1/8 inch (3 mm) from plan location.  Cumulative error shall not exceed minimum fastening requirements of sheathing or other finishing materials.

3.4 LOAD-BEARING WALL INSTALLATION

If this article is required, retain with "Installation, General" Article.

A. Install continuous top and bottom tracks sized to match studs.  Align tracks accurately and securely anchor at corners and ends, and at spacings as follows:

Retain option in "Anchor Spacing" Subparagraph below to suit requirements and type of anchor.

1. Anchor Spacing:  [24 inches (610 mm)] [32 inches (813 mm)] [To match stud spacing] [As shown on Shop Drawings].

B. Squarely seat studs against top and bottom tracks with gap not exceeding of 1/8 inch (3 mm) between the end of wall framing member and the web of track.  Fasten both flanges of studs to top and bottom tracks.  Space studs as follows:

Retain from options in first "Stud Spacing" Subparagraph below for IP spacings and equivalent SI (metric) dimensions.

1. Stud Spacing:  [12 inches (305 mm)] [16 inches (406 mm)] [19.2 inches (488 mm)] [24 inches (610 mm)] [As indicated].

C. Set studs plumb, except as needed for diagonal bracing or required for nonplumb walls or warped surfaces and similar configurations.

D. Align studs vertically where floor framing interrupts wall-framing continuity.  Where studs cannot be aligned, continuously reinforce track to transfer loads.

E. Align floor and roof framing over studs.  Where framing cannot be aligned, continuously reinforce track to transfer loads.

F. Anchor studs abutting structural columns or walls, including masonry walls, to supporting structure as indicated.

G. Install headers over wall openings wider than stud spacing.  Locate headers above openings as indicated.  Fabricate headers of [compound shapes] [Heavy Duty Stud (HDS) components] [ProX Header] [RedHeader RO System components] indicated or required to transfer load to supporting studs, complete with clip-angle connectors, web stiffeners, or gusset plates.

1. Frame wall openings with not less than a double stud at each jamb of frame as indicated on Shop Drawings.  Fasten jamb members together to uniformly distribute loads.  A single proprietary jamb member designed specifically for the purpose of supporting the header may be used in lieu of multiple members.

2. Install runner tracks and jack studs above and below wall openings.  Anchor tracks to jamb studs with clip angles or by welding, and space jack studs same as full-height wall studs.

H. Install supplementary framing, blocking, and bracing in stud framing indicated to support fixtures, equipment, services, casework, heavy trim, furnishings, and similar work requiring attachment to framing.

1. If type of supplementary support is not indicated, comply with stud manufacturer's written recommendations and industry standards in each case, considering weight or load resulting from item supported.

Retain spacing of bridging from options in first paragraph below or revise to suit Project.

I. Install horizontal bridging in stud system, spaced vertically [48 inches (1220 mm)] [as indicated] [as indicated on Shop Drawings].  Fasten at each stud intersection.

Retain type of bridging required from three "Bridging" subparagraphs below or revise to suit Project.  Insert locations if more than one type of bridging is required.  Insert minimum size of flat steel strap, such as 1-1/2 by 0.0329 inch (38 by 0.84 mm), if default size is required.

1. Bridging:  Steel channel, welded or mechanically fastened to webs of punched studs with a minimum of two screws into each flange of the clip angle for framing members up to 6 inches (152.4 mm) deep.

2. Bridging:  Combination of flat, taut, steel sheet straps of width and thickness indicated and stud-track solid blocking of width and thickness to match studs.  Fasten flat straps to stud flanges and secure solid blocking to stud webs or flanges.

3. Bridging:  Proprietary bridging bars installed according to manufacturer's written instructions.

Retain first paragraph below only if required.  Revise bracing type and reinforcement at terminations to suit Project.  Description below usually applies to shear walls.  Insert track reinforcing or gusset plate requirements to Drawings if applicable, particularly for shear walls.  If sheathing provides bracing to one or both stud flanges, revise to suit Project.

J. Install steel sheet diagonal bracing straps to both stud flanges, terminate at and fasten to reinforced top and bottom tracks.  Fasten clip-angle connectors to multiple studs at ends of bracing and anchor to structure as shown in shop drawings.

K. Install miscellaneous framing and connections, including supplementary framing, web stiffeners, clip angles, continuous angles, anchors, and fasteners, to provide a complete and stable wall-framing system.

3.5 EXTERIOR NON-LOAD-BEARING WALL INSTALLATION

If this article is required, retain with "Installation, General" Article.

A. Install continuous tracks sized to match studs.  Align tracks accurately and securely anchor to supporting structure as indicated.

Retain fastening of studs to top track only if required.  Do not fasten studs to deflection track, such as in infill wall framing.

B. Fasten both flanges of studs to[ top and] bottom track unless otherwise indicated.  Space studs as follows:

Retain from options in "Stud Spacing" Subparagraph below for IP spacings and equivalent SI (metric) dimensions.

1. Stud Spacing:  [12 inches (305 mm)] [16 inches (406 mm)] [19.2 inches (488 mm)] [24 inches (610 mm)] [As indicated].

C. Set studs plumb, except as needed for diagonal bracing or required for nonplumb walls or warped surfaces and similar requirements.

Indicate isolation details on Drawings or insert detailed description here.

D. Isolate non-load-bearing steel framing from building structure to prevent transfer of vertical loads while providing lateral support.

Retain one of first two subparagraphs below if using deflection track with infill wall framing.  Delete both subparagraphs and retain third if deflection clips are required for bypassing or infill wall framing.  Vertical deflection clips are used if wall is stick built or panelized.  Single deflection track is used if stud wall is stick built; double deflection track is used with wall panels.  Drift clips may be used in seismic areas.

1. Install [single leg deflection tracks] [MaxTrak] [SLP-TRK] and anchor to building structure.

2. Install double deep-leg deflection tracks and anchor outer track to building structure.

3. Connect [Fast Clip Slide Clip (FCSC)] [Fast Strut Clip (FS)] [QuickClip (QC)] [Slide Clip (SC)] vertical deflection clips to [bypassing] [infill] studs and anchor to building structure.

4. Connect drift clips to cold-formed metal framing and anchor to building structure.

E. Install horizontal bridging in wall studs, spaced in rows indicated on Shop Drawings but not more than 48 inches (1220 mm) apart.  Fasten at each stud intersection.

Retain "Top Bridging for Single Deflection Track" Subparagraph below if required.

1. Top Bridging for Single Deflection Track:  Install [row of horizontal bridging] [Spazzer Proprietary Bridging Bar (SPZS)] within [12 inches (305 mm)] [18 inches (457.2 mm)] of single deflection track.  [Bridging is not required in top row when using proprietary slotted deflection track (MaxTrak) (SLP-TRK).]  Install a combination of flat, taut, steel sheet straps of width and thickness indicated and stud or stud-track solid blocking of width and thickness matching studs.  Fasten flat straps to stud flanges and secure solid blocking to stud webs or flanges.

a. Install solid blocking at [96-inch (2438.4-mm) centers] [centers indicated] [centers indicated on Shop Drawings].

Retain type of bridging required from three "Bridging" subparagraphs below or revise to suit Project.  Insert locations if more than one type of bridging is required.  Insert minimum size of flat steel strap, such as 1-1/2 by 0.0329 inch (38 by 0.84 mm), if default size is required.

2. Bridging:  Cold-formed steel channel, welded or mechanically fastened to webs of punched studs.

3. Bridging:  Combination of flat, taut, steel sheet straps of width and thickness indicated and stud-track solid blocking of width and thickness to match studs.  Fasten flat straps to stud flanges and secure solid blocking to stud webs or flanges.

4. Bridging:  Proprietary bridging bars installed according to manufacturer's written instructions.

F. Install miscellaneous framing and connections, including stud kickers, web stiffeners, clip angles, continuous angles, anchors, fasteners, and stud girts to provide a complete and stable wall-framing system.

3.6 JOIST INSTALLATION

If this article is required, retain with "Installation, General" Article.

A. Install in accordance with AISI's S210 "North American Standard for Cold-Formed Steel Framing - Floor and Roof System Design".

B. Install perimeter joist track sized to match joists.  Align and securely anchor or fasten track to supporting structure at corners, ends, and spacings indicated on Shop Drawings.

C. Install joists bearing on supporting frame, level, straight, and plumb; adjust to final position, brace, and reinforce.  Fasten joists to both flanges of joist track.

Revise minimum end bearing in first subparagraph below if joists bearing on flange of joist track are acceptable.  Web stiffeners will usually be required to reinforce joist web.

1. Install joists over supporting frame with a minimum end bearing of 1-1/2 inches (38 mm).

2. Reinforce ends and bearing points of joists with web stiffeners, end clips, joist hangers, steel clip angles, or steel-stud sections as indicated on Shop Drawings.

D. Space joists not more than 2 inches (51 mm) from abutting walls, and as follows:

Retain from options in first "Joist Spacing" Subparagraph below for IP spacings and equivalent SI (metric) dimensions.

1. Joist Spacing:  [12 inches (305 mm)] [16 inches (406 mm)] [19.2 inches (488 mm)] [24 inches (610 mm)] [As indicated].

Retain from options in "Joist Spacing" Subparagraph below if SI (metric) spacings are required.

2. Joist Spacing:  [300 mm] [400 mm] [480 mm] [600 mm] [As indicated].

E. Frame openings with built-up joist headers consisting of joist and joist track, nesting joists, or another combination of connected joists if indicated.

Retain first paragraph below if joist reinforcement is required.

F. Install joist reinforcement at interior supports with single, short length of joist section located directly over interior support, with lapped joists of equal length to joist reinforcement, or as indicated[ on Shop Drawings].

1. Install web stiffeners to transfer axial loads of walls above.

G. Install bridging at intervals indicated[ on Shop Drawings].  Fasten bridging at each joist intersection as follows:

Retain type of bridging required from two "Bridging" subparagraphs below or revise to suit Project.  Insert locations if more than one type of bridging is required.  Insert minimum size of flat steel strap, such as 1-1/2 by 0.0329 inch (38 by 0.84 mm), if default size is required.  Revise if also fastening flat steel strap bridging to top flange of joists.

1. Bridging:  Joist-track solid blocking of width and thickness indicated, secured to joist webs.

2. Bridging:  Combination of flat, taut, steel sheet straps of width and thickness indicated and joist-track solid blocking of width and thickness indicated.  Fasten flat straps to bottom flange of joists and secure solid blocking to joist webs.

H. Secure joists to load-bearing interior walls to prevent lateral movement of bottom flange.

I. Install miscellaneous joist framing and connections, including web stiffeners, closure pieces, clip angles, continuous angles, hold-down angles, anchors, and fasteners, to provide a complete and stable joist-framing assembly.

Insert additional article titles here if other framing classifications warrant more detailed installation requirements than those in "Installation, General" Article.

3.7 FIELD QUALITY CONTROL

Retain this article if field testing and inspecting are required.  Provisions below include shop and field testing and inspecting.  Revise and insert a "Source Quality Control" Article to end of Part 2 if separate shop and field testing and inspecting are required.  Revise "Testing" Paragraph below to include code-mandated special inspections or revise if Contractor engages testing agency.

A. Testing:  Owner will engage a qualified independent testing and inspecting agency to perform field tests and inspections and prepare test reports.

B. Field and shop welds will be subject to testing and inspecting.

C. Testing agency will report test results promptly and in writing to Contractor and Architect.

D. Remove and replace work where test results indicate that it does not comply with specified requirements.

E. Additional testing and inspecting, at Contractor's expense, will be performed to determine compliance of replaced or additional work with specified requirements.

3.8 REPAIRS AND PROTECTION

Retain "Galvanizing Repairs" Paragraph below if applicable.

A. Galvanizing Repairs:  Prepare and repair damaged galvanized coatings on fabricated and installed cold-formed steel framing with galvanized repair paint according to ASTM A 780 and manufacturer's written instructions.

B. Provide final protection and maintain conditions, in a manner acceptable to manufacturer and Installer, to ensure that cold-formed steel framing is without damage or deterioration at time of Substantial Completion.

END OF SECTION 054000
	COLD-FORMED METAL FRAMING
	054000 - 6



