Parallam® PSL Specifications

# 1.0	GENERAL

## 1.1	Scope

This work includes the complete furnishings and installation of all Parallam® parallel strand lumber (PSL) as shown on the drawings herein specified and necessary to complete the work.

## 1.2	Code Approvals

These products shall be designed and manufactured to the standards set forth in the ICC Evaluation Service Report No.  ESR-1387.

## 1.3	Related Work Specified Elsewhere

- Carpentry and Millwork

- Glu-Laminated Members

## 1.4	Design

- Products

Parallam® PSL shall be designed to fit the dimensions and loads indicated on the plans.

- Design Calculations

__ Member calculations shall be prepared by Weyerhaeuser.  (Service Fees may apply)

__ Not required.

## 1.5	Submittals

- A	Drawings

__ Drawings showing layout and detail necessary for determining fit and placement in the building shall

be provided by Weyerhaeuser.

__ Not required.

- B	Production

Fabrication and/or cutting shall not proceed until the architect and/or engineer have approved the submittal package.

# 2.0	PRODUCTS

## 2.1	Materials

- Code Reports

Materials shall comply with ICC ES ESR-1387.

- Adhesives

Adhesives shall be of the waterproof type conforming to the requirements of ASTM D-2559.

## 2.2	Fabrication

Parallam® PSL shall be manufactured by Weyerhaeuser in a plant listed in the reports referred to above and under the supervision of an approved third-party inspection agency. It shall be manufactured from strands of wood fiber in a continuous process with all strands oriented to the length of the member and then fed into a press in the desired lay-up pattern. All members are to be free of finger or scarf joints or mechanical connections in full-length members.

## 2.3	Tolerances

Finished Length (as specified):	± 1/4"

Depth:	± 1/16"

Width:	± 1/16"

## 2.4	Identification

Parallam® PSL shall be identified by a stamp indicating the product type and grade and ICC-ES evaluation report number, manufacturer's name, plant number and the independent inspection agency’s logo.

## 2.5	Hardware

Not applicable.

# 3.0	EXECUTION

## 3.1	Installation

Parallam® PSL, if stored prior to installation, shall be protected from the weather. It shall be installed in accordance with the plans and any Weyerhaeuser drawings and installation suggestions. Temporary construction loads that cause stresses beyond design limits are not permitted. Safety bracing is to be provided by the installer to keep the Parallam® PSL straight and plumb as required and to assure adequate lateral support for the individual Parallam® PSL members and the entire system until the sheathing material has been applied.

## 3.2	Installation Review

The contractor shall give notification to the building inspector prior to enclosing product to provide opportunity for review of the installation within conformance of construction documents and/or manufacturer’s literature.

## 3.3	Performance Standards

Products shall be proven by testing and evaluation in accordance with the provisions of ASTM D-5456.

## 3.4	Fire Rating

Parallam® PSL is permitted as a substitute for conventional wood framing in fire-resistive assemblies. Parallam® PSL shall be sized for the same load-carrying capacity as the sawn lumber specified in the assembly, and its dimensions shall be equal to or greater than those specified for the sawn lumber. The fire-resistive design provisions for heavy-timber construction in all U.S. model building codes are applicable to Parallam® PSL.

## 3.5	Warranty

The products delivered shall be free from manufacturing errors or defects in workmanship and material. The products, when correctly installed and maintained, shall be warranted to perform as designed for the normal and expected life of the building.

# 4.0	ALTERNATES AND/OR EQUALS

## 4.1	Base Bid

Due to the customized detailing and engineering characteristics of the roof and/or floor framing assembly, it is a requirement that Parallam® PSL be used in the base bid.

## 4.2	Alternate Manufacturers

Other manufacturers’ bids are to be listed in the alternate section of your proposal. All framing plans, detailing and calculations for the alternate bids will be reviewed by the owner, architect and engineer for structural performance, possible conflicts with related trades, and compatibility with the overall building requirements and building code.

## 4.3	Alternate Products

Alternate products will only be permitted if written approval and acceptance is obtained by both architect and owner at least ___ days prior to the bid date.

## 4.4	Acceptable Alternates

At the discretion of the specifier of record, accepted alternates will be listed on the final addendum prior to the bid date.