SHORT FORM SPECIFICATION

For assistance with detail review, additional options, radius shapes and specialty one-of-a-kind products, contact Johns Manville Roofing Accessories Technical Services 1-800-445-1500,

E-mail - roofacc@jm.com  Website - www.jm.com

Roof Edge Fascia System (Presto-Tite extruded aluminum edge attached system with snap-on cover - available as part of JM total roof system guarantee)

Warranty description for insertion into Part 1 of spec.   (Note:  Include the following where JM Single Ply,     Built-Up, or Modified Bitumen systems are specified to provide a total system guarantee.)

- Total System Guarantee:  Roof edge fascia system shall be included as part of the Johns Manville Roof System Guarantee.  See Section [       ].

Product description for insertion into Part 2 of spec.

- Roof Edge Fascia System:  Johns Manville Presto-Tite.

  - Performance Requirements:  20 year warrantee against blow-off up to 120 mph wind.  FM 1-90 approved.  Exceeds 540 pounds per liner foot per ANSI/SPRI ES-1-98 Wind Design Standards test method RE-1.

  - Style:  [As indicated]  [{Modified} {BUR} version {PFM-50} {PFM-65} {PFM-85}]  [ Single ply {adhered} {mechanically fastened} version {PF-40} {PF-55} {PF-70} {PF-85}]  [ Single ply stone ballasted version {PFB-50} {PFB-65} {PFB-80}].  (Note: Curved configurations available, contact JM.)

  - Anchor Bar:  Extruded 6063-T6 alloy aluminum, 12 feet ( 3.65m) long, pre-slotted 12 inches (304mm) on center.

  - Anchor Bar Splice Plates:  .025 inch (0.6mm) thick mill finish aluminum with closed cell compression gaskets.

  - Fascia Cover:  Snap-on fascia cover, 12 feet ( 3.65m) long, ends notched for 1 inch (25.4mm) overlap.

  - Fascia Metal: [.040 inch (.91mm) thick aluminum in {mill} {clear anodized} {color anodized} {Kynar 500} finish.] [24 gage (.7mm) galvanized steel with Kynar 500 finish] [Color {as indicated} {selected by Architect from manufacturer’s standard colors}.]  (Note: Other metals, colors and finishes available,  contact JM.)

  - [Extender] and [Brick/Wall Cap]:  Same metal type and finish as fascia, [{.080 inch (2.8mm)} {.063 inch (1.6mm)} {.050 inch (1.3mm)}{.040 inch (.91mm)}] [{20 gage (1.0mm)} {22 gage (.9mm)} {24 gage (.7mm)}] thickness with 1/8 inch offset pre-punched nailing edge.  [Continuous 22 gage (.9mm) galvanized steel concealed cleat for securing bottom edge.] [Continuous soffit cleat for securing bottom edge, of same gage, and finish as {extender} {brick/wall cap}.]

  - Accessories:  Factory fabricated [inside and outside mitered corners,] [spillout scupper with 4 inch (101mm) extended lip,] [ overflow scupper with 1/2 inch extended lip] and [downspout scupper with welded aluminum collector box and outlet of required shape and size].  Same metal type and finish as fascia cover.

  - Fasteners:   Corrosion resistant #12 x 1-5/8 inch (41mm) long, with hexagonal head and 5/8 inch (16mm) bonded washer with EPDM washer seal, furnished as part of fascia system for attachment to wood blocking.  Provide equivalent corrosion resistant fastener of type and size required for other substrate types.

Product installation for insertion into Part 3 of spec.

- Roof Edge Fascia System:  Install extruded aluminum edge attached system with snap-on cover and accessories in strict accordance with manufacturer’s written instructions, providing a permanent weather tight installation.

  - Inspect and determine substrate to be in satisfactory condition, with pressure treated blocking fully anchored and aligned at proper location and elevation.  All surfaces shall be smooth, dry, clean, free of splinters, sharp edges, and other irregularities.

  - Apply non-curing sealant that is compatible with roof system at required locations for water tight installation.  Position anchor bars with 1/2 inch (13mm) separation between ends, properly sealing at splice plates and fastened at required spacing.

  - Position accessories and secure into place before installation of full length runs.

  - Remove protective film from covers immediately before installation.

  - Snap fascia cover in place, overlapping ends approximately 1 inch (25.4mm).