SHORT FORM SPECIFICATION

For assistance with detail review, contact Johns Manville Roofing Accessories Technical Services

1-800-445-1500, E-mail - roofacc@jm.com  Website - www.jm.com

Retrofit Drain System (PC/PETand SuperDome RetroDrain engineered resin sleeve drain body with resin or cast aluminum strainer basket - available as part of JM total roof system guarantee)

Warranty description for insertion into Part 1 of spec.   (Note:  Include the following where JM Single Ply,     Built-Up, or Modified Bitumen systems are specified to provide a total system guarantee.)

- Total System Guarantee:  Roof drain system shall be included as part of the Johns Manville Roof System

- Guarantee.  See Section [       ].

Product description for insertion into Part 2 of spec.

- Roof Drain System:  Johns Manville [PC/PET RetroDrain] [SuperDome RetroDrain].

- Drain Body:  14 inch (356mm) diameter one piece resin co-polymer flange and 12 inch (305mm) long tapered drain body stem with compression seal, sized for sealing tight to inside of existing drainage pipe.

- Compression Seal:  Elastomeric compression seal held with compression ring and stainless steel screws.

- Strainer: [4 inch (102mm) high resin co-polymer snap fit] [ 5-1/2 inch (140mm) high cast aluminum screw down] basket strainer.

- Outlet Size: [3 inch (76mm)] [4 inch (102mm)] [5 inch (127mm)] [6 inch (152mm)].

Product installation for insertion into Part 3 of spec.

- Roof Drain System:  Install roof drain and accessories in strict accordance with manufacturer’s written instructions, providing a permanent weather tight installation.

- Preparation:  Remove existing strainer dome, clamping ring, and other components as required to enable retrofit drain flange to lie flush with roof membrane.  Remove debris and constricting material built up on the interior of the existing drain pipe that interferes with proper installation and sealing of compression seal.  All surfaces shall be smooth, dry, clean, free of sharp edges, and other irregularities.

- Insert drain body down into existing drain pipe until flange is flush with roof membrane, seating flange in membrane compatible mastic.  Tighten seal compression ring.

- Flash flange into roof system per roof system and roof drain manufacturer requirements.

- Securely attach clamping ring and strainer basket.

SHORT FORM SPECIFICATION

For assistance with detail review, contact Johns Manville Roofing Accessories Technical Services

1-800-445-1500, E-mail -roofacc@jm.com  Website - www.jm.com

Retrofit Drain System (USII aluminum drain body with resin or cast aluminum strainer basket - available as part of JM total roof system guarantee)

Warranty description for insertion into Part 1 of spec.   (Note:  Include the following where JM Single Ply,     Built-Up, or Modified Bitumen systems are specified to provide a total system guarantee.)

- A.	Total System Guarantee:  Roof drain system shall be included as part of the Johns Manville Roof System Guarantee.  See Section [       ].

Product description for insertion into Part 2 of spec.

- A.	Roof Drain System:  Johns Manville USII RetroDrain.

- Drain Body:  0.050 inch ( 1.3mm) thick spun 1100 aluminum alloy, 17-1/2 inch (445mm) diameter one piece flange and 12 inch (305mm) long tapered drain stem with compression seal, sized for sealing tight to inside of existing drainage pipe.

- Compression Seal:  Elastomeric compression seal held with compression ring and stainless steel screws.

- Strainer: [4 inch (102mm) high resin co-polymer] [ 7 inch (178mm) high cast 362 aluminum alloy] basket strainer.

- Outlet Size: [3 inch (76mm)] [4 inch (102mm)] [5 inch (127mm)] [6 inch (152mm)].

Product installation for insertion into Part 3 of spec.

- A.	Roof Drain System:  Install roof drain and accessories in strict accordance with manufacturer’s written instructions, providing a permanent weather tight installation.

- Preparation:  Remove existing strainer dome, clamping ring, and other components as required to enable retrofit drain flange to lie flush with roof membrane.  Remove debris and constricting material built up on the interior of the existing drain pipe that interferes with proper installation and sealing of compression seal.  All surfaces shall be smooth, dry, clean, free of sharp edges, and other irregularities.

- Insert drain body down into existing drain pipe until flange is flush with roof membrane, seating flange in membrane compatible mastic.  Tighten seal compression ring.

- Flash flange into roof system per roof system and roof drain manufacturer requirements.

- Securely attach clamping ring and strainer basket.

SHORT FORM SPECIFICATION

For assistance with detail review, contact Johns Manville Roofing Accessories Technical Services

1-800-445-1500, E-mail -roofacc@jm.com  Website - www.jm.com

Retrofit Drain System (Hercules RetroDrain heavy duty copper or aluminum drain body with resin or cast aluminum strainer basket - available as part of JM total roof system guarantee)

Warranty description for insertion into Part 1 of spec.   (Note:  Include the following where JM Single Ply,     Built-Up, or Modified Bitumen systems are specified to provide a total system guarantee.)

- A	Total System Guarantee:  Roof drain system shall be included as part of the Johns Manville Roof System Guarantee.  See Section [       ].

Product description for insertion into Part 2 of spec.

- Roof Drain System:  Johns Manville Hercules RetroDrain.

- Drain Body:  [0.125 inch (3.2 mm) thick spun 1100 aluminum alloy] [0.050 inch (1.3mm) thick spun punched copper (Note:  For 5 and 6 inch diameter stems)] [0.032 inch (.8mm) thick spun punched copper (Note: For 3 and 4 inch diameter stems)], 17-1/2 inch (445mm) diameter one piece flange and 12 inch (305mm) long tapered drain stem with compression seal, sized for sealing tight to inside of existing drainage pipe.  Cast aluminum clamping ring.

- Compression Seal:  Elastomeric compression seal held with compression ring and stainless steel screws.

- Strainer:  [ 7 inch (178mm) high cast 362 aluminum alloy] [ 5-1/2 inch (140mm) high cast aluminum screw down] basket strainer.

- Outlet Size: [3 inch (76mm)] [4 inch (102mm)] [5 inch (127mm)] [6 inch (152mm)].

Product installation for insertion into Part 3 of spec.

- Roof Drain System:  Install roof drain and accessories in strict accordance with manufacturer’s written instructions, providing a permanent weather tight installation.

- Preparation:  Remove existing strainer dome, clamping ring, and other components as required to enable retrofit drain flange to lie flush with roof membrane.  Remove debris and constricting material built up on the interior of the existing drain pipe that interferes with proper installation and sealing of compression seal.  All surfaces shall be smooth, dry, clean, free of sharp edges, and other irregularities.

- Insert drain body down into existing drain pipe until flange is flush with roof membrane, seating flange in membrane compatible mastic.  Tighten seal compression ring.

- Flash flange into roof system per roof system and roof drain manufacturer requirements.

- Securely attach clamping ring and strainer basket.