SHORT FORM SPECIFICATION

For assistance with detail review, additional options, radius closures, penetration closures and specialty one-of-a-kind products, contact Johns Manville Roofing Accessories Technical Services 1-800-445-1500,

E-mail - roofacc@jm.com  Website - www.jm.com

Metal/Membrane Flashing  (Single ply membrane strip with one metal flange)

Warranty description for insertion into Part 1 of spec.   (Note: Include the following where JM Single Ply,      Built-Up, or Modified Bitumen systems are specified to provide a total system guarantee.)

A.	Total System Guarantee:  Metal/membrane flashing shall be included as part of the Johns Manville Roof System Guarantee.  See Section [       ].

Product description for insertion into Part 2 of spec.

A.	Metal/Membrane Flashing:  Johns Manville SPM Metal Membrane Flashing.

1.	Single Flange Style:  EJ.

2.	Flange Metal:  4 inch  (100mm) wide, 26 gage (0.056mm) G60 hot dipped galvanized steel  (Note:  Other metals available, contact JM).

3.	Membrane:  6-1/2 inch wide (163mm), 0.060 (1.5mm) inch thick non-reinforced black E(EPDM) (Note:  Other membrane types and widths available.  Contact JM)  adhesively and mechanically combined to metal flange by bifurcation process.

4.	Fasteners:  Corrosion resistant fasteners compatible with flange metal and equivalent in corrosion resistance, of proper length and type for substrate.

Product installation for insertion into Part 3 of spec.

A.	Metal/Membrane Flashing:  Install metal/membrane flashing in strict accordance with manufacturer’s written instructions, providing a permanent weather tight installation.

1.	Attach flange securely to substrate, allowing proper spacing at splices.

2.	Splice adjoining sections using manufacturer furnished splice kit.  Clean splice area to assure proper adhesion of splice strip.  Prime metal flanges to assure proper adhesion of splice strip.

3.	Flash membrane and flange metal into roof system per roof system manufacturer requirements.  [Prime and strip flange metal into existing built-up roofing with minimum {    ply} {hot} {cold} tie in.]