SECTION 12 46 27 (12461)

DECORATIVE CANOPIES

PART 1 - GENERAL

1.1  	RELATED DOCUMENTS

Drawings and general conditions of Contract, including General and Supplementary Conditions and Divisions-1 Specification sections apply to work of this section.

1.2	SUMMARY

A.	Section Includes:

1.	Ceiling canopies

2.	Cable hangers and fasteners

B.	Related Sections:

- Section 09 51 00 (09510) – Acoustical Ceilings

- Section 09 20 00 (09250 – Plaster and Gypsum Board

- Division 23 (15) Sections - HVAC

- Division 26 (16) Sections - Electrical

C. 	Alternates

1.	Prior Approval: Unless otherwise provided for in the Contract documents, proposed product substitutions may be submitted no later than TEN (10) working days prior to the date established for receipt of bids.  Acceptability of a proposed substitution is contingent upon the Architect’s review of the proposal for acceptability and approved products will be set forth by the Addenda.  If included in a Bid are substitute products which have not been approved by Addenda, the specified products shall be provided without additional compensation.

2.	Submittals which do not provide adequate data for the product evaluation will not be considered.  The proposed substitution must meet all requirements of this section, including but not neccessarily limited to, the following: Single source materials suppliers (if specified in Section 1.5); Panel design, size, composition, color, and finish; suspension system component profiles and sizes; compliance with the referenced standards.

1.3	REFERENCES

A.   International Building Code - Chapter 8, Chapter 26

B.   NFPA 286: Standard Methods of Fire Tests for Evaluating Contribution of Wall and Ceiling Interior Finish to Room Fire Growth

1.4	SUBMITTALS

- Product Data:  Submit manufacturer’s technical data for each type of canopy system required.

- Installation Instructions: Submit manufacturer’s installation instructions, LA297055.

- Samples:  Minimum 2 inch x 3 inch samples of specified canopy.

- Shop Drawings:  Layout and details of canopies.  Show locations of items which are to be coordinated with canopies.

- Certifications:  Manufacturer’s certifications that products comply with specified requirements, including laboratory reports showing compliance with specified tests and standards.

1.5	QUALITY ASSURANCE

- Single-Source Responsibility:  Provide canopies and method of attachment by a single manufacturer.

- Coordination of Work:  Coordinate canopy work with installers of related work including, but not limited to suspended ceilings, building insulation, gypsum board, light fixtures, mechanical systems, electrical systems, and sprinklers.

- Infusions panels, as with other architectural features located at the ceiling, may obstruct or skew the planned fire sprinkler water distribution pattern, or possibly delay or accelerate the activation of the sprinkler or fire detection systems by channeling heat from a fire either toward or away from the device.  Designers and installers are advised to consult a fire protection engineer, NFPA 13, or their local codes for guidance where automatic fire detection and supression systems are present.

1.6	DELIVERY, STORAGE, AND HANDLING

A.	Deliver canopies to project site in original, unopened packages and store them in a fully enclosed space where they will be protected against damage from moisture, direct sunlight, surface contamination, and other causes.

- Before installing canopies, permit them to reach room temperature and a stabilized moisture content.

- Handle canopies carefully to avoid damaging units in any way.

1.7	PROJECT CONDITIONS

A.	Space Enclosure:

Building areas to receive canopies shall be free of construction dust and debris. Products can be installed up to 100°F (38°C) and in spaces before the building is enclosed, where HVAC systems are cycled or not operating. Cannot be used in exterior applications, where standing water is present, or where moisture will come in direct contact with the canopy.

1.8	WARRANTY

A.	Ceiling canopy:  Submit a written warranty executed by the manufacturer, agreeing to repair or replace canopies that fail within the warranty period.  Failures include, but are not limited to:

- Canopies:  Manufacturer’s defects.

- Attachment devices:  Rusting and manufacturer’s defects.

- B.	Warranty Period:

- 1.	Canopies: One (1) year from date of substantial completion.

- 2.	Attachment devices:  One (1) year from date of substantial completion.

- C.   The Warranty shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and will be in addition to and run concurrent with other warranties made by the Contractor under the requirements of the Contract Documents.

- PART 2 - PRODUCTS

- 2.1	MANUFACTURERS

- A.	Canopies:

- 1.	Armstrong World Industries, Inc.

- B.	Attachment devices:

- 1.	Armstrong World Industries, Inc.

- 2.2.1	CANOPY UNITS

- A.	Canopies (2x5) and (2x6):

- 1.   Surface Texture: Smooth

- 2.	Composition: (Light transmitting plastic) (Aluminum)

- Color:

- Channeled Polycarbonate – 6 colors, indicate with color suffix on item number

- Solid Polycarbonate – 26 colors, indicate with color suffix on item number

- Screen Printed Polycarbonate (printed both sides)  – 5 designs with 3 colors each, indicate design and color with suffix on item number.

- Size:  (2 feet x 5 feet) (2 feet x 6 feet)

- Thickness:   [Aluminum: 1/8 inch]  [Light Transmitting Plastic: 1/4 inch].

- Weight:   [Light Transmitting Plastic: (Solid and Screen Printed Polycarbonate, 1.8 psf) (Channeled Polycarbonate, 0.26 psf)]  (Aluminum, 1.72 psf).

- Edge Detail:  Square edge

- Panel Configuration:  (Convex) (Concave)

- Arc Radius:  (60 degrees) (90 degrees)

- Fire Properties:  (Aluminum – Noncombustible); (Infusions Canopies have been tested according to NFPA 286 and are equivalent to Class A  interior finish as defined in Chapter 8 of the International Building Code).

- Acceptable Product:  Infusions Accent Canopy, Item # (5405_ _ _ _) (5406_ _ _ _), as manufactured by Armstrong World Industries.

- B.	Canopies (4x10):

- 1.   Surface Texture: Smooth

- 2.	Composition: Light transmitting plastic

- Color: (5 choices, indicate with color suffix on item number)

- Size:  (4 feet x 10 feet)

- Thickness:  5/8 inch

- Weight:  Light Transmitting Plastic: Channeled Polycarbonate, 0.50 psf.

- Edge Detail:  Square edge

- Panel Configuration:  (Convex) (Concave)

- Arc Radius:  (30 degrees) (50 degrees)

- Fire Properties:  Infusions Canopies have been tested according to NFPA 286 and are equivalent to Class A  interior finish as defined in Chapter 8 of the International Building Code.

- Acceptable Product:  Infusions Accent Canopy, Item # (5410_ _ _ _), as manufactured by Armstrong World Industries.

2.2.2	ATTACHMENT SYSTEMS

- Installation Hardware Kits

Standard kits include component requirements for various types of installations and are compatible with all canopy sizes.  Refer to installation instructions regarding use of these components.

- Hanging Kits: Include Hanging Cables and Forming Cables:   1/16 inch diameter 7x7 steel aircraft cable (commercial grade) with integral die cast end.

a.  7004 –  8 foot hanging kit

b.  7005 -  16 foot hanging kit

c.  7010 – 30 foot hanging kit

d.  7041 – Dual canopy hanging kit

- Wall & Ceiling Kits:

a.  7006 – Escutcheon kit

b.  7008 – Wall attachment kit

c.  7009 – Wall end attachment kit

- Linking Kits:

a.  7042 – Flush spacing kit

b.  7043 – 1/2 inch spacing kit

c.  7007 – 3 inch spacing kit

d.  7044 – Single hinge linking kit

e.  7045 – Dual hinge flush linking kit

f.  7046 – Dual hinge 1/2 inch linking kit

g.  7047 – Dual hinge 3 inch linking kit

PART 3 - EXECUTION

3.1	PREPARATION

- Measure each ceiling area and establish layout of canopies.  Comply with reflected ceiling plans.  Coordinate panel layout with mechanical, electrical and sprinkler fixtures.

3.2	INSTALLATION

A.	Install canopies in accordance with the most current manufacturer’s instructions: LA- 297055 and in compliance with the authorities having jurisdiction.

3.3	ADJUSTING AND CLEANING

A.	Replace damaged and broken panels.

B.	Clean exposed surfaces of canopies per installation instructions.

END OF SECTION