Armstrong World Industries

Canopy Specification

| Job Number: |  | E-mail Address: |  |
| --- | --- | --- | --- |
| Job Type: |  | Zip Code: |  |
| Project Name: |  | Date & Time: |  |
| Firm Name: |  | Rep Name: |  |
| Contact Name: |  | Rep Phone: | 877 276 7876, ext. |

Please understand that you are responsible for the accuracy of all project specifications, including any Armstrong guide specifications that you use.  ARMSTRONG SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.

SECTION 09 84 13

Sound Absorptive Panels

PART 1 - GENERAL

1.1  	RELATED DOCUMENTS

Drawings and general conditions of Contract, including General and Supplementary Conditions and Divisions-1 Specification sections apply to work of this section.

1.2	SUMMARY

A.	Section Includes:

1.	Acoustical panels

2.	Cable hangers and fasteners

B.	Related Sections:

- Section 09 51 00 (09510) – Acoustical Ceilings

- Section 09 20 00 (09250 – Plaster and Gypsum Board

- Division 23 (15) Sections - HVAC

- Division 26 (16) Sections - Electrical

C. 	Alternates

1.	Prior Approval: Unless otherwise provided for in the Contract documents, proposed product substitutions may be submitted no later than TEN (10) working days prior to the date established for receipt of bids.  Acceptability of a proposed substitution is contingent upon the Architect’s review of the proposal for acceptability and approved products will be set forth by the Addenda.  If included in a Bid are substitute products which have not been approved by Addenda, the specified products shall be provided without additional compensation.

2.	Submittals which do not provide adequate data for the product evaluation will not be considered.  The proposed substitution must meet all requirements of this section, including but not neccessarily limited to, the following: Single source materials suppliers (if specified in Section 1.5); Panel design, size, composition, color, and finish; suspension system component profiles and sizes; compliance with the referenced standards.

1.3	REFERENCES

A.	American Society for Testing and Materials (ASTM):

- ASTM C 423 Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

- ASTM E 84 Standard Test Method for Surface Burning Characteristics of Building Materials.

- ASTM E 1477 Standard Test Method for Luminous Reflectance Factor of Acoustical Materials by Use of Integrating-Sphere Reflectometers.

B.   International Code Council Evaluation Service

AC156 - Acceptance Criteria for Seismic Qualification Testing of Non-structural Components

1.4	SUBMITTALS

- Product Data:  Submit manufacturer’s technical data for each type of canopy system required.

- Installation Instructions: Submit manufacturer’s installation instructions.

- Shop Drawings:  Layout and details of canopies.  Show locations of items which are to be coordinated with canopies.

- Certifications:  Manufacturer’s certifications that products comply with specified requirements, including laboratory reports showing compliance with specified tests and standards.

1.5	QUALITY ASSURANCE

- Single-Source Responsibility:  Provide canopies and method of attachment by a single manufacturer.

- Coordination of Work:  Coordinate canopy work with installers of related work including, but not limited to suspended ceilings, building insulation, gypsum board, light fixtures, mechanical systems, electrical systems, and sprinklers.

- Seismic Performance:  System seismic performance verified through full-scale testing in accordance with ICC-ES – AC-156 Acceptance Criterial for Seismic Qualification Testing of Non-Structural components.

- Soundsoak Baffles, as with other architectural features located in the ceiling plane, may obstruct or skew the existing or planned fire sprinkler water distribution pattern, or possibly delay the activation of the fire sprinkler or fire detection system.  Designers and installers are advised to consult a fire protection engineer, NFPA 13, and their local codes for guidance on the proper installation techniques where fire detection or suppression systems are present.

1.6	DELIVERY, STORAGE, AND HANDLING

A.	Deliver Soundsoak Baffles to project site in original, unopened packages and store them in a fully enclosed space between 40ºF (4ºC) and 120ºF (49ºC) where they will be protected against damage from moisture, direct sunlight, surface contamination, and other causes.

- Before installing Soundsoak Baffles, permit them to reach room temperature and a stabilized moisture content.

- Handle Soundsoak Baffles carefully to avoid damaging units in any way.

1.7	PROJECT CONDITIONS

A.	Space Enclosure:

Building areas to receive Soundsoak Baffles shall be free of construction dust and debris. Products can be installed up to 120°F (49°C) with humidity not exceeding 90% RH. Cannot be used in exterior applications where standing water is present or where moisture will come in direct contact with the canopy. Following installation, conditions must be maintained below 70% RH.

1.8	WARRANTY

A.	Soundsoak Baffles:  Submit a written warranty executed by the manufacturer, agreeing to repair or replace canopies that fail within the warranty period.  Failures include, but are not limited to:

- Soundsoak Baffles:  Manufacturing defects.

- Attachment devices:  Manufacturing defects.

- B.	Warranty Period:

- 1.	Soundsoak Baffles: One (1) year from date of substantial completion.

- 2.	Attachment devices:  One (1) year from date of substantial completion.

- C.   The Warranty shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and will be in addition to and run concurrent with other warranties made by the Contractor under the requirements of the Contract Documents.

- PART 2 - PRODUCTS

- 2.1	MANUFACTURERS

- A.	Soundsoak Baffles:

- 1.	Armstrong World Industries, Inc.

- B.	Attachment devices:

- 1.	Armstrong World Industries, Inc.

- 2.2.1	SOUNDSOAK BAFFLE UNITS

- A.	Soundsoak Baffles:

- 1.   Composition:

- a. Fiberglass Core

- b. BioAcoustic Core

- Color

a. Woven Fabric Surface: Fiberglass Core or BioAcoustic Core (Silver Papier)(Bone)(Quartz)(Opal)(Wheat)(Straw)(Terra)

(Leaf)(Eucalyptus)(Cherry Neutral)(Silver Neutral)(Cement Mix)(Blue Neutral)(Blue Papier)(Grey Mix)(Crystal Blue)(Blue Spruce)(Violet)(Ultramarine)(Deep Burgundy)(Blue Plum)(Black)

b.Sailcloth Fabric:Fiberglass Core only   (White)(Silver)(Yellow)(Red)(Royal)(Maroon)(Navy)(Black)

2.   Size:  (Fiberglass FR-701 Fabric Wrapped 24 inches x 48 inches, BioAcoustic FR-701 Frabic Wrapped 24 inches x 47 1/2 inches), (Sailcloth Fabric 24 inches x 48                inches)(also available in custom sizes up to 4 feet x 8 feet)

- Thickness:   (Fiberglass FR-701 Fabric Wrapped - 2 inches)(BioAcoustic FR-701 Fabric Wrapped – 1 1/4 inches)(Sailcloth – 1 1/2 inches)

- Edge Detail:  (Fiberglass and BioAcoustic FR-701 Fabric Wrapped - Square edge)(Sailcloth – Nylon Stitched Sealed Edge)(also available with custom edge details – square, beveled, radiused, mitered and stitched)

- Flame Spread:  (ASTM E 84), Class A

- Light Reflectance: (LR):  (ASTM E 1477), N/A

- Acoustical Absorption: (ASTM C423), (Fiberglass FR-701 Fabric 1.8 sabins per square foot)(BioAcoustic FR-701 1.14 sabins per square foot)(Sailcloth Fabric – 1.19 sabins per square foot)

- Acceptable Product:  Soundsoak Baffles item #_______ as manufactured by  Armstrong World Industries.

2.2.2	ATTACHMENT SYSTEM

- Installation Hardware

- Hardware is partially embedded.

PART 3 - EXECUTION

3.1	PREPARATION

- Measure each ceiling area and establish layout of Baffels.  Comply with reflected ceiling plans.  Coordinate panel layout with mechanical, electrical and sprinkler fixtures.

3.2	INSTALLATION

A.	Install Baffles in accordance with the manufacturer’s instructions and in compliance with the authorities having jurisdiction.

3.3	ADJUSTING AND CLEANING

A.	Replace damaged and broken panels.

B.	Clean exposed surfaces of Baffles per manufacturer’s instructions.

END OF SECTION