SECTION 08310

TYPE TER HORIZONTAL ACCESS DOOR SPECIFICATION

I. 			PART ONE - GENERAL

1.01		SUMMARY

A.	Work included: Furnishing and installing factory fabricated vault access doors

B.	Related Work: [Insert applicable specifications sections]

1.02		REFERENCES

A.	American Society for Testing and Materials (ASTM), 100 Bar Harbor Drive,

West Conshocken, PA 19428-2959; (610) 832-9585, fax (610) 832-9555

- ASTM A 36-93a: Standard Specification for Structural Steel

B.	International Organization for Standardization (ISO), ISO Central Secretariat, 1, ch. de la Voie-Creuse,

CP 56, CH-1211 Geneva 20, Switzerland, phone +41 22 749 01 11, fax +41 22 733 34 30

- ISO 9001:2008 Certified

1.03		SUBMITTALS

A.	Product Data: Provide manufacturer’s product data for all materials in this specification.

B.	Shop Drawings: Show profiles, accessories, location, and dimensions.

C.	Samples: Manufacturer to provide upon request; sized to represent material adequately.

D.	Contract Closeout: Vault access door manufacturer shall provide the manufacturer’s Warranty prior to the contract closeout [if applicable].

1.04 	PRODUCT HANDLING

A.	All materials shall be delivered in manufacturer’s original packaging.

- Store materials in a dry, protected, well-vented area. The contractor shall thoroughly inspect product upon receipt and report damaged material immediately to delivering carrier and note such damage on the carrier’s freight bill of lading.

C.	Remove protective wrapping immediately after installation [if applicable].

1.05 	SUBSTITUTIONS

A.	Proposals for substitution products shall be accepted only from bidding contractors and not less than (10) working days before bid due date. Contractor guarantees that proposed substitution shall meet the performance and quality standards of this specification.

1.06 	JOB CONDITIONS

A.	Verify that other trades with related work are complete before installing vault access door(s).

B.	Mounting surfaces shall be straight and secure; substrates shall be of proper width.

C.	Refer to the construction documents, shop drawings, and manufacturer’s installation instructions.

D.	Observe all appropriate OSHA safety guidelines for this work.

1.07 	WARRANTY/GUARANTEE

A.	Manufacturer’s standard warranty: Materials shall be free of defects in material and workmanship for a period of (5) five years from the date of purchase. Should a part fail to function in normal use within this period, manufacturer shall furnish a new part at no charge. Electrical motors, special finishes, and other special equipment (if applicable) shall be warranted separately by the manufacturers of those products.

B.	Manufacturer’s Quality System: Registered to ISO 9001:2008 Quality Standards including in-house engineering for product design activities.

II. 		PART TWO - PRODUCTS

2.01		MANUFACTURER

A.	The BILCO Company, P.O. Box 1203, New Haven, CT 06505; phone: 1-203-934-6363,

fax: 1-203-933-8478, Web: www.bilco.com

2.02		ACCESS DOOR

A.	Furnish and install where indicated on plans vault access door TER, size [insert: width (_____) x length (_____)]. Length denotes hinge side. The vault access door shall be single leaf. The vault access door shall be pre-assembled from the manufacturer.

B.	Performance characteristics:

1.	Cover: Shall be reinforced to support a minimum live load of 150 psf (732 kg/m2) with a maximum deflection of 1/150th of the span.

2.	Operation of the cover shall be smooth and easy with controlled operation throughout the entire arc of opening and closing.

3.	Operation of the cover shall not be affected by temperature.

C.		Cover shall have a 1” (25.4mm) fillable pan to receive concrete or a combination of concrete and [insert specified flooring material, including type, thickness, and weight _______]. All fill material to be furnished and installed by others in the field.

D.		Frame: Shall be 1/4” (6.3 mm) extruded aluminum with full anchor flange around the perimeter.

E.		Lifting mechanisms: Manufacturer shall provide the required number and size of compression spring operators enclosed in telescopic tubes to provide, smooth, easy, and controlled cover operation throughout the entire arc of opening and to act as a check in retarding downward motion of the cover when closing. The upper tube shall be the outer tube to prevent accumulation of moisture, grit, and debris inside the lower tube assembly. The lower tube shall interlock with a flanged support shoe fastened to a formed 1/4” gusset support plate.

F.		A removable exterior turn/lift handle with a spring loaded ball detent shall be provided to open the and the latch release shall be protected by a flush, gasketed, removable screw plug.

G.	Hardware:

1.	Hinges: Shall be a continuous heavy duty Type 316 stainless steel hinge that is accessible only when the cover is in the open position.

2.	Cover shall be equipped with an aluminum hold open arm that automatically locks the cover in the open position.

3.		Cover shall be fitted with the required number and size of	compression spring operators. Springs shall have an electrocoated acrylic finish.

4.	A Type 316 stainless steel snap lock with fixed handle shall be mounted on the underside of the cover.

5.	Hardware: Compression spring tubes shall be an anti-corrosive composite, all fasteners shall be Type 316 stainless steel material, and all other hardware shall be zinc plated and chromate sealed. [For installation in highly corrosive environments or when prolonged exposure to hot water or steam is anticipated, specify Type 316 stainless steel hardware].

H. 	Finishes: Factory finish shall be mill finish aluminum with bituminous coating applied to the exterior of the frame.

III. 		PART THREE - EXECUTION

3.01		INSPECTION

A.		Verify that the vault access door installation will not disrupt other trades. Verify that the substrate is dry, clean, and free of foreign matter. Report and correct defects prior to any installation.

3.02 	INSTALLATION

A.			Submit product design drawings for review and approval to the architect or specifier before fabrication.

B.		The installer shall check as-built conditions and verify the manufacturer’s vault access door details for accuracy to fit the application prior to fabrication. The installer shall comply with the vault access door manufacturer’s installation instructions.

C.	The installer shall furnish mechanical fasteners consistent with the vault access door manufacturer’s instructions.

END OF SECTION 08310