SECTION 07720

TYPE VERSAMOUNT® ACCESS HATCH SPECIFICATION

I. 		PART ONE - GENERAL

1.01		SUMMARY

A.	Work included: Furnishing and installing factory fabricated roof hatches

B.	Related Work: [Insert applicable specifications sections]

1.02		REFERENCES

A.	American Society for Testing and Materials (ASTM), 100 Bar Harbor Drive,

West Conshocken, PA 19428-2959; (610) 832-9585, fax (610) 832-9555

- ASTM A 36-93a: Standard Specification for Structural Steel

B.	International Organization for Standardization (ISO), ISO Central Secretariat, 1, ch. de la Voie-Creuse,

CP 56, CH-1211 Geneva 20, Switzerland, phone +41 22 749 01 11, fax +41 22 733 34 30

- ISO 9001:2008 Certified

1.03		SUBMITTALS

A.	Product Data: Provide manufacturer’s product data for all materials in this specification.

B.	Shop Drawings: Show profiles, accessories, location, and dimensions.

C.	Samples: Manufacturer to provide upon request; sized to represent material adequately.

D.	Contract Closeout: Roof hatch manufacturer shall provide the manufacturer’s Warranty prior to the contract closeout [if applicable].

1.04		PRODUCT HANDLING

A.	All materials shall be delivered in manufacturer’s original packaging.

B.	Store materials in a dry, protected, well-vented area. The contractor shall thoroughly inspect product upon receipt and report damaged material immediately to delivering carrier and note such damage on the carrier’s freight bill of lading.

C.	Remove protective wrapping immediately after installation [if applicable].

1.05		SUBSTITUTIONS

A.	Proposals for substitution products shall be accepted only from bidding contractors and not less than (10) working days before bid due date. Contractor guarantees that proposed substitution shall meet the performance and quality standards of this specification.

1.06		JOB CONDITIONS

A.	Verify that other trades with related work are complete before installing roof hatch(s).

B.	Mounting surfaces shall be straight and secure; substrates shall be of proper width.

C.	Refer to the construction documents, shop drawings, and manufacturer’s installation instructions.

D.	Coordinate installation with roof membrane and roof insulation manufacturer’s instructions before starting.

E.	Observe all appropriate OSHA safety guidelines for this work.

1.07		WARRANTY/GUARANTEE

A.	Manufacturer’s standard warranty: Materials shall be free of defects in material and workmanship for a period of five years from the date of purchase. Should a part fail to function in normal use within this period, manufacturer shall furnish a new part at no charge. Electrical motors, special finishes, and other special equipment (if applicable) shall be warranted separately by the manufacturers of those products.

B.	Manufacturer’s Quality System: Registered to ISO 9001:2008 Quality Standards including in-house engineering for product design activities.

II. 		PART TWO - PRODUCTS

2.01		MANUFACTURER

A.	The BILCO Company, P.O. Box 1203, New Haven, CT 06505,

1-203-934-6363, Fax: 1-203-933-8478, Web: www.bilco.com

2.02		ROOF HATCH

A.	Furnish and install where indicated on plans metal access hatch Type ______ (select: S, NB, L, EW) , size width: _______ x length: ______. Length denotes hinge side. The access hatch shall be single leaf. The access hatch shall be pre-assembled from the manufacturer with the exception of the handles that will be supplied for field installation.

B. 	Performance characteristics:

1.		Cover shall be reinforced to support a minimum live load of 40 psf (195 kg/m2) with a maximum 	deflection of 1/150th of the span or 20 psf (97 kg/m2) wind uplift.

2.		Operation of the cover shall be smooth and easy with controlled operation throughout the entire arc of 	opening and closing.

3.		Operation of the cover shall not be affected by temperature.

4.		Entire hatch shall be weathertight with fully welded corner joints on cover and curb.

C.	Cover: Shall be 14 gauge (1.9mm) paint bond G-90 galvanized steel [or 11 gauge (2.3mm) aluminum] [or 14 gauge (1.9mm) type 304 stainless steel] with a 3” (76mm) beaded flange with formed reinforcing members. Cover shall have an EPDM rubber gasket adhered to the cover interior to assure a continuous seal when compressed to the top surface of the curb.

D.	Cover insulation: Shall be fiberglass of 1” (25mm) thickness, fully covered and protected by a 22 gauge (.8mm) paint bond G-90 galvanized steel [or 18 gauge (1mm) aluminum] [or 22 gauge (.8mm) type 304 stainless steel] metal liner.

E.	Curb: Shall be 4” (101mm) in height and of 14 gauge (1.9mm) paint bond G-90 galvanized steel [or 11 gauge (2.3mm) aluminum] [or 14 gauge (1.9mm) type 304 stainless steel]. The curb shall be formed with a 2-5/8” (67mm) mounting flange and 1-1/2” (38mm) apron, fully welded at the corners. Stainless steel anchor clips are included.

F.	Curb insulation: Shall be rigid fiberglass of 1” (25mm) thickness throughout curb.

G.		Lifting mechanisms: Manufacturer shall provide compression spring operators enclosed in self-lubricating telescopic tubes to provide, smooth, easy, and controlled cover operation throughout the entire arc of opening and closing. The upper tube shall be the outer tube to prevent accumulation of moisture, grit, and debris inside the lower tube assembly. The lower tube shall interlock with a flanged support shoe [for aluminum construction: welded to the curb assembly; for steel construction: through bolted to the curb assembly].

H.		Hardware

1.		Heavy pintle hinges shall be provided

2.		Cover shall be equipped with a spring latch with interior and exterior turn handles

3.		Roof hatch shall be equipped with interior and exterior padlock hasps.

4.		The latch strike shall be a stamped component bolted to the curb assembly.

5.		Cover shall automatically lock in the open position with a rigid hold open arm equipped with a 1” 	(25mm) diameter red vinyl grip handle to permit easy release for closing.

6.		Compression spring tubes shall be an anti-corrosive composite material and all other hardware shall be zinc plated and chromate sealed. [For installation in highly corrosive environments or when prolonged exposure to hot water or steam is anticipated, specify Type 316 stainless steel hardware].

7.		Cover hardware shall be bolted into heavy gauge channel reinforcing welded to the underside of the 	cover and concealed within the insulation space.

I. 	Finishes: Factory finish shall be alkyd based red oxide primed steel [or mill finish aluminum] [or sand blast stainless steel].

III. 		PART THREE - EXECUTION

3.01		INSPECTION

A.		Verify that roof hatch installation will not disrupt other trades. Verify that the substrate is dry, clean, and free of foreign matter. Report and correct defects prior to any installation.

3.02 	INSTALLATION

A.		Submit product design drawings for review and approval to the architect or specifier before fabrication.

B.		The installer shall check as-built conditions and verify the manufacturer’s roof hatch details for accuracy to fit the application prior to fabrication. The installer shall comply with the roof hatch Manufacturer's installation instructions.

C.		The installer shall furnish mechanical fasteners consistent with the roof requirements.

END OF SECTION 07720