BRICKFORM® • (800) 483-9628 toll free • (909) 484-3318 fax  • 11061 Jersey Blvd.• Rancho Cucamonga, CA 91730

These specification guidelines are intended for use by a qualified concrete construction specifier and are not intended to be used as a project specification. Coordinate each section with other specification sections and the drawings. Prepare specification in accordance with ACI recommendations and other applicable industry guidelines.

Each section must be carefully reviewed and edited to meet the requirements of the project and local building code. Bracketed sections are intended for modifications regarding specific use or color selection and are included for the convenience of the specifier.

Section 03360

Specification GuidelineS

DRY-SHAKE COLOR HARDENER

Part 1 GENERAL

1.1 SUMMARY

A. Drawings and general provisions of Contract, including General and Supplementary Conditions and Division 1 Specification sections, apply to Work of this Section.

B. Section Includes:

1. Dry-shake colored hardener applied to [slabs-on-grade,] [sidewalks,] [driveways,] [patios,] [roads,] [parking lots,] [_______,] [and] [other exterior concrete pavements].

2. [Stamping/Imprinting]

1.2 RELATED SECTIONS

A. Section 03300 – Cast-in-Place Concrete.

B. Section 03330 – Architectural Concrete.

1.3 REFERENCES

Other useful publications about colored concrete include:

PCA PA124 - Finishing Concrete Slabs with Color and Texture.

PCA SP021 - Color and Texture in Architectural Concrete.

A. American Concrete Institute (ACI):

1. ACI 301 - Specification for Structural Concrete for Buildings.

2. ACI 302 IR - Recommended Practice for Concrete Floor and Slab Construction.

3. ACI 303.1 - Standard Specification for Cast-In-Place Architectural Concrete.

4. ACI 304 - Recommended Practice for Measuring, Mixing, Transporting and Placing of Concrete.

5. ACI 305R - Recommended Practice for Hot Weather Concreting.

6. ACI 306R - Recommended Practice for Cold Weather Concreting.

7. ACI 316 - Recommendations for Construction of Concrete Pavements and Bases.

B. American Society of Testing and Materials (ASTM):

1. ASTM C 494 -Standard Specification for Chemical Admixtures for Concrete.

1.4 SUBMITTALS

A. Comply with Section 01330 - Submittal Procedures.

B. Product Data: Submit manufacturer's complete technical information sheets for the following:

1. BRICKFORM® Color Hardener (dry-shake).

C. Design Mixes: For each type of concrete.

D. Samples for Initial Selection: Manufacturer's color charts showing full range of colors available.

E. Qualification Data: For firms indicated in 1.5 Quality Assurance, including list of completed projects.

1.5 QUALITY ASSURANCE

A. Concrete Contractor Qualifications: Concrete work shall be performed by firm with [three] [________] years experience and must have successfully completed [not less than 6 projects] [work of similar scope and quality] comparable in scale and complexity.

1. Statement of Contractor Qualifications

a. Submit list of [at least 6] completed projects including project name, project address and owner contact information.

B. Comply with the requirements of [ACI 301,] [ACI 302 IR,] [ACI 303.1,] [ACI 304,}

[ACI 305,] [ACI 306,] and [ACI 316].

Page 2 of 3

C. Obtain each specified material from same source and maintain high degree of consistency in workmanship throughout Project.

D. Notification of manufacturer's authorized representative shall be given at least 1-week before start of Work.

Please Note:

Color charts and small sample submittals provide only a general indication of color; color and appearance of completed work may differ. On large or critical projects, specify a mockup or field sample to demonstrate that proposed materials and workmanship produce acceptable concrete appearance.

E. Colored Concrete [Mockup] [Field Samples]:

1. Provide under provisions of Division 1 Section [Quality Control] [________.]

2. At location on Project selected by [Architect] [Landscape Architect] [Engineer], place and finish [10’ X 10’] [4’ X 4’]] [________] area.

3. Construct [mockup][sample panel] using processes and techniques intended for use on permanent work, including curing procedures. Include samples of control, construction, and expansion joints in sample panels. Individual workers who will perform the work for the Project will produce [mockup] [field sample].

4. Accepted [mockup] [field sample] provides visual standard for work of Section.

[Mockup] [Field sample] shall remain through completion of the work for use as a quality standard for finished work.

5. Remove [mockup] [field sample] when directed.

1.6 DELIVERY, STORAGE, AND HANDLING

A. Products shall be delivered in original factory packaging, unopened and undamaged. Packaging will bear identification of product, manufacturer’s identification and batch numbers. A technical information sheet and MSDS should be available for each product throughout the project.

B. Store the product in strict accordance with the manufacturer's recommendations in a location protected from damage, construction activity, and weather conditions.

1.7 JOBSITE CONDITIONS

A. Protection: Precautions shall be taken to protect surrounding areas and landscaping.

B. Schedule placement to minimize exposure to wind and hot sun

C. Avoid placement if rain, snow, or frost is forecast within a 24-hour period. Protect fresh concrete from moisture and freezing.

D. Comply with professional practices described in ACI 305R and ACI 306R.

1.8 PRE-JOB CONFERENCE

A. One week prior to placement of concrete a meeting shall be held to discuss the Project and application methods.

B. It is suggested that the [Architect,] [Landscape Architect,] [Engineer,] General Contractor, [Construction Manager,] Subcontractor, Ready-Mix Concrete Representative, and a Manufacturer's Representative be present.

PART 2 PRODUCTS

2.1 ACCEPTABLE MANUFACTURER

A. Dry-shake Colored Hardener: BRICKFORM® Color Hardener is a UV-resistant, engineered blend, designed to color the surface of new concrete.

BRICKFORM® Color Hardener as manufactured by BRICKFORM®, Rancho Cucamonga, CA conforms to the requirements of this specification.

Insert the following paragraph to aid in the finishing of concrete flatwork during the following conditions: Hot weather, low humidity, high winds or direct sunlight on concrete during placement. Also use when dry shake color hardeners are to be applied, or when concrete mixes have a low water-cement ratio, superplasticizers, or silica fume. This product reduces occurrence of shrinkage cracks and allows for proper application of dry-shake hardeners.

B. Evaporation Retarder: BRICKFORM® Evaporation Retarder, a VOC compliant, water-based, pigmented, film-forming compound that reduces evaporation of moisture in fresh concrete to prevent shrinkage cracks and facilitate finishing.

BRICKFORM® Evaporation Retarder as manufactured by BRICKFORM®, Rancho Cucamonga, CA conforms to the requirements of this specification.

Page 3 of 3

2.2 MATERIALS

- Dry-shake Colored Hardener: BRICKFORM® Color Hardener; BRICKFORM® factory pre-mixed and packaged, ready-to-use surface hardener.

Insert the following paragraph when specifying an evaporation retarder.

B. Evaporation Retarder. BRICKFORM® Evaporation Retarder, BRICKFORM® factory concentrated surface retardant.

2.3 COLORS

A. Colors:  As selected by [Architect] [Landscape Architect] [Engineer] from the BRICKFORM®, BRICKFORM® Color Hardener • Antique Release • Cem-Coat • Freestyle Color Selection Guide.

2.4 CONCRETE MIX DESIGN

A. Minimum Cement Content:  [5] [________] sacks per cubic yard of concrete.

B. Slump of concrete shall be consistent throughout Project at 4-inches or less. At no time shall slump exceed 5-inches. [If super plasticizers or mid-range water reducers are allowed, slump shall not exceed 8-inches.]

C. Do not add calcium chloride to mix as it causes mottling and surface discoloration.

D. Supplemental admixtures shall not be used unless approved by manufacturer.

E. Do not add water to the mix in the field.

F. Maximum air content shall not exceed 3 percent.

PART 3 EXECUTION

3.1 CONCRETE PLACEMENT

A. Move concrete into place with square-tipped shovels or concrete rakes.

B. Vibrators, when used, shall be inserted and withdrawn vertically.

C. Concrete shall be struck to specified level with wood or magnesium straight edge or mechanical vibrating screed.

D. The concrete surface shall be further leveled and consolidated with highway magnesium straight edge and/or magnesium bull float.

E. Mechanically float concrete surfaces as soon as concrete surface has taken its initial set and will support weight of a power float machine equipped with float shoes or combination blades and operator.

3.2 INSTALLATION – DRY-SHAKE COLORED HARDENER

A. Surrounding areas must be protected as described in 1.7 JOBSITE CONDITIONS, section A.

B. Apply 2/3 of specified application rate to freshly floated concrete surface. Bleed water shall not be present during or following application of first and second shake.

C. Do not throw dry-shake; distribute evenly by hand or mechanical spreader designed to apply floor hardeners

D. As soon as dry-shake material has absorbed moisture, indicated by uniform darkening of surface, mechanically float concrete surface a second time, just enough to bring moisture from base slab through dry-shake color hardener.

E. Immediately following second floating, apply remaining 1/3 of specified application rate. If applied by hand, broadcast in opposite direction of first application for a more uniform coverage. If a mechanical spreader is used, apply the same manner as previously described.

F. As soon as dry-shake material has absorbed moisture, mechanically float concrete surface a third time.

G. Do not add water to the surface.

H. Do not use plastic sheeting to cure the concrete.

I. After the initial curing period, a clear, non-staining, non-yellowing curing compound that conforms to ASTM C 309 may be used.

3.3 PROTECTION OF FINISHED WORK

A. Protect finished work under provisions of Division 1, Section: Temporary Facilities and Controls  [________.]

B. Prohibit foot or vehicular traffic on floor surface.

C. Barricade area to protect flooring.

F. Protect floor surface from damage until final inspection and acceptance by Owner.

BRICKFORM®				Local Contact:__________________________

Corporate Headquarters

(800) 483-9628 toll free 			Telephone Number______________________

(909) 484-3318 fax

(909) 484-3399 phone • 11061 Jersey Blvd. • Rancho Cucamonga, CA 91730