NOTE** This Section is based on systems of

NOTE** Butterfield Color®

NOTE** 625 West Illinois Avenue

NOTE** Aurora, Illinois 60506

NOTE** Telephone: 1-800-282-3388

NOTE** Fax: 630-906-1982

NOTE** www.butterfieldcolor.com

NOTE** This guide specification is intended to be used by a qualified construction specifier. It should not be used without appropriate modifications.

NOTE** Notes for review by specifier begin with “NOTE**.” Optional text requiring a selection is enclosed within brackets, e.g., “Color shall be [gray] [white].” Items requiring specifier input are indicated by a blank line enclosed within brackets, e.g., “Color: [_______].” Delete specifier notes and non-applicable optional text in final specification.

NOTE** Specification Section numbers and Section titles are according to The Construction Specifications Institute MasterFormat™ 1995 edition and indicated within brackets for MasterFormat™ 2004 edition.

GUIDE SPECIFICATION

NOTE ** Retain one Section number and title below and revise footer Section number and Section title to match.

SECTION 02769 [32 13 16.25]

STAINED CEMENT CONCRETE PAVEMENT [STAINED CONCRETE PAVING]

SECTION 09930 [09 93 50]

CONCRETE FLOOR STAIN

- GENERAL

  - SUMMARY

    - Section Includes:

NOTE** Delete optional text below for exterior concrete pavement or interior concrete floor that is not required.

      - Stained concrete for [pavement] [floors].

      - Sealer.

    - Related Sections:

NOTE** Delete subparagraphs below that are not required; add others as required.

      - [Division 2 Section “Cement Concrete Pavement”] [Division 32 Section “Concrete Paving”] for stained cast-in-place concrete pavement.

      - Division 3 Section “Cast-in-Place Concrete” for interior stained cast-in-place concrete.

      - Division 3 Section “Cementitious Stampable Overlay” for stained cementitious stampable overlays.

      - Division 3 Section “Concrete Resurfacing” for stained overlay cementitious toppings.

  - SUBMITTALS

    - Submit according to Conditions of the Contract and Division 1 Specification Sections.

    - Product Data: For each product indicated.

    - Samples for Initial Selection: Manufacturer's color charts.

    - Qualification Data: For Installer and manufacturer specified in Quality Assurance Article, including names and addresses of completed projects, architects, and owners.

  - QUALITY ASSURANCE

    - Installer Qualifications: Two year's experience with projects of similar scope and quality.

NOTE ** Delete paragraph above or below.

    - Installer Qualifications: Trained and approved by Manufacturer of stain products.

    - Manufacturer's Qualifications: Three year's experience manufacturing products required.

    - Source Limitations: Obtain products from same source throughout Project.

    - Field Samples: Locate at site and obtain approval before start of final work. Field samples shall be minimum [4 by 4 feet (1.2 by 1.2 m)] [_______].

      - If stain does not penetrate the surface, additional cleaning or preparation is required.

      - Demonstrate range of colors, finishes, and workmanship, including sealing procedures.

      - Approved field samples set quality standards for comparison with remaining work.

      - [Remove field samples when directed.] [Approved field samples may become part of the completed Work if undisturbed at completion of Project.]

NOTE ** Delete paragraph below if preinstallation conference is not required.

    - Preinstallation Conference: Conduct conference at site to comply with requirements of Division 1 Section “Project Meetings.”

  - DELIVERY, STORAGE AND HANDLING

    - Deliver materials in original packaging with labels intact.

    - Store in clean, dry and protected location, according to manufacturer's requirements.

  - PROJECT CONDITIONS

    - Environmental Requirements: Comply with stain manufacturer's instructions.

      - Do not apply stain if concrete substrate is frozen.

      - In hot weather, install stain in early morning or when surfaces are shaded.

      - Do not install stain if rain is predicted within 8 hours after installation.

      - Substrates shall be uniformly dry, and free of standing water.

- PRODUCTS

  - STAIN MATERIALS

    - Stain: Water based penetrating stain designed for permanently color concrete [floors] [pavements].

      - Product: Butterfield Color® Elements™ Transparent Concrete Stain.

      - Colors:  As selected by [Architect] [_______] from manufacturer's full range.

NOTE ** Delete subparagraph above if colors are selected and indicate color number in subparagraph below. Add a schedule if multiple colors are required. Delete subparagraph below if colors are not selected.

      - Colors:  Provide the following color[s]:

        - Color [1] [_______]: ELE-[_______].

    - Water: Potable.

  - SEALING MATERIALS

    - Clear, Solvent-Borne, Membrane-Forming Sealing Compound: ASTM C 309, non-yellowing, VOC-compliant, high-gloss, clear liquid.

      - Product: Butterfield Color® Clear-Guard™ Cure & Seal.

NOTE ** Delete paragraph below if matte finish for sealer is not required.

    - Flattening Paste: Manufacturer's standard product designed to reduce sealer gloss finish to matte finish.

      - Product: Butterfield Color® Flattening Paste.

NOTE ** Delete paragraph below if slip-resistive additive is not required for interior concrete.

    - Slip-Resistive Additive: Finely graded aggregate or polymer additive designed to add to sealer for slip-resistant surface.

  - MIXING

    - Combine one bottle of stain concentrate with three bottles of potable water in a plastic container and mix well, according to manufacturer's instructions. Do not use stain concentrate without mixing with water.

- EXECUTION

  - EXAMINATION

    - Examine substrate and conditions for compliance with requirements.

    - Do not proceed with stained concrete installation until unacceptable conditions are corrected.

  - SUBSTRATE PREPARATION

NOTE ** Delete paragraph below if concrete is not new.

    - Allow concrete to cure minimum 28 days before staining. Allow overlays to cure for a minimum of 72 hours at 70° F before staining.

    - Protect surfaces not to be stained from over-spray, run-off, and tracking.

    - Clean substrates thoroughly using Butterfield Color’s CHO Concrete Cleaner and water with a scrub brush, according to manufacturer's instructions. Do not use muriatic acid cleaners to clean substrates before staining.

    - For hard troweled surfaces, open surface of substrates by sanding with 80-grit sanding disk or black scrubbing pad. Test sanding in an unobtrusive location to assure that sand and aggregate are not exposed by excessive sanding.

    - Test substrates by spotting with water. If water beads, or does not penetrate surface, continue preparing substrates.

    - Rinse substrates until rinse water is clean. Collect cleaning residue by wet vacuuming or squeegee to drains.

    - Allow substrates to dry thoroughly prior to application of stains.

    - Divide substrates into small areas by natural stopping points, such as walls and joint lines.

    - Protection: Protect substrates during the construction period as follows:

      - Use safe-release blue painter's masking tape. Do not use adhesive or duct tape.

      - Prevent petroleum products from staining substrates.

      - Diaper hydraulic equipment.

      - Do not park vehicles on substrates.

      - Do not permit pipe-cutting equipment on substrates.

      - Do not place steel reinforcing bars on substrates to avoid rust stains.

      - Notify other trades of protection requirement before and after stain application.

NOTE ** Delete paragraph below if scoring is not required. Select optional text for time of scoring. If only one stain color, score after staining.

    - Scoring: Score concrete surfaces [before] [after] staining with diamond blades to a depth of 1/8-inch (3.2 mm). Rinse until rinse water is clean.

  - STAINING

    - Apply stain to concrete surfaces by high volume, low pressure (HVLP) sprayer, airless sprayer or foam brush, applying onto surface in a random motion, according to manufacturer's instructions.

    - Maintain a wet edge, working newly applied stain into edges of adjacent wet edges of previously treated surfaces. Maintain consistent saturation throughout application. Do not splash, drip, or puddle stain on substrates to prevent darker effects, unless desired.

    - If applied material appears wet for longer than 1 minute, do not attempt to apply more product.

NOTE ** Delete paragraph below if staining vertical surfaces is not required.

    - Apply stain to vertical surfaces in similar manner, from bottom working upward, avoiding excessive rundown of material on substrates to prevent darker streaks.

    - After initial application of stain has dried 8 hours minimum, evaluate surface to determine if sufficient color has been applied to match field samples. If evaluation indicates color is not sufficient, apply additional stain, allow to dry and evaluate.

  - SEALING

    - Preparation:  Wipe areas of the stained surface with a clean, dry cloth. If a color residue is retained on the cloth, then clean the surface by dry buffing with a white pad. Do not use water or any cleaning compound.

    - Sealer Application: Uniformly apply sealer in continuous operation by sprayer or short nap roller according to sealer manufacturer's instructions. After initial application is dry and tack free, apply a second coat.

      - Do not over apply or apply in a single heavy coat.

NOTE ** Delete subparagraph below if flattening paste not required for reducing sealer gloss finish to matte finish.

      - Thoroughly mix flattening paste in sealer according to manufacturer's instructions. Stir occasionally to maintain uniform distribution of paste.

NOTE ** Delete subparagraphs below for interior concrete. For interior applications subject to wetting, consider slip-resistant floor wax over sealer.

      - Thoroughly mix slip-resistant additive in sealer according to manufacturer's instructions. Stir occasionally to maintain uniform distribution of additive.

      - Verify adequacy of slip resistance before opening up surfaces to traffic.

  - PROTECTION

    - Protect stained concrete from damage or deterioration until date of Substantial Completion.

END OF SECTION

Copyright © 2006 Butterfield Color®. All Rights Reserved.

A copyright license to reproduce this specification is granted to specifying architects and engineers.