SPECIFICATION GUIDE

PART 1 - GENERAL

1.01 SUMMARY


A.
Section Includes: Provides weather-resistive barriers including sealing joints and protrusions through membranes, with accessories as required for complete installation.

1.02 SUBMITTALS


A.
Product Data: Submit manufacturer’s literature for each type of membrane.


B.
Samples: Submit samples of each type of material. 


C.
Quality Control Submittals: Submit manufacturer’s installation instructions for each material.

PART 2 - PRODUCTS

2.01 MANUFACTURER


A
.
Henry®, 1-800-773-4777.


B.

Substitutions: Comply with provisions of Division 1.

2.02 MATERIALS


A.       Vapor Permeable Weather-Resistive Barriers:  Asphalt saturated kraft Grade D 60-minute building paper.



1.
Types:

a.
Building Paper: Henry/ - Super Jumbo Tex® 60 Minute. 



2.
Reference Standard; Federal Specification UU-B-790a, Type 1, Grade D, Style 2.



3.
Reference Standard; ASTM E2556, Type II.




4.
Moisture Vapor Transmission: 10 perms minimum; ASTM E96.



5.
Water Resistance: 60 minutes, ASTM D779.

B.
Weather-Resistive Barrier Seam Tape: pressure sensitive tape as recommended by membrane manufacturer.



1.
Types:




a.
Sheathing Tape: Henry/ – Sheathing Tape.



2.
Roll Dimensions: 1-7/8” x 55 yards.



3.
Adhesive Type: Acrylic.

C.
Sealant: One component, moisture curing, non-sag, gun-grade elastomeric polymer for use as a sealant or liquid applied flashing.





1.
Types:

a.
Sealant: Henry/ – Moistop® Sealant.

2 Referenced Standards: Must meet AAMA 808.3 and ASTM C920. 

3.
Movement Capability: ±25%; ASTM C719.


4.
Max VOC: 9 g/L; ASTM D3960.


5.
Compatibility: Chemically compatible with flexible flashing; AAMA 713.

PART 3 - EXECUTION

3.01 PREPARATION


A.
Ensure items which pass through membrane are properly and rigidly installed, substrate is free of projections and irregularities which may be detrimental to proper installation of membrane.

3.02 INSTALLATION


A.
Apply membrane in accordance with manufacturer’s recommendations, laid smooth without folds or bunches of material.



1.
Seam Overlap: As recommended by membrane manufacturer for specific membrane material and application indicated.



2.
Sealing: Seal edges and items projecting through vapor retarders and vapor barriers.

B.         Inspect and repair membrane prior to application of finish material over membrane; tape tears, perforations and    similar damage.
All values presented are determined on specimens prepared to ASTM standards. These are nominal values and should not be interpreted as specifications. Henry believes all information and data herein to be accurate and reliable. 


Residential and Light Commercial Fortifiber® 1-2-3 Moisture Control System™ Warranty
This system is expressly warranted for a period of fifteen (15) years from purchase that it will perform to Henry Company LLC (and its subsidiaries and affiliates, including Fortifiber, LLC, hereinafter collectively “Henry”) published specifications when installed according to Henry installation procedures and accepted industry standards or following installation details prepared by a properly licensed design professional. In the event of a warranty claim, Henry will pay the cost of materials and labor to correct leaks that result solely from the failure of Henry product as expressly warranted above.
The installed system must include one of each of the following three items:

1. A Weather resistive barrier (Blueskin® VP 100, WeatherSmart®, WeatherSmart Drainable, WeatherSmart Commercial, Hydro Tex®, Super Jumbo Tex®, or Jumbo Tex brands)
2. Flashing (FortiFlash®, FortiFlash Butyl, FortiFlash 365, FortiFlex® Butyl, Moistop neXT®, Moistop PF®, Moistop EZ Seal® or Air-Bloc LF®)
3. Sealant (Moistop® Sealant and Liquid Flashing)

TO THE EXTENT PERMITTED BY APPLICABLE LAW, HENRY DISCLAIMS ALL OTHER WARRANTIES, GUARANTEES, CONDITIONS AND REPRESENTATIONS, EXPRESS OR IMPLIED, ORAL OR WRITTEN, STATUTORY OR OTHERWISE, INCLUDING BUT NOT LIMITED TO ANY IMPLIED CONDITIONS OR WARRANTIES AS TO THE MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

THIS WARRANTY SHALL BE THE OWNER’S SOLE AND EXCLUSIVE REMEDY AGAINST HENRY AND UNDER NO CIRCUMSTANCES SHALL HENRY BE LIABLE FOR (1) ANY DAMAGES BASED ON ANY LEGAL THEORY OF LIABILITY OTHER THAN THE EXCLUSIVE LIABILITY SET FORTH IN THIS WARRANTY OR (2) ANY DIRECT, INDIRECT, CONSEQUENTIAL, EXEMPLARY, SPECIAL, INCIDENTAL OR OTHER DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS, LOSS OF USE, OR DAMAGE TO THE BUILDING OR ITS CONTENTS.

IF ANY TERM OR CONDITION OF THIS WARRANTY IS LIMITED BY APPLICABLE LAW, THEN SUCH TERM OR CONDITION IS DEEMED SEVERED TO THE EXTENT SO LIMITED.

Warranty claims must be made in writing to Henry by email at warranty@henry.com no later than thirty (30) days after a potential claim covered under this warranty is or should by reasonable diligence have been discovered. Failure to give such notice within the time stated herein shall relieve Henry of all liability and any obligation to repair any leak otherwise covered by this warranty.

If the installation or use of these products do not qualify for this fifteen (15) warranty, Henry’s standard product warranty will apply.

For additional information on Henry’s warranty programs, including Henry’s standard product warranty, visit https://us.henry.com/warranty
© 2020 Henry Company 9/20

