This guide specification was prepared utilizing 3-part format recommended by the Construction Specifications Institute (CSI), and generally incorporates recommendations from their SectionFormat™/Page Format™, and MasterFormat™, latest Editions, insofar as practicable. Carefully review and edit the text to meet the Project requirements and coordinate this Section with the remainder of the Specifications and the Drawings. Where bracketed text is indicated, e.g. [text], make appropriate selection and delete the remainder of text within additional brackets, highlighting, and bold face type, if any. Consult the manufacturer for assistance in editing this guide specification for specific Project applications where necessary. This Specification was current at the time of publication but is subject to change. Please confirm the accuracy of these specifications with the manufacturer prior to use.

Section: 10 43 00

aed’S AND AED cabinets

PART 1 - GENERAL

- SUMMARY

- Section Includes:

- SPECIFIER: Revise subparagraphs below to suit Project requirements.

  - Automated External Defibrillators (AED’s)

  - AED Cabinets

- Related Requirements:

  - Drawings and general provisions of the Contract including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

  - Section 01 33 00 – Submittal Procedures: For administrative and procedural requirements for processing of submittals during the construction phase.

  - Section 01 77 00 – Closeout Procedures: For administrative and procedural requirements for the completion of the Work.

- SPECIFIER: Retain subparagraphs below to suit Project requirements; revise section numbers and titles per project requirements.

  - Section 09 91 00 – Painting: For field painting of access panes and frames.

1.02	REFERENCES

SPECIFIER:  Use care when indicating the edition date of the referenced standards; these standards are subject to regular review and updated accordingly.

A.	Reference Standards:

1.	American Heart Association (AHA):

a.	American Heart Association Guidelines for Cardiopulmonary Resuscitation and Emergency Cardiovascular Care - current Edition.

2.	ASTM International (ASTM):

a.	ASTM E814-11a, Standard Test Method for Fire Tests of Penetration Firestop Systems.

SPECIFIER: Indicate date of current Edition in affect in the Project location if known, to avoid problems with interpretation.

3.	International Code Council (ICC):

a.	International Building Code (IBC) - [current] Edition.

SPECIFIER:  Retain subparagraph below if wired alarms are specified for defibrillator cabinets.

5.	National Fire Protection Association (NFPA):

a.	NFPA 70-2011, National Electrical Code.

6.	Underwriters Laboratories, Inc.  (UL)

1.03	ACTION SUBMITTALS

SPECIFIER:  Revise Section number in the paragraph below to match that used in the Project Manual.

A.	Submit in accordance with Section [01 33 00] [other]:

1.	Product Data:

a.	Automated External Defibrillator (AED): Choose from an FDA-approved model or local government approved model for schools, public spaces including sports arenas, ball fields, and parks.

b.	Cabinets:  Materials description for defibrillator cabinets include roughing-in dimensions, details showing mounting methods, relationships to surrounding construction, door hardware, cabinet type and materials, trim style and door construction, door style and materials.

c.	Installation instructions for each product specified.

2.	Shop Drawings:

a.	Small-scale plans showing locations of defibrillator cabinets.

b.	Schedules showing each type of cabinet to ensure proper fit and function.

c.	Indicate installation procedures and accessories required for a complete installation.

3.	Samples:

a.	Defibrillator Cabinet Door and Trim Finishes:  For each type of exposed finish required, prepared on samples of size indicated below:

1)	Size:  6 inches (150 mm) square.

1.04	QUALITY ASSURANCE

A.	Comply with standards referenced in Article 1.02 - REFERENCES.

B.	Provide cabinets and accessories produced by a single manufacturer.

SPECIFIER:  Retain paragraph below for fire-rated, recessed and semi-recessed cabinets, if any.

C.	Fire-Rated, Defibrillator Cabinets:  Listed and labeled to comply with requirements in

ASTM E814 for fire-resistance rating of walls where they are installed.

SPECIFIER:  Retain paragraph below if wired alarms are specified for fire extinguisher cabinets.

- Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

1.05	DELIVERY, STORAGE, AND HANDLING

A.	Deliver, store, and protect defibrillator cabinets, AED’s and related materials using means and methods that will prevent damage, deterioration, or loss.

1.	Deliver components in manufacturer's original packaging, properly labeled for identification.

  - WARRANTY

- Cabinets Manufacturer’s Warranty: Provide manufacturer’s standard warranty. Materials shall be free of defects in material and workmanship for a period of one year from the date of purchase.

- AED’s: See manufacturer’s warranty.

PART 2 - PRODUCTS

- ACCEPTABLE MANUFACTURERS

- Acceptable Manufacturers:

- JL Industries - Part of Activar Construction Products Group, Inc.

- 9702 Newton Ave. S.

- Bloomington, MN 55431

- (800) 554-6077

- https://www.activarcpg.com

- Substitutions:  Manufacturers seeking approval of their products are required to comply with the Owner's Instructions to Bidders, generally contained in the Project Manual.

2.02	AUTOMATED EXTERNAL DIFIBRILLATORS

SPECIFIER:  Select AED’s Series and Type in paragraphs below.

A.	AED Model: [insert Model Number(s)].

1.	AED Model: Heartsine Samaritan Public Access Defibrillator [PAD-350]

Components:

- AED

- Battery and Electrodes

- Storage case

- AED Model: LIFEPAK CR2 Defibrillator – [LP-CR2-WIFI]

Components

    - AED

    - Storage case

    - Battery

    - Handle

    - USB Cable

    - Quick-Start Electrodes

2.03	1400 LIFESTART SERIES AED CABINETS

SPECIFIER:  Select cabinet Series and Type in paragraphs below.

SPECIFIER:  Refer to manufacturer's catalog for KEY to Cabinet Model Number System.

A.	Cabinet with Steel Trim and Door:  1400 Lifestart™ Series, Model [insert Cabinet Model Number(s)].

1.	Cabinet Style: [Recessed] [Semi-recessed] [Surface-mounted].

2.	Components:

a.	Tub:  Cold-rolled steel.

1)	Finish:  Factory-applied powder coat paint finish.

SPECIFIER:  Select White or optional color in options paragraph below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

- Standard Color:  White

- Optional Color: [insert color]:

  - Door and Trim Construction:  Cold-rolled steel; flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with zinc-plated with roller catch.

- Trim Dimensions:  1-3/4 inch (44.45 mm) face trim on door and frame.

SPECIFIER:  Select White or optional color in options paragraph below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

1)	Finish:  Factory-applied powder coat paint finish.

a)	Standard Color:  White.

b)	Optional Color: Color: [Black], [Bronze], [AMS Beige], [Flat Black], [SC Beige], [Gray], [Silver], [Red], [Yellow], [Orange].

c)	Trim Style and Depth:

SPECIFIER:  Select trim style and depth (Series number) in subparagraphs below from manufacturer's catalog based on available wall depth or desired appearance.

1)	Recessed Cabinet:  3/8 inch (9.53 mm) flat trim.

2)	Semi-Recessed Cabinet:

a)	Square Edge:  1-1/2 inch (38.10 mm).

b)	Rolled Edge:  3 inches (76.20 mm).

c)	Rolled Edge:	  4 inches (101.6 mm)

3)	Surface-Mount Cabinet:

a)	Square edge.

b)	Rolled edge.

3.	Alarm:  Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed

SPECIFIER:  Indicate fire-rating in subparagraph below; NOT available with surface-mounted cabinets.

3.	Fire-Rating: [Non-fire-rated] [Fire-Rated for 1-hour and 2-hour combustible and noncombustible wall systems].

SPECIFIER:  Select desired options from list below:

4.	Options:

Alarms:

a.	["Brigadier"] alarm with 85dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

5.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

SPECIFIER:  Select desired wall signs or custom door graphics below.

6.	Wall Signs and Cabinet Lettering:

a.	AED wall signs.[14S] [14TX] [14XS]

b.	Custom door graphics.

B.	Cabinet with Aluminum Trim and Door:  1400 Lifestart™ Series, Model [insert Cabinet Model Number(s)].

1.	Cabinet Style: [Recessed] [Semi-recessed] [Surface-mounted].

2.	Components:

a.	Tub (Recessed and Semi-recessed cabinets):  Cold-rolled steel.

1)	Finish:  Factory-applied powder coat paint finish.

SPECIFIER:  Select White or optional color under options below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

a)	Standard Color:  White.

SPECIFIER:  Select subparagraph above or below based on mounting style.

a.	Tub (Surface mount cabinets):  Anodized aluminum; clear color.

SPECIFIER:  Aluminum tub indicated in subparagraph below is available for recessed and semi-recessed cabinets if located in non-fire-rated wall construction; this aluminum tub is an additional cost.

a.	Tub:  Anodized aluminum; clear color.

b.	Door and Trim Construction:  Aluminum; flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with zinc-plated pull and roller catch.

1)	Finish:  Factory-anodized finish.

a)	Standard Finish:  Clear.

c.	Trim Style and Depth:

SPECIFIER:  Select trim style and depth (Series number) in subparagraphs below from manufacturer's catalog based on available wall depth or desired appearance.

1)	Recessed Cabinet:  3/8 inch (9.53 mm) flat trim.

2)	Semi-Recessed Cabinet:

a)	Square Edge:  1-1/2 inch (38.10 mm).

b)	Rolled Edge:  3 inches (76.20 mm).

c)	Rolled Edge:  4 inches (101.6 mm).

3)	Surface-Mount Cabinet:

a)	Square edge.

4)	Trim Dimensions:  1-3/4 inch (44.45 mm) face trim on door and frame.

3.	Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Indicate fire-rating in subparagraph below; NOT available with surface-mounted cabinets.

3.	Fire-Rating: [Non-fire-rated] [Fire-Rated for 1-hour and 2-hour combustible and noncombustible wall systems].

SPECIFIER:  Select desired options from list below:

4.	Options:

Alarms:

a.	["Brigadier"] alarm with 85dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

5.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

SPECIFIER:  Select desired wall signs or custom door graphics below.

6.	Wall Signs and Cabinet Lettering:

a.	AED wall signs.[14S] [14TX] [14XS]

b.	Custom door graphics.

7.	Finish:

a.	Finish: Dark bronze.

C.	Cabinet with Stainless Steel Trim and Door:  1400 Lifestart™ Series, Model [insert Cabinet Model Number(s)].

1.	Cabinet Style: [Recessed] [Semi-recessed] [Surface-mounted].

2.	Components:

a.	Tub (Recessed and Semi-recessed Cabinets):  Cold-rolled steel.

1)	Finish:  Factory-applied powder coat paint finish.

SPECIFIER:  Select White or optional color in subparagraph below; refer to manufacturer's catalog for additional color selections; optional colors are available at an additional cost.

a)	Standard Color:  White.

SPECIFIER:  Select subparagraph above or below based on mounting style.

a.	Tub (Surface mount cabinets):  Stainless steel; #4 directional satin finish.

SPECIFIER:  Select subparagraph below for recessed and semi-recessed cabinets if desired; stainless steel tubs for these mounting styles are an additional cost.

a.	Tub (Recessed and Semi-recessed Cabinets):  Stainless steel; #4 directional satin finish.

b.	Door and Trim Construction:  Stainless steel; flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with zinc-plated with roller catch.

SPECIFIER:  Select standard or custom finish in subparagraph below; custom finishes are an additional cost.

1)	Finish:  Factory-applied ground and polished finish.

a)	Standard Finish:  #4 directional satin finish.

SPECIFIER:  The #6 finish is nonreflective; the #7 and #8 finishes are considered reflective finishes, but the #7 finish will not show scratches as readily as a #8 finish will.

2)	Door Style:

c.	Trim Style and Depth:

SPECIFIER:  Select trim style and depth (Series number) in subparagraphs below from manufacturer's catalog based on available wall depth or desired appearance.

1)	Recessed Cabinet:  3/8 inch (9.53 mm) flat trim.

2)	Semi-Recessed Cabinet:

a)	Square Edge:  1-1/2 inch (38.10 mm).

b)	Rolled Edge:  3 inches (76.20 mm).

c)	Rolled Edge:  4 inches (101.6 mm).

3)	Surface-Mount Cabinet:

a)	Square edge.

b)	Rolled edge.

4)	Trim Dimensions:  1-3/4 inch (44.45 mm) face trim on door and frame.

3.	Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Indicate fire-rating in subparagraph below; NOT available with surface-mounted cabinets.

4.	Fire-Rating: [Non-fire-rated] [Fire-Rated for 1-hour and 2-hour combustible and noncombustible wall systems].

SPECIFIER:  Select desired options from list below:

5.	Options:

Alarms:

a.	["Brigadier"] alarm with 85dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

6.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

SPECIFIER:  Select desired wall signs or custom door graphics below.

7.	Wall Signs and Cabinet Lettering:

a.	AED wall signs.[14S] [14TX] [14XS]

b.	Custom door graphics.

8.	Finish:

a.	Finish: [#6 dull satin] [#7 directional polished] [#8 mirror-like] finish.

D.	Cabinet with [Bronze] [Brass] Trim and Door:  1400 Lifestart™ Series, Model [insert Cabinet Model Number(s)].

1.	Cabinet Style: [Recessed] [Semi-recessed].

2.	Components:

a.	Tub:  Cold-rolled steel.

1)	Finish:  Factory-applied powder coat paint finish.

SPECIFIER:  Select White or optional color in subparagraph below; refer to manufacturer's catalog for additional color selections; optional colors are available at an additional cost.

a)	Standard Color:  White.

SPECIFIER:  Select bronze or brass material in subparagraph below; bronze and brass hardware finishes generally match the door and trim finish selected.

b.	Door and Trim Construction: [Bronze] [Brass] (solid not plated); flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with [pull to match finish]  with roller catch.

1)	Bronze Finish:

a)	Standard Finish:  US10/BHMA 612 Satin bronze, clear coated.

SPECIFIER:  Select standard bronze finish above, or; see additional finishes in options subparagraph, available at an additional cost.

SPECIFIER:  Select bronze finish above or brass finish below as applicable.

2)	Brass Finish:

a)	Standard Finish:  US3/BHMA 605 Bright brass, clear coated.

SPECIFIER:  Select standard brass finish above, or; see additional finishes in options subparagraph, available at an additional cost.

3)	Door Style:

c.	Trim Style and Depth:

SPECIFIER:  Select trim style and depth (Series number) in subparagraphs below from manufacturer's catalog based on available wall depth or desired appearance.

1)	Recessed Cabinet:  3/8 inch (9.53 mm) flat trim.

2)	Semi-Recessed Cabinet:

a)	Square Edge:  1-1/2 inch (38.10 mm).

b)	Rolled Edge:  3 inches (76.20 mm).

3)	Trim Dimensions:  1-3/4 inch (44.45 mm) face trim on door and frame.

3.	Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Indicate fire-rating in subparagraph below

4.	Fire-Rating: [Non-fire-rated] [Fire-Rated for 1-hour and 2-hour combustible and noncombustible wall systems].

SPECIFIER:  Select desired options from list below:

5.	Options:

Alarms:

a.	["Brigadier"] alarm with 85dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

Alarms Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

SPECIFIER:  Select desired wall signs or custom door graphics below.

Wall Signs and Cabinet Lettering:

a.	AED wall signs.[14S] [14TX] [14XS]

b.	Custom door graphics.

SPECIFIER:  Select desired finish options below.

Optional Finishes:

a.	Optional Finish for Bronze: [US9/BHMA 611 Bright bronze, clear coated] [US10B/BHMA 613 Dark Oxidized satin bronze, oil-rubbed].

b.	Optional Finish for Brass: [US4/BHMA 606 Satin brass, clear coated].

2.04	1480 SERIES FREESTANDING AED CABINET & STAND

- Cabinet with Steel Trim, Door and Tub:

1. Components:

a.	Tub:  Cold-rolled steel.

b.	Door and Trim Construction:  Cold-rolled steel; flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with zinc-plated pull and roller catch.

SPECIFIER:  Select White or Silver Metallic, or optional color in subparagraph below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

c. 	Finish:  Factory-applied powder coat paint finish.

1.)	Standard Color: [White] [Silver Metallic].

SPECIFIER:  Select desired alarm or alarm contact type in subparagraph below; the 85 dB Commander

audible alarm is standard.

2.	Alarm:  Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

3.	Options:

Alarms:

a.	["Brigadier"] alarm with 85 dB (audible) Alarm (battery operated). Alarm stays on 				for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes 				strobe light when door opened. Alarm stays on for 2 minutes after door is closed.

.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

- Cabinet with Aluminum Trim, Door and Tub:

1. Components:

a.	Tub:  Anodized aluminum; clear color

b.	Door and Trim Construction:  Anodized aluminum; clear color; flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with zinc-plated pull and roller catch.

c. 	Finish:  Factory-applied powder coat paint finish.

a)	Standard Color: [White] [Silver Metallic].

2.	Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Select desired alarm or alarm contact type in subparagraph below; the 85 dB Commander audible alarm is standard.

3.	Options:

Alarms:

a.	["Brigadier"] alarm with 85 dB (audible) Alarm (battery operated). Alarm stays on 				for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes 				strobe light when door opened.  Alarm stays on for 2 minutes after door is closed.

4.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

C.	Freestanding Steel AED Stand:

1.	Construction:  Structural steel tubing which forms an inverted U-shape welded to a steel base plate.

2.	Finish:  Factory-applied powder coat paint finish.

SPECIFIER:  Select White or Silver Metallic, or optional color in subparagraph below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

a)	Standard Color: [White] [Silver Metallic].

b)	Optional Color: [insert color].

3.	Height:  52 inches (1320.8 mm).

2.05	1463 SERIES GALVANNEAL STEEL WATER-RESISTANT AED CABINETS

A.	Galvannealed Steel Cabinet Tub and Door: 1463 Series, Model [insert Cabinet Model Number(s)].

1.	Cabinet Style:  Surface-mounted.

2.	Components:  Tub and door constructed from galvannealed steel.  Door includes continuous bulb gasket to protect against water intrusion.

SPECIFIER:  Standard color is white, see options paragraph for additional color.

- Standard Color:  White

3.	Door Hardware:  Keyed turn.

4.	Cabinet Size:  15 inches by 15 inches by 9 inches (381 mm by 381 mm by 228.6 mm) deep.

SPECIFIER: Standard alarm is Commander 85dB alarm. Optional alarm or alarm contact type listed under options.

5.	Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Select desired options from list below:

6.	Options:

Alarms:

a.	["Brigadier"] alarm with 85 dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

7.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

SPECIFIER:  Select desired wall signs or custom door graphics below.

8.	Wall Signs and Cabinet Lettering:

a.	AED wall signs. [14S] [14TX] [14XS]

b.	Custom door graphics.

9.	Finish: Color option [Red]

2.06	1499 WATER-RESISTANT EXTERIOR AED CABINET

A.	White UV-inhibited halogen-free fiberglass reinforced polyester (FRP): 1499 Series, Model.

1.	Cabinet Style:  Surface-mounted.

2.	Components:  Tub and door constructed from fiberglass reinforced polyester (FRP).  Door includes door gasket seal to protect against water intrusion.

3.	Components: Hinge is full length stainless steel and includes stainless steel rivets and quick-opening latches.

4.	Components: Window is ¼” transparent Lexan polycarbonate. AED die cut (1-1/4” x 5-3/8”) and Warning decal (3/4” x 3-1/2”) included with cabinet.

SPECIFIER:  Standard color is white, no additional color options available.

- Standard Color:  White

5.	Door Hardware:  Keyed turn.

6.	Cabinet Size:  15 inches by 15 inches by 9 inches (381 mm by 381 mm by 228.6 mm) deep.

SPECIFIER: Standard alarm is Commander 85dB alarm. Optional alarm or alarm contact type listed under options.

7.	Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Select desired options from list below:

8.	Options:

Alarms:

a.	["Brigadier"] alarm with 85 dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

9.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

SPECIFIER:  Select desired wall signs or custom door graphics below.

10.	Wall Signs and Cabinet Lettering:

a.	AED wall signs. [14S] [14TX] [14XS]

b.	Custom door graphics.

2.07	1900 OLYMPUS SERIES DUAL CABINET FOR AED AND EMERGENCY OXYGEN

SPECIFIER:  Select cabinet Series and Type in paragraphs below.

SPECIFIER:  Refer to manufacturer's catalog for KEY to Cabinet Model Number System.

A.	Cabinet with Steel Trim and Door:  1900 Olympus Series, Model [insert Cabinet Model Number(s)].

1.	Cabinet Style: [Recessed] [Semi-recessed] [Surface-mounted].

2.	Components:

a.	Tub:  Cold-rolled steel.

1)	Finish:  Factory-applied powder coat paint finish.

SPECIFIER:  Standard color is white or choose optional color in subparagraph below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

a)	Standard Color:  White

b.	Door and Trim Construction:  Cold-rolled steel; flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with zinc-plated with roller catch.

SPECIFIER:  Standard color is white or choose optional color in subparagraph below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

1)	Finish:  Factory-applied powder coat paint finish.

a)	Standard Color:  White.

c.	Trim Style and Depth:

SPECIFIER:  Select trim style and depth (Series number) in subparagraphs below from manufacturer's catalog based on available wall depth or desired appearance.

1)	Recessed Cabinet:  3/8 inch (9.53 mm) flat trim.

2)	Semi-Recessed Cabinet:

a)	Square Edge:  1-1/2 inch (38.10 mm).

b)	Rolled Edge:  3 inches (76.20 mm).

3)	Surface-Mount Cabinet:

a)	Square edge.

4)	Trim Dimensions:  1-3/4 inch (44.45 mm) face trim on door and frame.

SPECIFIER: Standard alarm is Commander 85dB alarm. Optional alarm or alarm contact type listed under options. Note that the strobe alarm is available only on cabinets with 3” (76.20mm) semi-recessed trim and surface-mounted cabinets.

3.	Alarm: Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Indicate fire-rating in subparagraph below; NOT available with surface-mounted cabinets.

4.	Fire-Rating: [Non-fire-rated] [Fire-Rated for 1-hour and 2-hour combustible and noncombustible wall systems].

SPECIFIER:  Select desired options from list below:

5.	Options:

Alarms:

a.	["Brigadier"] alarm with 85 dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

6.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

7.	Oxygen:  Accommodates up to "D" size cylinder

SPECIFIER:  Select desired wall signs or custom door graphics below.

7.	Wall Signs and Cabinet Lettering:

a.	AED wall signs.[14S] [14TX] [14XS]

b.	Custom door graphics.

8.	Finish: Color Options: [Black], [Bronze], [AMS Beige], [Flat Black], [SC Beige], [Gray], [Silver], [Red], [Yellow], [Orange].

B.	Cabinet with Aluminum Trim and Door:  1900 Olympus Series, Model [insert Cabinet Model Number(s)].

1.	Cabinet Style: [Recessed] [Semi-recessed] [Surface-mounted].

2.	Components:

a.	Tub (Recessed and Semi-recessed cabinets):  Cold-rolled steel.

1)	Finish:  Factory-applied powder coat paint finish.

SPECIFIER:  Standard color is white or choose optional color in subparagraph below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

a)	Standard Color:  White.

SPECIFIER:  Select subparagraph above or below based on mounting style.

a.	Tub (Surface mount cabinets):  Anodized aluminum; clear color.

SPECIFIER:  Aluminum tub indicated in subparagraph below is available for recessed and semi-recessed cabinets if located in non-fire-rated wall construction; this aluminum tub is an additional cost.

a.	Tub:  Anodized aluminum; clear color.

b.	Door and Trim Construction:  Aluminum; flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with zinc-plated with roller catch.

SPECIFIER: Standard aluminum finish is clear. Optional color listed under Options subparagraph and are an additional cost. Surface mount models tub finish matches door and trim.

1)	Finish:  Factory-anodized finish.

a)	Standard Color:  Clear.

c.	Trim Style and Depth:

SPECIFIER:  Select trim style and depth (Series number) in subparagraphs below from manufacturer's catalog based on available wall depth or desired appearance.

1)	Recessed Cabinet:  3/8 inch (9.53 mm) flat trim.

2)	Semi-Recessed Cabinet:

a)	Square Edge:  1-1/2 inch (38.10 mm).

b)	Rolled Edge:  3 inches (76.20 mm).

3)	Surface-Mount Cabinet:

a)	Square edge.

4)	Trim Dimensions:  1-3/4 inch (44.45 mm) face trim on door and frame.

SPECIFIER: Standard alarm is Commander 85dB alarm. Optional alarm or alarm contact type listed under options. Note that the strobe alarm is available only on cabinets with 3” (76.20mm) semi-recessed trim and surface-mounted cabinets.

3.	Alarm: Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Indicate fire-rating in subparagraph below; NOT available with surface-mounted cabinets.

4.	Fire-Rating: [Non-fire-rated] [Fire-Rated for 1-hour and 2-hour combustible and noncombustible wall systems].

SPECIFIER:  Select desired options from list below:

5.	Options:

Alarms:

a.	["Brigadier"] alarm with 85 dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

6.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

7.	Oxygen:  Accommodates up to "D" size cylinder.

SPECIFIER:  Select desired wall signs or custom door graphics below.

8.	Wall Signs and Cabinet Lettering:

a.	AED wall signs.[14S] [14TX] [14XS]

b.	Custom door graphics.

9.	Finish: Color Options for Tubs: [Black], [Bronze], [AMS Beige], [Flat Black], [SC Beige], [Gray], [Silver], [Red], [Yellow], [Orange].

C.	Cabinet with Stainless Steel Trim and Door:  1900 Olympus Series, Model [insert Cabinet Model Number(s)].

1.	Cabinet Style: [Recessed] [Semi-recessed] [Surface-mounted].

2.	Components:

a.	Tub (Recessed and Semi-recessed Cabinets):  Cold-rolled steel.

1)	Finish:  Factory-applied powder coat paint finish.

SPECIFIER:  Standard color is white or choose optional color in subparagraph below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

a)	Standard Color:  White

SPECIFIER:  Select subparagraph above or below based on mounting style.

a.	Tub (Surface mount cabinets):  Stainless steel; #4 directional satin finish.

SPECIFIER:  Select subparagraph below for recessed and semi-recessed cabinets if desired; stainless steel tubs for these mounting styles are an additional cost.

a.	Tub (Recessed and Semi-recessed Cabinets):  Stainless steel; #4 directional satin finish.

b.	Door and Trim Construction:  Stainless steel; flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with zinc-plated with roller catch.

SPECIFIER: Standard stainless steel finish is #4 directional satin finish. Other finish options are listed in the Options subparagraph and are an additional cost.

1)	Finish:  Factory-applied ground and polished finish.

a)	Standard Finish:  #4 directional satin finish.

c.	Trim Style and Depth:

SPECIFIER:  Select trim style and depth (Series number) in subparagraphs below from manufacturer's catalog based on available wall depth or desired appearance.

1)	Recessed Cabinet:  3/8 inch (9.53 mm) flat trim.

2)	Semi-Recessed Cabinet:

a)	Square Edge:  1-1/2 inch (38.10 mm).

b)	Rolled Edge:  3 inches (76.20 mm).

3)	Surface-Mount Cabinet:

a)	Square edge.

4)	Trim Dimensions:  1-3/4 inch (44.45 mm) face trim on door and frame.

SPECIFIER: Standard alarm is Commander 85dB alarm. Optional alarm or alarm contact type listed under options. Note that the strobe alarm is available only on cabinets with 3” (76.20mm) semi-recessed trim and surface-mounted cabinets.

3.	Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Indicate fire-rating in subparagraph below; NOT available with surface-mounted cabinets.

4.	Fire-Rating: [Non-fire-rated] [Fire-Rated for 1-hour and 2-hour combustible and noncombustible wall systems].

SPECIFIER:  Select desired options from list below:

5.	Options:

Alarms:

a.	["Brigadier"] alarm with 85 dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

6.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems

7.	Oxygen:  Accommodates up to "D" size cylinder.

SPECIFIER:  Select desired wall signs or custom door graphics below.

8.	Wall Signs and Cabinet Lettering:

a.	AED wall signs.[14S] [14TX] [14XS]

b.	Custom door graphics.

SPECIFIER:  The #6 finish is nonreflective; the #7 and #8 finishes are considered reflective finishes, but the #7 finish will not show scratches as readily as a #8 finish will.

9.	Finish: Optional Finish: [#6 dull satin] [#7 directional polished] [#8 mirrorlike] finish.

D.	Cabinet with [Bronze] [Brass] Trim and Door:  1900 Olympus Series, Model [insert Cabinet Model Number(s)].

1.	Cabinet Style: [Recessed] [Semi-recessed].

2.	Components:

a.	Tub:  Cold-rolled steel.

1)	Finish:  Factory-applied powder coat paint finish.

SPECIFIER:  Standard color is white or choose optional color in options subparagraph below; refer to manufacturer's catalog for additional color selections; optional colors are an additional cost.

a)	Standard Color:  White.

SPECIFIER:  Select bronze or brass material in subparagraph below and select operating hardware material; bronze and brass hardware finishes generally match the door and trim finish selected.

b.	Door and Trim Construction: [Bronze] [Brass] (solid not plated); flush doors with 5/8 inch (15.88 mm) doorstop attached by continuous hinge and equipped with [pull to match finish] [black ABS recessed pull] with roller catch.

1)	Bronze Finish:

a)	Standard Finish:  US10/BHMA 612 Satin bronze, clear coated.

SPECIFIER:  Select standard bronze finish above, or; select an optional bronze finish listed in options subparagraph, which are available at an additional cost.

SPECIFIER:  Select bronze finish above or brass finish below as applicable.

2)	Brass Finish:

a)	Standard Finish:  US3/BHMA 605 Bright brass, clear coated.

SPECIFIER:  Select standard brass finish above, or; select optional brass finish listed in options subparagraph, which is available at an additional cost.

c.	Trim Style and Depth:

SPECIFIER:  Select trim style and depth (Series number) in subparagraphs below from manufacturer's catalog based on available wall depth or desired appearance.

1)	Recessed Cabinet:  3/8 inch (9.53 mm) flat trim.

2)	Semi-Recessed Cabinet:

a)	Square Edge:  1-1/2 inch (38.10 mm).

b)	Rolled Edge:  3 inches (76.20 mm).

3)	Trim Dimensions:  1-3/4 inch (44.45 mm) face trim on door and frame.

SPECIFIER: Standard alarm is Commander 85dB alarm. Optional alarm or alarm contact type listed under options. Note that the strobe alarm is available only on cabinets with 3” (76.20mm) semi-recessed trim and surface-mounted cabinets.

3.	Alarm: Commander (audible) cabinet-mounted alarm standard (battery operated) to protect against theft or tampering. Alarm deactivated when door is closed.

SPECIFIER:  Indicate fire-rating in subparagraph below. Not available with surface mount cabinet.

4.	Fire-Rating: [Non-fire-rated] [Fire-Rated for 1-hour and 2-hour combustible and noncombustible wall systems].

SPECIFIER:  Select desired alarm or alarm contact type in subparagraph below; the 85 dB Commander audible alarm is standard.

5.	Options:

Alarms:

a.	["Brigadier"] alarm with 85 dB (audible) Alarm (battery operated). Alarm stays on for 2 minutes after door is closed.

b. 	[Strobe Alarm] siren strobe alarm with 85dB (audible) warning horn and flashes strobe light when door is opened.  Alarm stays on for 2 minutes after door is closed.

6.	Alarm Contacts:  Contact devices

a.	[14MAG] Magnetic Door Contact for existing alarm systems.

b.	[14RR] Roller Reed Ball Contact for existing alarm systems.

7.	Oxygen:  Accommodates up to "D" size cylinder.

SPECIFIER:  Select desired wall signs or custom door graphics below.

8.	Wall Signs and Cabinet Lettering:

a.	AED wall signs.[14S] [14TX] [14XS]

b.	Custom door graphics.

SPECIFIER: Select optional finishes listed below. Optional finishes are an additional cost.

9.	Finishes:

a.	Bronze Optional Finish: [US9/BHMA 611 Bright bronze, clear coated] [US10B/BHMA 613 Dark Oxidized satin bronze, oil-rubbed].

b.	Brass Optional Finish:  US4/BHMA 606 Satin brass, clear coated

PART 3 - EXECUTION

3.01	EXAMINATION

A.	Examine walls and partitions for suitable framing depth and blocking where recessed and semi-recessed cabinets will be installed and blocking where surface mounted cabinets will be installed.

1.	Notify the Contractor in writing of conditions detrimental to proper and timely completion of the installation.

2.	Proceed with installation only after unsatisfactory conditions have been corrected.

3.02	INSTALLATION

A.	Install cabinets in locations and at mounting heights indicated, or if not indicated, at heights to comply with applicable regulations of governing authorities.

1.	Prepare recesses in walls for defibrillator cabinets as required by type and size of cabinet and style of trim and to comply with manufacturer's instructions.

2.	Securely fasten cabinets to structure, square and plumb, to comply with manufacturer=s instructions.

3.	Maintain fire ratings where cabinets are recessed into fire-rated wall systems.

SPECIFIER:  Retain paragraph below for wall signs which identify an AED location.

B.	Wall Signs:

1.	Location:  Where shown or directed.

2.	Apply on walls after field painting is completed and has been accepted.

SPECIFIER:  Retain paragraph below for fire extinguisher cabinets unless engraved or etched text is specified.

C.	Cabinet Lettering:

1.	Location: [Face of door framing] [Face of glass surface] [Where shown or directed].

2.	Apply lettering on field-painted cabinets after painting is complete and has been accepted.

SPECIFIER:  Retain subparagraph above for field-painted cabinets, or; retain below for factory-finished cabinets.

3.	Apply lettering on factory-finished (no further painting specified) cabinets either at the factory or just prior to Substantial Completion.

3.03	ADJUSTING AND CLEANING

A.	Remove temporary protective coverings and strippable films, if any, as defibrillator cabinets are installed unless otherwise indicated in manufacturer's written installation instructions.

B.	Adjust cabinet doors to operate easily without binding.  Verify that integral locking devices operate properly.

C.	On completion of cabinet installation, clean interior and exterior surfaces as recommended by manufacturer.

D.	Touch up marred finishes or replace cabinets that cannot be restored to factory-finished appearance.  Use only materials and procedures recommended or furnished by cabinet manufacturer.

E.	Replace cabinets that have been damaged or have deteriorated beyond successful repair by finish touchup or similar minor repair procedures.

END OF SECTION