This guide specification was prepared utilizing 3-part format recommended by the Construction Specifications Institute (CSI), and generally incorporates recommendations from their SectionFormat™/Page Format™, and MasterFormat™, latest Editions, insofar as practicable. Carefully review and edit the text to meet the Project requirements and coordinate this Section with the remainder of the Specifications and the Drawings. Where bracketed text is indicated, e.g. [text], make appropriate selection and delete the remainder of text within additional brackets, highlighting, and bold face type, if any. Consult the manufacturer for assistance in editing this guide specification for specific Project applications where necessary. This Specification was current at the time of publication but is subject to change. Please confirm the accuracy of these specifications with the manufacturer prior to use.

Section: 08 31 00

Floor doors

PART 1 - GENERAL

SUMMARY

Section Includes: Floor doors for access between building floors

REFERENCES

- Aluminum Diamond Tread Plate: ASTM B632-02, ¼ inch 6061-T6 aluminum with mill finish.

- Aluminum Extrusion: 6061-T6 aluminum.

- Stainless Steel Hardware: ASTM A276 stainless steel, No. 304 finish.

- General: Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for recommendations for applying the designated finishes.

COORDINATION

- If product is to be recessed, coordinate with concrete work so that products are available for placing integrally with floor slabs.

SUBMITTALS

- Product Data: Manufacturers product specifications, installation and maintenance instructions.

Shop Drawings: Submit shop drawings including dimensions, profiles, location, and adjacent construction.

Installation Instructions and Operation & Maintenance: Provide installation instructions, operation and maintenance requirements.

QUALITY ASSURANCE

- Manufacturer: Minimum 5 years experience manufacturing similar products.

- Installer: Minimum 2 years’ experience installing similar products.

delivery, storage and handling

- Deliver product in manufacturer’s original packaging.

- Deliver materials to Project site ready to use.

- Store materials under cover in a dry, clean location.

WARRANTY

- Manufacturer’s Warranty: Provide manufacturer’s standard warranty. Materials shall be free of defects in material and workmanship for a period of five years from the date of purchase.

PART 2 - PRODUCTS

ACCEPTABLE MANUFACTURERS

- Acceptable Manufacturers:

- JL Industries – part of Activar Construction Products Group, Inc.

- 9702 Newton Ave. S.

- Bloomington, MN 55431

- (800) 554-6077

- www.activarcpg.com

Substitutions:  Manufacturers seeking approval of their products are required to comply with the Owner's Instructions to Bidders, generally contained in the Project Manual.

non-fire-rated floor door

- Angled Floor Door – Model FLDA

Frame Construction: 1/4-inch mill finish aluminum, angle profile with bituminous coating on outer frame.

Opening is order size: [24 by 24 inches (610 by 610 mm)] [24 by 36 inches (610 by 914 mm)] [30 by 30 inches (762 by 762 mm)] [30 by 48 inches (762 by 1219 mm)] [36 by 36 inches (914 by 914 mm)] [36 by 48 inches (914 by 1219 mm)] [37 by 30 inches (940 by 762 mm)] [42 by 42 inches (1067 by 1067 mm)] [48 by 48 inches (1219 by 1219 mm)] [60 by 60 inches (1524 by 1524 mm)] [Insert Dimensions]

Door: [Single] [Double] leaf, 1/4-inch thick, diamond-pattern mill-finish, reinforced, aluminum plate.

Load: 300 psf pedestrian live load and a maximum deflection of 1/150 of span.

Hardware:

Material and Finish: Type 304 stainless steel, including latch and lifting mechanism assemblies, hold-open arms and brackets, hinges, pins, and fasteners.

Hinges: Type 304 stainless steel.

Operating Mechanism: Type 304 stainless steel gas assist opener, heavy-duty hold-open arm that automatically locks door open at 90 degrees, release handle with vinyl grip that allows for one-handed closure, and recessed lift handle.

Latch: 304 Stainless steel slam latch with flush key operator on exterior and handle opener on inside. Removable stainless 18-8 threaded plug and 5/16″ steel square key included.

Optional Lock: [304 Stainless Padlock hasp in addition to above latch] [Recessed padlock hasp].

Angled Existing Door – Model FLDE for Retrofit Openings

Frame Construction: 3/16-inch extruded aluminum, mill finish.

Opening is order size: [24 by 24 inches (610 by 610 mm)] [24 by 36 inches (610 by 914 mm)] [30 by 30 inches (762 by 762 mm)] [30 by 48 inches (762 by 1219 mm)] [36 by 36 inches (914 by 914 mm)] [36 by 48 inches (914 by 1219 mm)] [37 by 30 inches (940 by 762 mm)] [42 by 42 inches (1067 by 1067 mm)] [48 by 48 inches (1219 by 1219 mm)] [60 by 60 inches (1524 by 1524 mm)] [Insert Dimensions]

Door: [Single] [Double] leaf, ¼-inch thick reinforced aluminum diamond plate, mill finish.

Load: 300 psf pedestrian live load and a maximum deflection of 1/150 of span.

Hardware:

Material and Finish: Type 304 stainless steel, including latch and lifting mechanism assemblies, hold-open arms and brackets, hinges, pins, and fasteners.

Hinges: Type 304 stainless steel.

Operating Mechanism: Type 304 stainless steel gas assist opener, heavy-duty hold-open arm that automatically locks door open at 90 degrees, release handle with vinyl grip that allows for one-handed closure, and recessed lift handle.

304 Stainless steel slam latch with flush key operator on exterior and handle opener on inside. Removable stainless 18-8 threaded plug and 5/16″ steel square key included.

Optional Lock: [304 Stainless Padlock hasp in addition to above latch] [Recessed padlock hasp].

Drainable Door – Model FLDD with Integral Channel Drainage System

Frame: Engineered gutter frame design is 1/4-inch extruded aluminum shapes with mill finish and bituminous coating on outer frame.  Channel frame with 1-1/2 inch NPT female threaded aluminum pipe fitting for water drainage.

Opening is order size: [24 by 24 inches (610 by 610 mm)] [24 by 36 inches (610 by 914 mm)] [30 by 30 inches (762 by 762 mm)] [30 by 48 inches (762 by 1219 mm)] [36 by 36 inches (914 by 914 mm)] [36 by 48 inches (914 by 1219 mm)] [37 by 30 inches (940 by 762 mm)] [42 by 42 inches (1067 by 1067 mm)] [48 by 48 inches (1219 by 1219 mm)] [60 by 60 inches (1524 by 1524 mm)] [Insert dimension].

Door: [Single] [Double] leaf, reinforced ¼ inch aluminum diamond plate, mill finish.

Load: 300 psf pedestrian live load and a maximum deflection of 1/150 of span.

Hardware:

Material and Finish: Type 304 stainless steel, including latch and lifting mechanism assemblies, hold-open arms, and brackets, hinges, pins, and fasteners.

Hinges: Type 304 stainless steel.

Operating Mechanism: Type 304 stainless steel gas assist opener, heavy-duty hold-open arm that automatically locks door open at 90 degrees, release handle with vinyl grip that allows for one-handed closure, and recessed lift handle.

304 Stainless steel slam latch with flush key operator on exterior and handle opener on inside. Removable stainless 18-8 threaded plug and 5/16″ steel square key included.

Optional Lock: [304 Stainless Padlock hasp in addition to above latch]

Recessed Door – Model FLDR with Recessed Door

Frame Construction: 1/4-inch x 3-inch by 3-inch angle aluminum with 1/8-inch lip for flooring infill and bolt-on masonry anchors. Mill finish.

Order Size is Frame Inside Dimension: [24 by 24 inches (610 by 610 mm)] [30 by 30 inches (762 by 762 mm)] [30 by 36 inches (762 by 914 mm)] [30 by 48 inches (762 by 1219 mm)] [36 by 30 inches (914 by 762 mm)] [36 by 36 inches (914 by 914 mm)] [36 by 48 inches (914 by 1219 mm)] [42 by 42 inches (1067 by 1067 mm)] [Insert Dimensions]

Door: Single leaf, 1/4-inch smooth aluminum plate with 1/8-inch lip for application of finish material.

Load: 150 psf pedestrian live load.

Hardware:

Hinges: Aluminum arm-type hinges with 304 stainless steel bolt.

Spring: Chrome silicone alloy steel torsion bar.

Hold Open Arm: Aluminum automatic locking hold-open arm with red vinyl grip handle.

Latch: Zinc-plated steel slam latch with flush key operator on exterior and handle opener on inside. Chrome plated 5/16” steel square key included.

Options

[Bituminous coating on outer frame]

[Neoprene gasketing]

[Door and frame with 3/16” lip for flooring infill].

[304 stainless steel slam latch with flush key operator on exterior and handle opener on inside]

PART 3 – EXECUTION

3.01	EXAMINATION

- Examine area and conditions affecting installation. Do not proceed with installation until unsatisfactory conditions have been met.

INSTALLATION

- Install system level and plumb and in accord with manufacturers written instructions, original design and reference standards.

PROTECTION

- If more than one type of floor door is required, provide a schedule similar to the following example below. Use the corresponding type designations here and on the drawings.

Type 1: [Describe by model]

Type 2: [Describe by model]

END OF SECTION