This guide specification was prepared utilizing 3-part format recommended by the Construction Specifications Institute (CSI), and generally incorporates recommendations from their SectionFormat™/Page Format™, and MasterFormat™, latest Editions, insofar as practicable. Carefully review and edit the text to meet the Project requirements and coordinate this Section with the remainder of the Specifications and the Drawings. Where bracketed text is indicated, e.g. [text], make appropriate selection and delete the remainder of text within additional brackets, highlighting, and bold face type, if any. Consult the manufacturer for assistance in editing this guide specification for specific Project applications where necessary. This Specification was current at the time of publication but is subject to change. Please confirm the accuracy of these specifications with the manufacturer prior to use.

SECTION 10 26 13

CORNER GUARDS – CGSS and CGSH SERIES

PART 1 - GENERAL

1.01	SUMMARY

A.	Section Includes:

SPECIFIER: Revise subparagraphs below to suit Project requirements.

1.	Corner Guards, Surface Mount.

B.	Related Requirements:

1.	Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section

SPECIFIER: Revise section numbers and titles in subparagraphs below per CSI MasterFormat™ and Project requirements.

SPECIFIER: Delete Section 01 21 00 - Allowances if locations are clearly shown on the Drawings and sizes are either indicated alongside the locations or specified herein.

2.	Section 01 21 00 - Allowances:  For cash or quantity allowances covering corner guards.

3.	Section 01 33 00 - Submittal Procedures:  For administrative and procedural requirements for processing of submittals during the construction phase.

4.	Section 01 77 00 - Closeout Procedures:  For administrative and procedural requirements for completion of the Work.

SPECIFIER: Retain products and work included in this Section that are covered by cash or quantity allowance.  Do not include amounts. Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the allowance.

1.02	REFERENCES

SPECIFIER:  Use care when indicating the edition date of the referenced standards; these standards are subject to regular review, and updated; accordingly, review list carefully for inclusion.

A.	Reference Standards:

1.	ASTM International (ASTM):

a.	ASTM A240, Material specification for stainless steel.

1.03	ACTION SUBMITTALS

SPECIFIER:  Revise Section number in the paragraph below to match that used in the Project Manual.

A.	Submit in accordance with Section [01 33 00] [other]:

1.	Product Data:

a.	Materials description for corner guards including details showing mounting type, relationships to surrounding construction, materials, and construction.

b.	Installation instructions for each product specified.

2.	Shop Drawing: Submit shop drawings showing components, dimensions, anchorage details.

3. 	Samples: Submit for approval 12 inches (304.8 mm) long sample for each model specified.

1.04	QUALITY ASSURANCE

A.	Comply with standards referenced in Article 1.02 REFERENCES.

B.	Provide corner guards produced by a single manufacturer.

1.05	DELIVERY, STORAGE, AND HANDLING

A.	Deliver, store, and handle corner guards using means and methods that will prevent damage, deterioration, or loss.

1.	Deliver units in manufacturer's original packaging, properly labeled for identification.

2. 	Store materials indoors in a clean, dry area protected from damage and in accordance with manufacturer’s instructions.

3. 	Protect materials during handling and installation to prevent damage.

PART 2 - PRODUCTS

2.01	CORNER GUARD MANUFACTURERS

A.	Acceptable Manufacturers:

JL Industries, part of Activar Construction Products Group, Inc.

9702 Newton Ave. S.

Bloomington, MN  55431

(800) 554-6077

www.activarcpg.com

sales@activarcpg.com

B.	Substitutions:  Manufacturers seeking approval of their products are required to comply with the Owner's Instructions to Bidders, generally contained in the Project Manual.

2.02	STAINLESS STEEL SURFACE MOUNT CORNER GUARD

- STYLE CGSS STAINLESS STEEL CORNERGUARD

1.	Model No.: CGSS corner guard exposed surfaces shall be free of imperfections.

2.	Material: Type 304 stainless steel with # 4 finish, 18 gauge.

3.	Dimensions:

a. Angle [90 degrees], [135 degrees], [Endwall], [other]

a. Wing width: [1-1/2 inches (38 mm)], [2 inches (50.8 mm)], [2-1/2 inches (63.5 mm)], [3 inches (76.2 mm)], [3-1/2 inches (88.9 mm)], [other]

b. Height: [4 feet (1219 mm)], [other] up to 9 feet (2740 mm) maximum.

c. Wall Thickness for Endwall Models [to be determined].

4. 	Mounting/Installation: [Construction Adhesive], [Double-Sided Tape], [Screws and countersunk holes]

Optional:  Construction adhesive, Off-white, water-based, 10oz tube [CG-KIT-ADH-PL]

5. 	Edges are eased/beveled to hug corners.

- Style CGSH STAINLESS STEEL HEAVY DUTY CORNERGUARD

1.	Model No.: CGSH corner guard exposed surfaces shall be free of imperfections.

2.	Material: Type 304 stainless steel with #4 finish, 16 gauge.

3.	Dimensions:

a. Angle [90 degrees], [135 degrees], Endwall], [other]

a. Wing width [1-1/2 inches (38 mm)], [2 inches (50.8 mm)], [2-1/2 inches (63.5 mm)], [3 inches (76.2 mm)], [3-1/2 inches (88.9 mm)], [other]

b. Height: [4 feet (1219 mm)], [other] up to 9 feet (2740 mm) maximum.

c. Wall Thickness for Endwall Models [to be determined].

4. 	Mounting/Installation: [Construction Adhesive], [Double-Sided Tape], [Screws and countersunk holes]

Optional:  Construction adhesive, Off-white, water-based, 10oz tube [CG-KIT-ADH-PL]

5. 	Edges are eased/beveled to hug corners.

PART 3 - EXECUTION

3.01	EXAMINATION

A.	Examine substrates for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

1.	Notify the Contractor in writing of conditions detrimental to proper and timely completion of the installation.

2.	Proceed with installation only after unsatisfactory conditions have been corrected.

3.02	INSTALLATION

A.	General:  Comply with manufacturer's written instructions for installing corner guards to wall securely.

B.	Install corner guards accurately in location, alignment, and elevation.

3.03	CLEANING

Use a soft cloth and a good metal cleaner. We recommend Mr. Metal by Northern Labs. Do not use an abrasive cleaner or steel wool.

END OF SECTION