This guide specification was prepared utilizing 3-part format recommended by the Construction Specifications Institute (CSI), and generally incorporates recommendations from their SectionFormat™/Page Format™, and MasterFormat™, latest Editions, insofar as practicable. Carefully review and edit the text to meet the Project requirements and coordinate this Section with the remainder of the Specifications and the Drawings. Where bracketed text is indicated, e.g. [text], make appropriate selection and delete the remainder of text within additional brackets, highlighting, and bold face type, if any. Consult the manufacturer for assistance in editing this guide specification for specific Project applications where necessary. This Specification was current at the time of publication but is subject to change. Please confirm the accuracy of these specifications with the manufacturer prior to use.

SECTION 08 31 16

ACCESS PANELS AND FRAMES

PART 1 - GENERAL

1.01	SUMMARY

A.	Section Includes:

SPECIFIER: Revise subparagraphs below to suit Project requirements.

1.	Fire-Resistive Access panels and frames for walls and ceilings.

2.	Accessories.

B.	Related Requirements:

1.	Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section

SPECIFIER: Revise section numbers and titles in subparagraphs below per CSI MasterFormat™ and Project requirements.

SPECIFIER: Delete Section 01 21 00 - Allowances if locations are clearly shown on the Drawings and sizes are either indicated alongside the locations or specified herein.

2.	Section 01 21 00 - Allowances:  For cash or quantity allowances covering access panels and frames.

3.	Section 01 33 00 - Submittal Procedures:  For administrative and procedural requirements for processing of submittals during the construction phase.

4.	Section 01 77 00 - Closeout Procedures:  For administrative and procedural requirements for completion of the Work.

SPECIFIER: Retain subparagraphs below to suit Project requirements; revise section numbers and titles per project requirements.

5.	Section 08 31 13 - Access Doors and Frames:

6.	Section 09 24 00 - Portland Cement Plastering:  For plaster infill.

7.	Section 09 25 23 - Lime Based Plastering:  For plaster infill.

8.	Section 09 26 00 - Veneer Plastering:  For plaster infill.

9.	Section 09 29 00 - Gypsum Board:  For gypsum wallboard infill.

10.	Section 09 51 00 - Acoustical Ceilings:  For acoustical panel/tile infill.

11.	Section 09 91 00 - Painting:  For field painting of access panels and frames.

12.	Section 11 19 00 - Detention Equipment:  For access panels and frames for detention facilities.

SPECIFIER: Retain this Article if provision of access panels and frames are covered under an allowance.

1.02	ALLOWANCES

SPECIFIER: Retain products and work included in this Section that are covered by cash or quantity allowance.  Do not include amounts.  Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the allowance.

A.	Access panels and frames are part of an access panel and frame allowance.

1.03	REFERENCES

SPECIFIER:  Use care when indicating the edition date of the referenced standards; these standards are subject to regular review, and updated accordingly; review list carefully for inclusion.

A.	Reference Standards:

1.	ASTM International (ASTM):

a.	ASTM A36/A36M-12, Standard Specification for Carbon Structural Steel

b.	ASTM A153/A153M-09, Standard Specification for Zinc Coating (Hot-Dip) on Iron and Steel Hardware

c.	ASTM A653/A653M-11, Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy Coated (Galvannealed) by the Hot-Dip Process

d.	ASTM A666-10, Standard Specification for Annealed or Cold-Worked Austenitic Stainless Steel Sheet, Strip, Plate, and Flat Bar]

e.	ASTM A879/A879M-12, Standard Specification for Steel Sheet, Zinc Coated by the Electrolytic Process for Applications Requiring Designation of the Coating Mass on Each Surface

f.	ASTM A1008/A1008M-12a, Standard Specification for Steel, Sheet, Cold-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, Solution Hardened, and Bake Hardenable

SPECIFIER:  Indicate date of current Edition in affect in the Project location if known, to avoid problems with interpretation.

4.	International Code Council (ICC):

a.	International Building Code (IBC) - [current] Edition.

5.	Intertek Testing Services/Warnock-Hersey International (ITS/WHI)

SPECIFIER:  Select applicable NFPA Standard in subparagraph below; delete subparagraph if no fire-rated access panels are specified.

6.	National Fire Protection Association (NFPA):

a.	NFPA 80-2013, Standard for Fire Doors and Other Opening Protectives

b.	NFPA 252-2012, Standard Methods of Fire Tests of Door Assemblies

c.	NFPA 288-2012, Standard Methods of Fire Tests of Horizontal Fire Door  Assemblies Installed in Horizontal Fire Resistance-Rated Assemblies.

SPECIFIER:  Delete subparagraph below if no fire-rated access panels are specified.

7.	Underwriters Laboratories, Inc.  (UL)

a.	UL 10B-2008, Fire Tests of Door Assemblies

b.	Classified Building Materials Index

CANADIAN TESTING STANDARDS UL

CAN/CSA-1440-00/1440.1-00 (R2005) – User Selection Guide to CSA Standard A440-00, Windows.

CAN/ULC S101-04 – Standard Methods of Fire Endurance Tests of Building Construction and Materials.

1.04	ACTION SUBMITTALS

SPECIFIER:  Revise Section number in the paragraph below to match that used in the Project Manual.

A.	Submit in accordance with Section [01 33 00] [other]:

1.	Product Data:

a.	Materials description for access panels and frames including details showing mounting type, relationships to surrounding construction, panel and frame type, materials, and construction, and locking features.

b.	Installation instructions for each product specified.

2.	Shop Drawings:

a.	Include details of each frame type, elevation of panel, anchorage and accessory items.

b.	Schedule showing each type of access panel and frame, locations, sizes, latching or locking provisions, and other data pertinent to installation

c.	Indicate installation procedures and accessories required for a complete installation.

SPECIFIER:  Delete subparagraph below for standard steel panels; retain for stainless steel or other optional  upscale finishes.

3.	Samples:  For each panel face material, at least 3 by 5 inches (75 by 125 mm) in size, in specified finish.

1.05	QUALITY ASSURANCE

A.	Comply with standards referenced in Article 1.03  REFERENCES.

B.	Provide access panels and frames produced by a single manufacturer.

SPECIFIER:  Retain paragraph below for fire-rated access panels and frames, if any.

C.	Wherever a fire-resistance classification is indicated, provide access assembly with panel, frame, hinge, and latch from manufacturer listed in the Intertek Listed Products Directory for rating shown..

1.	Provide Intertek Warnock Hersey Label on each fire-rated access panel.

1.06	DELIVERY, STORAGE, AND HANDLING

A.	Deliver, store, and handle access panels and frames using means and methods that will prevent damage, deterioration, or loss.

1.	Deliver units in manufacturer's original packaging, properly labeled for identification.

PART 2 - PRODUCTS

2.01	ACCESS PANELS AND FRAMES MANUFACTURERS

A.	Acceptable Manufacturers:

J. L. Industries - a Division of Activar Construction Products Group, Inc.

9702 Newton Av S

Bloomington, MN  55431

(800) 554-6077

www.activarcpg.com

sales@activarcpg.com

B.	Substitutions:  Manufacturers seeking approval of their products are required to comply with the Owner's Instructions to Bidders, generally contained in the Project Manual.

2.02	PERFORMANCE REQUIREMENTS

SPECIFIER:  Retain paragraph below for fire-rated access panels and frames, if any.

A.	Fire-Rated Access Panels and Frames:  Units complying with NFPA 80 that are identical to access panel and frame assemblies tested for fire-test-response characteristics according to the following test method, and that are listed and labeled by UL or another testing and inspecting agency acceptable to authorities having jurisdiction:

SPECIFIER:  Retain one or both subparagraphs below if retaining paragraph A above.  Verify test method recognized by authorities having jurisdiction and availability of complying products with manufacturers.

1.	NFPA 252 or UL 10B for fire-rated access panel assemblies installed vertically.

2.	NFPA 288 for fire-rated access panel assemblies installed horizontally.

SPECIFIER:  The Model number for these access panels is established by specifying the Series identifier, “C” for ceiling, or “W” for wall, followed by optional finish, the width and height, latch/lock type, and other optional features; e.g. Model No. FD2C – 1212LT, meaning an FD, 2 hour for ceiling series fire-rated flush access panel, 12 inches (305 mm) wide by 12 inches (305 mm) high, with an optional “locking T” pull handle.  Model No. FD2WSS-4860APH would be FD, 2 hour for wall series, stainless steel finish, 48 inches (1219 mm) wide by 60” (1524 mm) high, with the standard latch “A” and optional continuous piano hinge.

2.03	FD2 and FD3 SERIES - INTERIOR FIRE-RATED ACCESS PANELS AND FRAMES WITH STANDARD TRIM

- Style FD2:  Two Hour Fire-rated and insulated flush access panel

1.	Model No. FD2 - [insert remainder of model number from manufacturer's chart]

2. 	Frame: Frame Depth; “D”= 2-1/4” (57mm).

a.  [Sizes up to and including 30” x 48”(762 mm x 1219mm);16 gauge 1.61mm)]

b.  [Sizes greater than 30” x 48” (726 mm x 1219 mm) ; 14 gauge (1.99mm)].

3. 	Trim Width;

a. [Sizes up to and including 36” x 36”: Trim is 7/8” (22mm) wide]

b. [Sizes greater than 36” x 36”: Trim is 1-1/4” (32mm) wide].

4. 	Door Panel: 18 gauge (1.31mm) with 22 gauge (0.85mm) liner, continuous hinge and 1-1/2” (38 					mm) high temperature insulation.  Automatic panel closer will close and latch the door from an 					open position of approximately 90° as required by code.

5.	Finish: Galvanized CRS with off-white prime powder coat finish.

SPECIFIER:  Add "A" designation following the Model number in subparagraph 1 above for the standard latch type indicated below or; refer to subparagraph 8 below for optional lock features.

6.	Standard Latch:  Flush mount 1/4” allen key. All fire-rated locks are self-latching and include an interior latch. The interior latch release is standard on doors including and exceeding 16”x 16”(406 mm x 406 mm)in size.

7.	General Use:  [Walls] [Ceilings].

8.	Fire-Rating:

SPECIFIER:  Select applicable subparagraph below for wall or ceiling location.

a.	Walls:  FD2W 2-hour Warnock Hersey listing; minimum size 36-1/4”(920.75 mm) in either dimension, maximum size 48” by 60” (1219.20 mm by 1524 mm).

b.	Ceilings:  FD2C -2 hour Warnock Hersey listing; minimum size 8” x 6” (203 mm x 152 mm), maximum size 36” by 36” (914.4 mm by 914.4 mm).

9.	Style FD2 Options:

SPECIFIER:  Select specific lock and latching options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- Lock Feature:

- [Raised Knurled Knob “KK”]

- [Central 2 Point Operator “2P”]

- [Mortise Cylinder Prep Only ”MP”] (for 1-1/8” (28.58 mm)cylinder)

- [Mortise Cylinder Installed ”MPI”] (for 1-1/8” (28.58 mm) cylinder)

- [Interior Latch] (optional for models under 16” x 16” (406.4 mm x 406.4 mm) size)

SPECIFIER:  Select specific options listed below at additional cost.; indicate selection as a part of the Model number in subparagraph 1 above.

- FD2CSS and FD2WSS Stainless Steel Frame and Panel:  Factory-applied ground and polished #4 directional satin finish on exterior surface.

- Continuous piano hinge (under 48” (1219.2 mm) only).

- Masonry straps – welded to frame.

B.	Style FD3:  Three Hour Fire-rated and insulated flush access panel

1.	Model No. FD3 - [insert remainder of model number from manufacturer's chart]

2. 	Frame: Frame Depth; “D”= 3-1/4” (83mm).

a.  [Sizes up to and including 30” x 48”(762 mm x 1219mm);16 gauge 1.61mm)]

b.  [Sizes greater than 30” x 48” (726 mm x 1219 mm) ; 14 gauge (1.99mm)].

3. 	Trim Width;

a. [Sizes up to and including 36” x 36”: Trim is 7/8” (22mm) wide]

b. [Sizes greater than 36” x 36”: Trim is 1-1/4” (32mm) wide].

4. 	Door Panel: 18 gauge (1.31mm) with 22 gauge (0.85mm) liner, continuous hinge and 2-1/2” (63 					mm) of high temperature insulation.  Automatic panel closer will close and latch the door from an 					open position of approximately 90° as required by code.

5.	Finish: Galvanized CRS with off-white prime powder coat finish.

SPECIFIER:  Add "A" designation following the Model number in subparagraph 1 above for the standard latch type indicated below or; refer to subparagraph 8 below for optional lock features.

6.	Standard Latch:  Flush mount 1/4” allen key. All fire-rated locks are self-latching and include an interior latch. The interior latch release is standard on doors including and exceeding 16”x 16” (406 mm x 406 mm)in size.

7.	General Use:  [Walls] [Ceilings].

8.	Fire-Rating:

SPECIFIER:  Select applicable subparagraph below for wall or ceiling location.

a.	Walls:  FD3W 3-hour Warnock Hersey listing; minimum size 12” x 12” (304.8 x 304.8 mm), maximum size 48” by 60” (1219.20 mm by 1524 mm).

b.	Ceilings:  FD3C -3 hour Warnock Hersey listing; minimum size 12” x 12” (304.8 x 304.8 mm), maximum size 36” by 36” (914.4 mm by 914.4 mm).

9.	Style FD3 Options:

SPECIFIER:  Select specific lock and latching options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- Lock Feature:

- [Raised Knurled Knob “KK”]

- [Central 2 Point Operator “2P”]

- [Mortise Cylinder Prep Only ”MP”] (for 1-1/8” (28.58 mm)cylinder)

- [Mortise Cylinder Installed ”MPI”] (for 1-1/8” (28.58 mm) cylinder

- [Interior Latch] (optional for models under 16” x 16” (406.4 x 406.4 mm) size)

SPECIFIER:  Select specific options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- FD3CSS and FD3WSS Stainless Steel Frame and Panel:  Factory-applied ground and polished #4 directional satin finish on exterior surface.

- Continuous piano hinge (under 48” (1219 mm) only).

- Masonry straps – welded to frame.

2.04	FD2CI and FD2WI SERIES - INTERIOR FIRE-RATED ACCESS PANELS AND FRAMES – INWARD SWINGING DOOR

- Style FD2:  Two Hour Fire-rated and insulated flush access panel with inward swinging door

1.	Model No. FD2 - [insert remainder of model number from manufacturer's chart]

2. 	Frame: Frame Depth; “D”= 3-1/2” (89mm).

a.  [Sizes up to and including 30” x 48”(762 mm x 1219mm);16 gauge 1.61mm)]

b.  [Sizes greater than 30” x 48” (726 mm x 1219 mm) ; 14 gauge (1.99mm)].

3. 	Trim Width; 7/8” (22mm) wide.

- 4. 	Door Panel: 18 gauge (1.31mm) with 22 gauge (0.85mm) liner, inside pull handle, continuous 90º 				piano hinge and 1-1/2” (38 mm) high temperature insulation.  Wall models feature a concealed 					door closer, and ceiling models are self-closing.

5.	Finish: Galvanized CRS with off-white prime powder coat finish.

SPECIFIER:  Add "A" designation following the Model number in subparagraph 1 above for the standard latch type indicated below or; refer to subparagraph 8 below for optional lock features.

6.	Standard Latch:  Flush mount 1/4” allen key. All fire-rated locks are self-latching and include an interior latch. Wall models feature an interior latch release, allowing the doors to be opened from the inside on all models, and ceiling models include interior latch on sizes including and exceeding 16”x 16”(406 mm x 406 mm)in size.

7.	General Use:  [Walls] [Ceilings].

8.	Fire-Rating:

SPECIFIER:  Select applicable subparagraph below for wall or ceiling location.

a.	Walls:  FD2WI 2-hour Warnock Hersey listing; maximum size 48” by 60” (1219.20 mm by 1524 mm).

b.	Ceilings:  FD2CI -2-4 hour Warnock Hersey listing; maximum size 36” by 36” (914.4 mm by 914.4 mm).

9.	Style FD2CI and FD2WI Options:

SPECIFIER:  Select specific lock and latching options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- Lock Feature:

- [Central 2 Point Operator “2P”]

- [Mortise Cylinder Prep Only ”MP”] (for 1-1/8” (28.58 mm)cylinder)

- [Mortise Cylinder Installed ”MPI”] (for 1-1/8” (28.58 mm) cylinder

- [Interior Latch] (optional for  models under 16” x 16” (406.4 x 406.4 mm) size)

- [Knurled Knob “KK”]

SPECIFIER:  Select specific options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- FD2CISS and FD2WISS Stainless Steel Frame and Panel:  Factory-applied ground and polished #4 directional satin finish on exterior surface.

2.05	FD2D and FD3D SERIES - INTERIOR FIRE-RATED ACCESS PANELS AND FRAMES WITH DOUBLE LEAF DOORS

- Style FD2D:  Two Hour Fire-rated and insulated flush access panel with double leaf doors

1.	Model No. FD2D - [insert remainder of model number from manufacturer's chart]

2. 	Frame: Frame Depth; “D”= 2-1/4” (57mm). 14 gauge (1.99mm) steel with removable center mullion.

3. 	Trim Width; 1-1/4” (32mm).

4. 	Door Panel: 18 gauge (1.31mm) with 22 gauge (0.85mm) liner, continuous hinge and 1-1/2” (38 					mm) high temperature insulation.  Automatic panel closer will close and latch the door from an 					open position of approximately 90° as required by code.

5.	Finish: Galvanized CRS with off-white prime powder coat finish.

SPECIFIER:  Add "A" designation following the Model number in subparagraph 1 above for the standard latch type indicated below or; refer to subparagraph 9 below for optional lock features.

6.	Standard Latch:  Flush mount 1/4” allen key slam latch. All fire-rated locks are self-latching and include an interior latch. The interior latch release is standard, allowing the doors to be opened from the inside.

7.	General Use:  [Walls] [Ceilings].

8.	Fire-Rating:

SPECIFIER:  Select applicable subparagraph below for wall or ceiling location.

a.	Walls:  FD2DW 2-hour Warnock Hersey listing; maximum size by width 60” x 48” (1524 mm x 1219.20 mm), maximum size by height 48” x 60” (1219 mm x 1524 mm)

b.	Ceilings:  FD2DC -2 hour Warnock Hersey listing; maximum 60” x 48” x” (1524 mm x 1219.20 mm)

9.        Style FD2D Options:

SPECIFIER:  Select specific lock and latching options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- Lock Feature:

- [Raised Knurled Knob “KK”]

- [Central 2 Point Operator “2P”]

- [Mortise Cylinder Prep Only ”MP”] (for 1-1/8” (28.58 mm)cylinder)

- [Mortise Cylinder Installed ”MPI”] (for 1-1/8” (28.58 mm) cylinder

SPECIFIER:  Select specific options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- FD2DCSS and FD2DWSS Stainless Steel Frame and Panel:  Factory-applied ground and polished #4 directional satin finish on exterior surface.

- Continuous piano hinge “PH” (under 48” (1219 mm) only).

- Masonry straps – welded to frame.

- Style FD3D:  Three Hour Fire-rated and insulated flush access panel with double leaf doors

1.	Model No. FD3D - [insert remainder of model number from manufacturer's chart]

2. 	Frame: Frame Depth; “D”= 3-1/4” (82.55mm). 14 gauge (1.99mm) steel with removable center 					mullion.

3. 	Trim Width; 1-1/4” (32mm).

4. 	Door Panel: 18 gauge (1.31mm) with 22 gauge (0.85mm) liner, continuous hinge and 1-1/2” (38 					mm) high temperature insulation.  Automatic panel closer will close and latch the door from an 					open position of approximately 90° as required by code.

5.	Finish: Galvanized CRS with off-white prime powder coat finish.

SPECIFIER:  Add "A" designation following the Model number in subparagraph 1 above for the standard latch type indicated below or; refer to subparagraph 9 below for optional lock features.

6.	Standard Latch:  Flush mount 1/4” allen key slam latch. All fire-rated locks are self-latching and include an interior latch. The interior latch release is standard, allowing the doors to be opened from the inside.

7.	General Use:  [Walls] [Ceilings].

8.	Fire-Rating:

SPECIFIER:  Select applicable subparagraph below for wall or ceiling location.

a.	Walls:  FD3DW 3-hour Warnock Hersey listing; maximum size by width 60” x 48”  (1524 mm x 1219.20 mm), maximum size by height 48” x 60” (1219 mm x 1524 mm)

b.	Ceilings:  FD3DC -3 hour Warnock Hersey listing; maximum 60” x 48” x” (1524 mm x 1219.20 mm)

9.	Style FD3D Options:

SPECIFIER:  Select specific lock and latching options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- Lock Feature:

- [Central 2 Point Operator “2P”]

- [Mortise Cylinder Prep Only ”MP”] (for 1-1/8” (28.58 mm)cylinder)

- [Mortise Cylinder Installed ”MPI”] (for 1-1/8” (28.58 mm) cylinder )

- [Raised Knurled Knob “KK”]

SPECIFIER:  Select specific options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- FD3DCSS and FD3DWSS Stainless Steel Frame and Panel:  Factory-applied ground and polished #4 directional satin finish on exterior surface.

- Continuous piano hinge (under 48” (1219 mm) only).

- Masonry straps – welded to frame.

2.06	FD2CTW and FD2CTC- SERIES - INTERIOR FIRE-RATED ACCESS PANELS AND FRAMES – RECESSED DOOR & RECESSED FLANGE

A.	Style FD2CTW and FD2CTC:  Two Hour Fire-rated and insulated flush access panel with recessed door with recessed door and recessed wallboard flange.

1.	Model No. FD2CT - [insert remainder of model number from manufacturer's chart]

2. 	Frame: Frame Depth; “D”= 2-7/8” (73mm). 16 gauge (1.61mm) steel construction.

3. 	Mounting Flange; 1-1/4” (32 mm), recessed 5/8” from face of frame to allow for wallboard 					installation on top.

4. 	Door Panel: 18 gauge (1.31mm) with 22 gauge (0.85mm) liner. Face of door is recessed 5/8” to 					accept tile or wallboard insert.  1-1/2” (38 mm) high temperature insulation.

5.	Finish: Galvanized CRS with off-white prime powder coat finish.

SPECIFIER:  Add "A" designation following the Model number in subparagraph 1 above for the standard latch type indicated below or; refer to subparagraph 8 below for optional lock features.

6.	Standard Latch “A”:  Flush mount 1/4” allen key. All fire-rated locks are self-latching and include an interior latch. Wall models feature an interior latch release, allowing the doors to be opened from the inside on all models, and ceiling models include interior latch on sizes including and exceeding 16”x 16”(406 mm x 406 mm)in size.

7.	General Use:  [Walls] [Ceilings].

8.	Fire-Rating:

SPECIFIER:  Select applicable subparagraph below for wall or ceiling location.

a.	Walls:  FD2CTW 2-hour Warnock Hersey listing; maximum size 24” x 36” (610 mm x 914 mm)

b.	Ceilings:  FD2CTC -2 hour Warnock Hersey listing; maximum size 24” x 36” (610 mm x 914 mm)

9.	Style FD2CTC and FD2CTW Options:

SPECIFIER:  Select specific lock and latching options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- Lock Feature:  [Interior Latch] (optional for models under 16” x 16” (406.4 x 406.4 mm) size)

SPECIFIER:  Select specific options listed below at additional cost; indicate selection as a part of the Model number in subparagraph 1 above.

- FD2CTWSS and FD2CTCSS Stainless Steel Frame and Panel:  2b mill finish stainless steel.

2.07	FD2RFW and FD2RFC- SERIES - INTERIOR FIRE-RATED ACCESS PANELS AND FRAMES – RECESSED FLANGE

A.	Style FD2RF:  Two Hour Fire-rated and insulated flush access panel with recessed wallboard flange.

1.	Model No. FD2RF - [insert remainder of model number from manufacturer's chart]

2. 	Frame: Frame Depth; “D”= 3-1/4” (83mm).

a.  [Sizes up to and including 30” x 48”(762 mm x 1219mm);16 gauge 1.61mm)]

b.  [Sizes greater than 30” x 48” (726 mm x 1219 mm) ; 14 gauge (1.99mm)].

3. 	Mounting Flange; 1-1/4” (32 mm), recessed 5/8” from face of frame to allow for wallboard 						installation on top.

4. 	Door Panel: 18 gauge (1.31mm) with 22 gauge (0.85mm) liner.  2-1/2” (63 mm) high temperature 					insulation.  The concealed continuous rod hinge is designed to secure the door panel.  Sizes 					greater than 48” (1219 mm) are provided with a continuous piano hinge.

5.	Finish: Galvanized CRS with off-white prime powder coat finish.

SPECIFIER:  Add "A" designation following the Model number in subparagraph 1 above for the standard latch type indicated below or; refer to subparagraph 9 below for optional lock features.

6.	Standard Latch “A”:  Flush mount 1/4” allen key. All fire-rated locks are self-latching from an open position of approximately 90º,  and feature an interior latch release, allowing the doors to be opened from the inside on all models including and exceeding 16”x 16”(406 mm x 406 mm)in size.

7.	General Use:  [Walls] [Ceilings].

8.	Fire-Rating:

SPECIFIER:  Select applicable subparagraph below for wall or ceiling location.

a.	Walls:  FD2RFW 2-hour Warnock Hersey listing; maximum size 48” x 60”(1219 mm x 1524 mm)

b.	Ceilings:  FD2RFC -2 hour Warnock Hersey listing; maximum size 36” x 36” (914 mm x 914 mm)

9.	Style FD3RFC and FD3RFW Options:

SPECIFIER:  Select specific lock and latching options listed below at additional cost.; indicate selection as a part of the Model number in subparagraph 1 above.

- Lock Feature:

- [Non-locking Handle “T”]

- [Raised Knurled Knob “KK”]

- Lock Feature:  [Interior Latch] (optional for models under 16” x 16” (406.4 x 406.4 mm) size)

SPECIFIER:  Select specific options listed below at additional cost.; indicate selection as a part of the Model number in subparagraph 1 above.

- FD2RFWSS and FD2RFCSS Stainless Steel Frame and Panel:  2b mill finish stainless steel.

- Hinge: Continuous piano hinge for under 48” (1219 mm) panel

2.08	FD3RFW and FD3RFC- SERIES - INTERIOR FIRE-RATED ACCESS PANELS AND FRAMES – RECESSED FLANGE

A.	Style FD3RF:  Three Hour Fire-rated and insulated flush access panel with recessed wallboard flange.

1.	Model No. FD3RF - [insert remainder of model number from manufacturer's chart]

2. 	Frame: Frame Depth; “D”= 2-1/4” (57mm).

a.  [Sizes up to and including 30” x 48”(762 mm x 1219mm);16 gauge 1.61mm)]

b.  [Sizes greater than 30” x 48” (726 mm x 1219 mm) ; 14 gauge (1.99mm)].

3. 	Mounting Flange; 1-1/4” (32 mm), recessed 5/8” from face of frame to allow for wallboard 						installation on top.

4. 	Door Panel: 18 gauge (1.31mm) with 22 gauge (0.85mm) liner.  1-1/2” (38 mm) high temperature 					insulation.  The concealed continuous rod hinge is designed to secure the door panel.  Sizes 					greater than 48” (1219 mm) are provided with a continuous piano hinge.

5.	Finish: Galvanized CRS with off-white prime powder coat finish.

SPECIFIER:  Add "A" designation following the Model number in subparagraph 1 above for the standard latch type indicated below or; refer to subparagraph 9 below for optional lock features.

6.	Standard Latch “A”:  Flush mount 1/4” allen key. All fire-rated locks are self-latching from an open position of approximately 90º,  and feature an interior latch release, allowing the doors to be opened from the inside on all models including and exceeding 16”x 16”(406 mm x 406 mm)in size.

7.	General Use:  [Walls] [Ceilings].

8.	Fire-Rating:

SPECIFIER:  Select applicable subparagraph below for wall or ceiling location.

a.	Walls:  FD3RFW 2-hour Warnock Hersey listing; maximum size 48” x 60”(1219 mm x 1524 mm)

b.	Ceilings:  FD3RFC -2 hour Warnock Hersey listing; maximum size 36” x 36” (914 mm x 914 mm)

9.	Style FD2RFC and FD2RFW Options:

SPECIFIER:  Select specific lock and latching options listed below at additional cost.; indicate selection as a part of the Model number in subparagraph 1 above.

- Lock Feature:

- [Non-locking Handle “T”]

- [Raised Knurled Knob “KK”]

- Lock Feature:  [Interior Latch] (optional for  models under 16” x 16” (406.4 x 406.4 mm) size)

SPECIFIER:  Select specific options listed below at additional cost.; indicate selection as a part of the Model number in subparagraph 1 above.

- FD3RFWSS and FD3RFCSS Stainless Steel Frame and Panel:  2b mill finish stainless steel.

- Hinge: Continuous piano hinge for under 48” (1219 mm) panel; “PH”.

2.09	FD2L SERIES - INTERIOR FIRE-RATED ACCESS PANELS AND FRAMES

A.	Style FD2L:  Two Hour Fire-rated and insulated flush access panel for Laundry and Garbage Chutes

1.	Model No. FD2L - [insert remainder of model number from manufacturer's chart]

2. 	Frame: Frame Depth; “D”= 2-3/4” (70mm).  16 gauge (1.61mm) construction.

3. 	Trim Width; 1-3/4” (45mm) wide with a 8/8” (9.52mm) return.

4. 	Door Panel: 18 gauge (1.31mm) with 22 gauge (0.85mm) liner, continuous steel piano hinge and 1-				1/2” (38	mm) high temperature insulation.

5.	Finish: Galvanized CRS with off-white prime powder coat finish.

SPECIFIER:  Add "T" designation following the Model number in subparagraph 1 above for the standard latch type indicated below or; refer to subparagraph 9 below for optional lock features.

6.	Standard Latch:  Non-locking “T” handle. Interior latch release not standard.

7.	General Use: Walls.

8.	Fire-Rating:  FD2L 2-hour Warnock Hersey listing.

9.	Style FD2L Options:

SPECIFIER:  Select specific lock and latching options listed below at additional cost.; indicate selection as a part of the Model number in subparagraph 1 above.

- Lock Feature:

1. [Mortise Cylinder Installed ”MPI”] (for 1-1/8” (28.58 mm)cylinder)

2. [Listed smoke gasket]

SPECIFIER:  Select specific options listed below at additional cost.; indicate selection as a part of the Model number in subparagraph 1 above.

- FD2L Stainless Steel Frame and Panel “SS”:  Factory-applied ground and polished #4 directional satin finish on exterior surface.

2.10	MATERIALS

A.	Steel Plates, Shapes, and Bars:  ASTM A36/A36M.

B.	Steel Sheet:  Uncoated or electrolytic zinc coated, ASTM A879/A879M, with cold-rolled steel sheet substrate complying with ASTM A1008/A1008M, Commercial Steel (CS), exposed.

C.	Metallic-Coated Steel Sheet:  ASTM A653/A653M, Commercial Steel (CS), Type B; with minimum G60 (Z180) or A60 (ZF180) metallic coating.

SPECIFIER:  Select Type 304 stainless steel for normal environments or select Type 316 for better corrosion resistance in coastal environments; revise to Type 304L or 316L if welding is involved.

D.	Stainless-Steel Sheet, Strip, Plate, and Flat Bars:  ASTM A 666, [Type 304] [Type 316].  Remove tool and die marks and stretch lines or blend into finish.

E.	Aluminum Extrusions:  ASTM B221, Alloy 6063-T6.

F.	Aluminum Sheet:  ASTM B209, alloy and temper recommended by aluminum producer and finisher for type of use and finish indicated, and with not less than strength and durability properties of Alloy 5005-H15; with minimum sheet thickness according to ANSI H35.2.

G.	Frame Anchors:  Same type as panel face.

H.	Inserts, Bolts, and Anchor Fasteners:  Hot-dip galvanized steel according to ASTM A153/A153M or ASTM F2329.

I.	Furnish inserts and anchoring devices which must be built into other work for installation of access panels.

2.11	FABRICATION

A.	General:  Furnish each access panel assembly manufactured as an integral unit, complete and ready for installation.

B.	Metal Surfaces:  For metal surfaces exposed to view in the completed Work, provide materials with smooth, flat surfaces without blemishes.  Do not use materials with exposed pitting, seam marks, roller marks, rolled trade names, or roughness.

C.	Panels and Frames:  Grind exposed welds smooth and flush with adjacent surfaces.  Furnish attachment devices and fasteners of type required to secure access panel frames to types of supports indicated.

SPECIFIER:  Retain applicable subparagraphs below.

1.	For concealed flanges with drywall bead, provide edge trim for gypsum board securely attached to perimeter of frames.

2.	For concealed flanges with plaster bead for full-bed plaster applications, provide zinc-coated expanded metal lath and exposed casing bead welded to perimeter of frames.

3.	Provide mounting holes in frames for attachment of units to metal or wood framing.

4.	Provide mounting holes in frame for attachment of masonry anchors.

D.	Recessed Access Panels:  Form face of panel to provide recess for application of applied finish.  Reinforce panel as required to prevent buckling.

SPECIFIER:  Retain subparagraph below for recessed access panels with plaster infill.

1.	For recessed panels with plaster infill, provide self-furring expanded metal lath attached to panel.

E.	Latching Mechanisms:  Furnish number required to hold panels in flush, smooth plane when closed.

1.	For cylinder locks, furnish 2 keys per lock and key all locks alike.

2.	For recessed panels, provide access sleeves for each locking device.  Furnish plastic grommets and install in holes cut through finish.

PART 3 - EXECUTION

3.01	EXAMINATION

A.	Examine substrates for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

1.	Notify the Contractor in writing of conditions detrimental to proper and timely completion of the installation.

2.	Proceed with installation only after unsatisfactory conditions have been corrected.

3.02	INSTALLATION

A.	General:  Comply with manufacturer's written instructions for installing access panels and frames.

B.	Install panels flush with adjacent finish surfaces or recessed to receive finish material.

3.03	ADJUSTING AND CLEANING

A.	Adjust access panels to operate easily without binding.  Verify that integral locking/latching  devices operate properly.

B.	Remove panels and frames that are warped, bowed, or otherwise damaged, and replace with new components.

C.	On completion of access panel installation, clean interior and exterior surfaces as recommended by manufacturer.

END OF SECTION