SECTION 04 20 00

MASONRY ACCESSORIES – Weep Vents

- GENERAL

  - SECTION INCLUDES

    - Weep Vents.

  - RELATED SECTIONS

    - Section 04 20 00 – Clay Unit Masonry:  Cavity wall construction of brick veneer.

  - REFERENCES

    - ASTM E 84 - Standard Test Method for Surface Burning Characteristics of Building Materials.

  - SUBMITTALS

    - Submit under provisions of Section 01 33 00.

    - Product Data:  Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications:  Minimum 2 years production of similar products.

    - Installer Qualifications:  Minimum 2 years experience as a masonry installer.

  - DELIVERY, STORAGE, AND HANDLING

    - Store products in manufacturer's unopened packaging until ready for installation.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:  Keene Building Products; PO Box 241353, Mayfield Heights, Ohio 44124.  Toll Free Tel: (877) 514-5336, Tel: (440) 605-1020.  Fax: (440) 605-1120.  Email: info@keeneBuilding.com.  Web: http://www.KeeneBuilding.com.

    - Substitutions:  Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 25 00.

  - WEEP VENTS

    - Weep Vents:  Polyester non-woven mesh vent that is placed in the vertical head joints approximately every two feet to create a space for moisture to drain and vent in a brick cavity wall.

    - Product: Driwall™ Weep Vents as manufactured by Keene Building Products.

      - Description:

        - 2.5” x 3.5” x 0.375”

        - Polyester

- EXECUTION

  - INSTALLATION

  - Place weep vents in open head joints of cavity wall located immediately above ledges and flashing, spaced 24 inches apart, unless otherwise shown.

END OF SECTION