Guide Specification for Metal Sales Manufacturing Corporation IC72-Panel metal roof panels.

This guide specification document is provided by Metal Sales Manufacturing Corporation as a technical support tool incident to the sale of its products. Metal Sales Manufacturing Corporation is solely responsible for its content. This document should be reviewed and edited to suit Project requirements by a qualified design professional. Contact manufacturer for more information on this or other products made by Metal Sales Manufacturing Corporation. Telephone: 800.406.7387 Metalsales.us.com

This document contains trademarks registered to their respective owners. Trademarks include MS Colorfast45 owned by Metal Sales Manufacturing Corporation, Kynar 500 owned by Arkema, Inc.

Editor Note: Edit document to suit Project requirements and specifier practice. Specifier notes are shown in blue text like this. Optional text [is shown in bold with brackets like this]. Locations where language for Project-specific requirements is to be inserted are shown like this: <insert language>. Remove specifier notes and unused optional text in final version of the specification document.

Editor Note: The Construction Specifications Institute (CSI) recommends and supports use of its current MasterFormat section title and numbering system, shown below as SECTION 07 41 13 - METAL ROOF PANELS. (If using the earlier MasterFormat 1995 section title and numbering designation, the section may be numbered and titled as SECTION 07610 – SHEET METAL ROOFING).

SECTION 07 41 13 - METAL ROOF PANELS

PART 1 - GENERAL

1.1 SUMMARY

A. Section Includes: Architectural metal roof panels, including trim accessories.

Editor Note: Revise paragraph below to suit Project requirements. Add section numbers and titles according to CSI MasterFormat and specifier practice. This paragraph is intended for use when a reader might reasonably expect to find work requirements in this Section, but those requirements are actually located in another, related section.

B. Related Sections: Section(s) related to this section include:

1. <Insert Work Title>: <Insert Division number> Section <Insert Section title>.

Editor Note: Standards numbers and titles in the article below are provided for specifier information and reference. Retain, edit or delete article to suit specifier practice and Project requirements.

1.2 REFERENCES

A. General: Standards listed by reference form a part of this specification section. Standards listed are identified by issuing authority, abbreviation, designation number, title or other designation. Standards subsequently referenced in this Section are referred to by issuing authority abbreviation and standard designation.

B. ASTM International:

1. ASTM A 792 - Standard Specification for Steel Sheet, 55% Aluminum-Zinc Alloy-Coated by the Hot-Dip Process.

2. ASTM D 2244 - Standard Practice for Calculation of Color Tolerances and Color Differences from Instrumentally Measured Color Coordinates.

3. ASTM D 4214 - Standard Test Methods for Evaluating the Degree of Chalking of Exterior Paint Films.

4. ASTM E 283 - Standard Test Method for Determining Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.

5. ASTM E 331 - Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Uniform Static Air Pressure Difference.

6. ASTM E 1592 - Standard Test Method for Structural Performance of Sheet Metal Roof and Siding Systems by Uniform Static Air Pressure Difference

7. ASTM E 1646 - Standard Test Method for Water Penetration of Exterior Metal Roof Panel Systems by Uniform Static Air Pressure Difference.

8. ASTM E 1680 - Standard Test Method for Rate of Air Leakage through Exterior Metal Roof Panel Systems

C. Underwriters Laboratories (UL):

1. UL 263 - Fire Tests of Building Construction and Materials.

2. UL 580 - Tests For Uplift Resistance of Roof Assemblies.

3. UL 790 - Standard Test Methods for Fire Tests of Roof Coverings.

4. UL 2218 - Impact Resistance of Prepared Roof Covering Materials.

D. Sheet Metal and Air Conditioning Contractors' National Association (SMACNA): "Architectural Sheet Metal Manual."

1.3 ADMINISTRATIVE REQUIREMENTS

A. Preinstallation Meetings: Conduct preinstallation meeting to clarify Project requirements, substrate conditions, manufacturer’s installation instructions and manufacturer’s warranty requirements.

1.4 ACTION SUBMITTALS

A. Product Technical Data: For each type of product required, including manufacturer's preparation recommendations, storage and handling requirements, and recommended installation methods.

Editor Note: Retain paragraph below if compliance with USGBC LEED is required. Edit to suit Project requirements and applicable version of LEED.

B. LEED Submittal Documentation:

1. Product Test Reports for applicable sustainable sites credits: For roof panels, indicating that panels comply with solar reflectance index requirement.

2. Product Data for applicable materials and resources credits: Indicating percentages by weight of post-consumer and pre-consumer recycled content for products having recycled content. Provide a statement indicating cost for each product having recycled content.

C. Shop Drawings: Showing methods of installation, plans, sections, elevations and details of roof and wall panels, specified loads, flashings, roof curbs, vents, sealants, interfaces with all materials not supplied by the metal panel system manufacturer, and identification of proposed component parts and their finishes. Do not proceed with fabrication prior to approval of shop drawings.

D. Samples: Selection and verification samples for finishes, colors and textures. Submit two complete sample sets of each type of panel, trim, clip and fastener required.

E. Certificates: Product certificates signed by manufacturer certifying materials comply with specified performance characteristics, criteria and physical requirements.

F. Test and Evaluation Reports: Showing compliance with specified performance characteristics and physical properties.

G. Qualifications Statements: For manufacturer and installer.

Editor Note: Retain paragraph below when system design will be provided by an independent professional engineer.

H. Design Submittal: Comply with performance requirements and design criteria, including analysis data and calculations signed and sealed by a qualified professional engineer.

1.5 CLOSEOUT SUBMITTALS

A. Operation and Maintenance Data: For installed products including maintenance methods and precautions against cleaning materials and methods detrimental to finishes and performance.

B. Warranty: Warranty documents required in this section.

Editor Note: Coordinate article below with Division 01 Closeout Submittals Section.

1.6 MAINTENANCE MATERIAL

A. Extra Materials: Deliver to Owner extra materials from same production run as products installed. Package products with protective covering and identify with descriptive labels. Comply with Division 01 Closeout Submittals Section.

Editor Note: Revise paragraph below specifying size and percentage as required for project.

1. Quantity: Furnish quantity of <Insert description of units> units equal to <Insert percentage> percent of amount installed. <Insert any additional requirements>.

2. Delivery, Storage and Protection: Comply with Owner’s requirements for delivery, storage and protection of extra materials.

1.7 QUALITY ASSURANCE

A. Manufacturer Qualifications:

1. Provider of advanced installer training.

2. Minimum of ten years experience in manufacturing metal roof systems.

3. Provider of products produced in a permanent factory environment with fixed roll-forming equipment.

B. Installer Qualifications:

1. At least five years experience in the installation of architectural metal roof panels.

2. Experience on at least five projects of similar size, type and complexity as this Project that have been in service for a minimum of two years with satisfactory performance of the roof system.

3. Employer of workers for this Project who are competent in techniques required by manufacturer for installation indicated and who shall be supervised at all times when material is being installed.

Editor Note: Retain paragraph below for assemblies constructed either onsite or offsite required for review of construction, coordination of work of multiple sections, testing, or observation of operation. Mock-ups, when accepted or approved, establish standards by which work will be judged.

C. Mock-Ups: Install at Project site a mock-up using required products and manufacturer's approved installation methods. Obtain Owner and Architect approval of finish, color, texture, pattern, trim, fasteners and quality of installation before proceeding with further work.

1. Size: <Insert mock-up size requirements>.

2. Maintenance: Maintain mock-up during construction for quality comparison. Remove and lawfully dispose of mock-up construction when no longer required.

3. Incorporation: Mock-up may be incorporated into final construction upon Owner approval.

Editor Note: Retain paragraph below when a preinstallation conference is required.

D. Preinstallation Conference: Conduct a preinstallation conference <Insert location and other requirements>.

Editor Note: Retain, edit or delete paragraph below to suit Project requirements.

E. Fire Resistance Ratings: Determined by testing identical products and assemblies according to UL 263 and UL 790 by a testing agency acceptable to authorities having jurisdiction.

1. Flame-Spread Index: 25 (Class A) <Insert index value> or less.

2. Smoke-Developed Index: [450] <Insert index value> or less.

1.8 DELIVERY, STORAGE AND HANDLING

A. General: Comply with manufacturer’s current printed product storage recommendations.

B. Delivery: Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

C. Storage: Store materials above ground, under waterproof covering, protected from exposure to harmful weather conditions and at temperature and humidity conditions recommended by manufacturer. Provide proper ventilation of metal panel system to prevent condensation build-up between each panel and trim or flashing component. Tilt stack to drain in wet conditions. Remove strippable plastic film before storage under high-heat conditions. Store products in manufacturer's unopened packaging until just prior to installation.

D. Handling: Exercise caution in unloading and handling metal panel system to prevent bending, warping, twisting and surface damage.

Editor Note: Coordinate article below with Conditions of the Contract and with Division 01 Closeout Submittals (Warranty) Section.

1.9 WARRANTY

Editor Note: Edit paragraph below to suit Project requirements. Request sample warranty documents from manufacturer for review and editing assistance.

A. Special Exposed Panel Finish Warranty: Manufacturer's standard form PVDF (Fluorocarbon) System Warranty for film integrity, chalk rating and fade rating in which manufacturer agrees to repair or replace panels that show evidence of deterioration within specified warranty period.

1. Deterioration shall include but is not limited to:

a. Color fading of more than 5 Hunter units when tested according to ASTM D 2244.

b. Chalking in excess of a No. 8 rating when tested according to ASTM D 4214.

c. Cracking, checking, peeling or failure of paint to adhere to bare metal.

2. Warranty Period: Film integrity for 45 years and chalk and fade rating for 35 years from date of Substantial Completion.

3.  Manufacturer’s warranty may exclude surface deterioration due to physical damage and exposure to salt air environments.

Editor Note: Retain paragraph below when standard proprietary two coat roll coated system warranty (MS Colorfast45) is required.

B. Special Exposed Panel Finish Warranty: Manufacturer's standard form proprietary two coat roll coated System Warranty for film integrity, chalk rating and fade rating in which manufacturer agrees to repair or replace panels that show evidence of deterioration within specified warranty period.

1. Deterioration shall include but is not limited to:

a. Color fading of more than 5 Hunter units on vertical applications or more than 6 Hunter units on non-vertical applications when tested according to ASTM D 2244.

b. Chalking in excess of a No. 8 rating on vertical applications or a No. 7 rating on non-vertical applications when tested according to ASTM D 4214.

c. Cracking, checking, peeling or failure of paint to adhere to bare metal.

d. Perforation.

2. Warranty Period: Film integrity for 45 years, chalk and fade rating for 30 years, and perforation for 25 years from date of Substantial Completion.

3. Manufacturer’s warranty may exclude surface deterioration due to physical damage and exposure to salt air environments.

Editor Note: Retain paragraph below only if a separate installer warranty is required and edit to suit Project requirements.

C. Special Warranty: Installer's standard form in which installer agrees to repair or replace standing seam panels that fail due to poor workmanship or faulty installation within the specified warranty period.

1. Warranty Period: <Insert number of years> years from date of Substantial Completion.

PART 2 - PRODUCTS

Editor Note: Add product features, performance characteristics, material standards, and descriptions as applicable. Use of such phrases as "or equal" or "approved equal" or similar phrases may cause ambiguity in specifications. Such phrases require verification (procedural, legal and regulatory) and assignment of responsibility for determining "equal" products.

Editor Note: Retain article below when IC72-Panel metal roof panels are required. Edit to suit Project requirements.

2.1 METAL ROOF PANELS

Editor Note: Contact manufacturer for more information.

A. Basis of Design Product: Subject to compliance with requirements provide Metal Sales Manufacturing Corporation; IC72-Panel.

B. Substitution Limitations: [No substitutions] [All other manufacturers: Submit substitution request in accordance with Section 012500 - "Substitution Procedures"] <Insert limitations>.

Editor Note: Edit to suit Project requirements and coordinate with "Performance Criteria" paragraphs below.

C. Product Options:

1. Panel coverage: 36 inches (914.4 mm).

2. Rib Height: 1-1/2 inches (38.1 mm).

Editor Note: Industry designation for material thickness is moving away from "gauge" to decimal thickness in inches. Metal Sales Manufacturing Corporation recommends use of a minimum thickness requirement of 0.0236-inch (0.60-mm) instead of 24 gauge, 0.0296-inch (0.75-mm) instead of 22 gauge, 0.0356 inch (0.904 mm) instead of 20 gauge. For Galvalume, select AZ50 for painted material or AZ55 for unpainted material. For galvanized, select G90.

3. Material:  Aluminum-zinc alloy-coated steel sheet, ASTM A 792, [AZ50] [AZ55] or zinc-coated steel sheet, ASTM A 653 [G90] coating designation, structural quality, Grade 50, [0.0236-inch (0.60-mm)] [0.0296-inch (0.75-mm)] [0.0356 inch (0.904 mm)] minimum thickness.

4. Minimum Roof Slope Capability: 1:12.

5. Attachment: Exposed direct fastened panel.

6. Application: Designed for application over open framing or solid substrate.

7. Rib Configuration: Trapezoidal.

8. Surface Finish: [PVDF (Kynar 500)] [Acrylic Coated Galvalume].

9. Color: [As selected by Architect from manufacturer's standard colors] <Insert color>.

10. Fire Resistance Rating: Comply with UL 263 and UL 790 Class A Fire Resistance Ratings.

11. Impact Resistance: Comply with UL 2218, Class 4.

12. Wind Uplift Resistance: Comply with UL 580, Class 90 Wind Uplift, Construction #137 and #244.

13. Air Leakage: 0.0148 cfm/sq. ft. at 12 psf when tested according to ASTM E 283.

14. Air Leakage: 0.0002 cfm/sq. ft. at 12 psf when tested according to ASTM E 1680.

15. Water Penetration: None at 6.24 psf when tested according to ASTM E 331.

15. Water Penetration: None at 12 psf when tested according to ASTM E 1646.

16. Structural Performance: Tested according to requirements of ASTM E 1592.

D. Performance Criteria:

Editor Note: Edit subparagraph below to suit Project requirements.

1. Wind Uplift Resistance: <Insert requirements>.

Editor Note: Edit subparagraph below to suit Project requirements.

2. Structural Performance: <Insert requirements for wind loads, snow loads, deflection limits>.

Editor Note: Edit subparagraph below if required for applicable LEED credits.

3. Solar Reflectance Index: <Insert requirements>.

Editor Note: Edit subparagraph below if ENERGY STAR compliance is required.

4. ENERGY STAR Performance: <Insert requirements>.

Editor Note: Retain this article if required. Insert requirements for vapor retarders, board insulation, mineral fiber blanket insulation, metal building insulation and other materials.

2.2 FIELD-INSTALLED THERMAL INSULATION

A. General: Refer to and coordinate with requirements in Division 07 - Thermal Insulation.

B. Products: <Insert requirements>.

Editor Note: Retain this article if required. Insert appropriate paragraphs in this article for metal roof panels applied directly over metal deck or solid sheathing. Related products might include self-adhering sheets, felts and slip sheets.

2.3 UNDERLAYMENT MATERIALS

A. General: <Insert requirements>.

B. Products: <Insert requirements>.

Editor Note: Retain this article if required. Insert appropriate paragraphs in this article for metal roof panels applied over substrate boards. Related products might include gypsum board, gypsum sheathing, perlite board and related fasteners.

2.4 SUBSTRATE BOARDS

A. General: <Insert requirements>.

B. Products: <Insert requirements>.

Editor Note: Retain this article if required. Insert appropriate paragraphs in this article for accessory components of metal roof panel assemblies made or approved by manufacturer. Related products might include trims, copings, fascia, corners, closures, clips, flashings, gutters, downspouts, roof curbs, sealants, gaskets, fillers, closure strips, and other items.

2.5 ACCESSORIES

A. General: <Insert requirements>.

B. Products: <Insert requirements>.

1. Basis of Design Product: Subject to compliance with requirements provide Metal Sales Manufacturing Corporation; <Insert manufacturer’s product designations>.

2. Color: [As indicated by manufacturer's product designations] [As selected by Architect from manufacturer's full product range] <Insert color>.

2.6 SOURCE QUALITY CONTROL

A. Source: Obtain architectural metal roof panels, trim and other accessories from a single manufacturer.

B. Quality Control: Obtain architectural metal roof panels, trim and other accessories from a manufacturer capable of providing on-site technical support and installation assistance.

PART 3 - EXECUTION

Editor Note: Retain paragraph below if installation by approved installers is required.

3.1 INSTALLERS

A. <Insert acceptable installers and contact information>.

3.2 PREPARATION

Editor Note: Retain paragraph below if installation of substrate board is required.

A. Substrate Board: Install substrate boards over roof deck and sheathing over entire roof surface using recommended fasteners.

B. Miscellaneous Framing: Install furring, eave angles, subpurlins, and other miscellaneous roof panel support members and anchorage according to metal roof panel manufacturer's recommendations.

3.3 UNDERLAYMENT INSTALLATION

Editor Note: Retain paragraph below if installation of self-adhering insulation sheet is required.

A. Self-Adhering Sheet Underlayment: <Insert requirements>.

Editor Note: Retain paragraph below if installation of felt underlayment is required.

B. Felt Underlayment: <Insert requirements>.

Editor Note: Retain paragraph below if installation of slip sheet is required.

C. Apply slip sheet over underlayment prior to installing metal roof panels.

D. Install flashing in compliance with requirements in Division 07 Section "Sheet Metal Flashing and Trim."

3.4 THERMAL INSULATION INSTALLATION

Editor Note: Retain paragraph below if installation of polyethylene vapor retarder is required.

A. Polyethylene Vapor Retarder: <Insert requirements>.

Editor Note: Retain paragraph below if installation of board insulation is required.

B. Board Insulation: <Insert requirements>. Comply with installation requirements in Division 07 Section "Thermal Insulation."

Editor Note: Retain paragraph below if installation of blanker insulation is required..

C. Blanket Insulation: <Insert requirements>. Comply with installation requirements in Division 07 Section "Thermal Insulation."

3.5 ARCHITECTURAL METAL ROOF PANEL INSTALLATION

A. General: Comply with panel manufacturer’s installation instructions including but not limited to special techniques, interface with other work, and integration of systems.

B. Fasten metal roof panels to supports with concealed clips at each standing-seam joint at location, spacing, and using proper fasteners as recommended by panel manufacturer.

C. Tolerances: <Insert requirements>.

3.6 ACCESSORY INSTALLATION

A. General: Install accessories using techniques recommended by manufacturer and which will assure positive anchorage to building and weather tight mounting. Provide for thermal movement. Coordinate installation with flashings and other components.

B. Flashing and Trim: Comply with performance requirements, manufacturer's written installation instructions, and the SMACNA "Architectural Sheet Metal Manual." Provide concealed fasteners where possible, and install units to true level. Install work with laps, joints, and seams that will be permanently watertight.

Editor Note: Retain article below if manufacturer site visits are required and edit to suit Project requirements. Consult with manufacturer for services required, and establish number and duration of periodic site visits with Owner and manufacturer. Coordinate paragraph below with Division 01 Quality Assurance Section.

3.7 FIELD QUALITY CONTROL

A. Manufacturer’s Field Services: If requested by Owner, provide manufacturer’s field service consisting of product use recommendations and periodic site visits for inspection of product installation in accordance with manufacturer’s instructions.

1. Site Visits: <Insert requirements for site visits>.

3.8 CLEANING

A. Remove temporary coverings and protection of adjacent work areas.

B. Repair or replace any installed products that have been damaged.

C. Clean installed panels in accordance with manufacturer’s instructions prior to Owner’s acceptance.

D. Remove and lawfully dispose of construction debris from Project site.

3.9 PROTECTION

A. Protect installed product and finish surfaces from damage during construction.

END OF SECTION 07 41 13 - METAL ROOF PANELS