This specification data sheet is provided by Metal Sales Manufacturing Corporation as a technical support tool incident to the sale of its IMPower Series™ insulated metal roof panel products. Metal Sales Manufacturing Corporation is solely responsible for its content. Contact manufacturer for more information on these and other products. Telephone: 800.406.7387. www.metalsales.us.com

SECTION 07 41 13 - METAL ROOF PANELS

1. PRODUCT NAME

IMPower Series™ insulated metal roof panels:

SR2 Standing Seam Roof panel

HR3 High Rib Series Roof panel

2. MANUFACTURER

Metal Sales Manufacturing Corporation

545 South 3rd Street

Suite 200

Louisville, KY 40202

Phone: 800.406.7387

Web site: www.metalsales.us.com

3. PRODUCT DESCRIPTION

Basic Use

Metal building components, sales and service. For more than 50 years, Metal Sales Manufacturing Corporation has earned a reputation as the premier provider of innovative metal building components and accessories. This manufacturer maintains the industry’s largest professional sales and service team, supported by 21 branches located throughout the United States, and offers a full line of high quality metal roof and wall panels for agricultural, commercial, architectural, industrial, and residential projects of every shape and size for both new construction and retrofit applications. Metal Sales is dedicated to leading the metal building component industry, by setting new standards for operating efficiency, product design, active service management and lasting value.

Manufacturer Memberships and Affiliations

CRRC - Cool Roof Rating Council

Metal Construction Association

Construction Specifications Institute

ENERGY STAR® Partner

MRA - Metal Roofing Alliance

National Frame Builders Association

National Roofing Contractors Association

Illustrations

EDITOR NOTE: INSERT LINE ART, SCANNED IMAGES, TABLES from mfr literature as appropriate

EDITOR NOTE: Suggested illustrations. Show installation process. Show typical finished installations

4. TECHNICAL DATA

Applicable Standards

ASTM International:

ASTM A 653 – Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process.

ASTM A 792 – Standard Specification for Steel Sheet, 55% Aluminum-Zinc Alloy-Coated by the Hot-Dip Process.

ASTM A 1011 – Standard Specification for Steel, Sheet and Strip, Hot-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, and Ultra-High Strength.

ASTM C 518 – Standard Test Method for Steady-State Thermal Transmission Properties by Means of the Heat Flow Meter Apparatus.

ASTM D 2244 – Standard Practice for Calculation of Color Tolerances and Color Differences from Instrumentally Measured Color Coordinates.

ASTM D 4214 – Standard Test Methods for Evaluating the Degree of Chalking of Exterior Paint Films.

ASTM E 72 – Standard Test Methods of Conducting Strength Tests of Panels for Building Construction.

FM Global (FM):

FM 4471 – Approval Standard for Class 1 Panel Roofs.

FM 4880 – Approval Standard for Class 1 Fire Rating of Insulated Wall or Wall and Roof/Ceiling Panels, Interior Finish Materials or Coatings and Exterior Wall Systems.

Sheet Metal and Air Conditioning Contractors' National Association (SMACNA): "Architectural Sheet Metal Manual."

Approvals

State of Florida Building Code.

Approval 14700.

Physical Properties

Test reports are available to design professionals upon request.

Technical Properties

IMPower Series SR2 Standing Seam Roof Insulated Metal Panel

Panel Thickness: 3.25 inches (82.6 mm) or 4 inches (101.6 mm) or 5 inches (127 mm) or 6 inches (152.4 mm).

Thermal Insulating Value: R26 or R32 or R41 or R49, when tested according to ASTM C 518.

Panel Width: 40 inches (1016 mm).

Panel Length: 8 feet – 0 inches (2438 mm) minimum to 50 feet – 0 inches (15.2 m) maximum.

Insulation Material and Density: Polyisocyanurate 2.1 to 2.5 lb./cu. ft.

Joint Configuration: 180-degree field seamed with concealed fasteners.

Facing Material: Galvanized sheet steel, ASTM A 653, G90 coating designation, or Aluminum-zinc alloy-coated steel sheet, ASTM A792, Class AZ55 coating designation.

Exterior Facing Material Thickness: 26 gauge is standard, or 22 or 24 gauge optional.

Interior Facing Material Thickness: 26 gauge.

Exterior Coating Type: PVDF (polyvinylidene fluoride) or SMP (silicone modified polyester).

Some exterior colors indicated below are designated by manufacturer as “standard stocked.” Some are designated as “standard non-stocked.” Contact manufacturer for more information on color availability and lead times.

Exterior Color: Regal White, Surrey Beige, Pearl Gray, Royal Blue, Sandstone, Natural Green, Warm White, Slate Gray, Evergreen, Slate Blue, Colonial Red, Weathered Copper.

Information below applies when SMP Coating is required.

Exterior Color: Sandstone, Surrey Beige, Imperial White.

Interior Color, Coating Type: Imperial White, SMP (silicone-modified polyester).

Interior Finish: Light embossed.

Fire Resistance Rating: Class 1 in compliance with FM 4880.

Fire, Wind, Foot Traffic and Hail Rating: Class 1 in compliance with FM 4471.

Structural Performance: Tested to withstand positive and negative wind loads according to requirements of ASTM E72.

Accessories: Provide fasteners, concealed fastening clips, sealants, trims and flashing as recommended by panel manufacturer.

Building Code Compliance: Comply with requirements of State of Florida Approval 14700.

Technical Properties

IMPower Series HR3 High Rib Series Roof Insulated Metal Panel

Panel Thickness: 2.5 inches (63.5 mm) or 4 inches (101.6 mm) or 5 inches (127 mm) or 6 inches (152.4 mm).

Thermal Insulating Value: R20 or R32 or R41 or R49, when tested according to ASTM C 518.

Panel Width: 40 inches (1016 mm).

Panel Length: 8 feet – 0 inches (2438 mm) minimum to 50 feet – 0 inches (15.2 m) maximum.

Insulation Material and Density: Polyisocyanurate 2.1 to 2.5 lb./cu. ft.

Joint Configuration: Overlapping with exposed fasteners and saddle clips.

Facing Material: Galvanized sheet steel, ASTM A 653, G90 coating designation, or Aluminum-zinc alloy-coated steel sheet, ASTM A 792, Class AZ55 coating designation.

Exterior Facing Material Thickness: 26 or 24 or 22 gauge.

Interior Facing Material Thickness: 26 gauge is standard, or 22 or 24 gauge optional.

Exterior Coating Type: PVDF (polyvinylidene fluoride) or SMP (silicone modified polyester).

Some exterior colors indicated below are designated by manufacturer as “standard stocked.” Some are designated as “standard non-stocked.” Contact manufacturer for more information on color availability and lead times.

Exterior Color: Regal White, Surrey Beige, Pearl Gray, Royal Blue, Sandstone, Natural Green, Warm White, Slate Gray, Evergreen, Slate Blue, Colonial Red, Weathered Copper.

Information below applies when SMP Coating is required.

Exterior Color: Sandstone, Surrey Beige, Imperial White.

Interior Color, Coating Type: Imperial White, SMP (silicone-modified polyester).

Interior Finish: Light embossed.

Fire Resistance Rating: Class 1 in compliance with FM 4880.

Fire, Wind, Foot Traffic and Hail Rating: Class 1 in compliance with FM 4471.

Structural Performance: Tested to withstand positive and negative wind loads according to requirements of ASTM E 72.

Accessories: Fasteners, concealed fastening clips, sealants, trims and flashing as recommended by panel manufacturer.

Building Code Compliance: Comply with requirements of State of Florida Approval 14700.

Environmental Considerations

Construction metals generally are readily recyclable at the end of their service life. The raw materials used in manufacture of standing seam panels also come from recycled sources. Post-industrial and post-consumer recycled content varies. Consult with manufacturer for more information.

5. INSTALLATION

Handling and Storage

Handle and store product according to Metal Sales Manufacturing Corporation recommendations. Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact. Store materials above ground, under waterproof covering, protected from exposure to harmful weather conditions and at temperature and humidity conditions recommended by manufacturer. Provide proper ventilation of metal panel system to prevent condensation build-up between each panel and trim or flashing component. Store metal panel products in manufacturer's unopened packaging until ready for installation. Exercise caution in unloading and handling metal panel system to prevent bending, warping, twisting and surface damage.

Preparation

Install substrate boards over roof deck and sheathing over entire roof surface using recommended fasteners. Install furring, eave angles, sub-purlins, and other miscellaneous roof panel support members and anchor according to metal roof panel manufacturer's recommendations.

Underlayment Installation

Install self-adhering sheet underlayment and felt underlayment as required. Apply slip sheet over underlayment prior to installing metal roof panels. Install flashing in compliance with requirements in Division 07 Section "Sheet Metal Flashing and Trim."

Thermal Insulation Installation

Install polyethylene vapor retarder if required. Install board insulation if required, in compliance with installation requirements in Division 07 Section "Thermal Insulation."

Install blanket insulation if required, in compliance with installation requirements in Division 07 Section "Thermal Insulation."

Metal Roof Panel Installation

Verify that site conditions are acceptable for installation. Do not proceed with installation until unacceptable conditions are corrected. Comply with panel manufacturer’s installation instructions including but not limited to special techniques, interface with other work, and integration of systems. Fasten metal roof panels to supports with concealed clips at each standing-seam joint at location, spacing, and using proper fasteners as recommended by panel manufacturer. Comply with installation tolerances as required.

Accessory Installation

Install accessories using techniques recommended by manufacturer and which will assure positive anchorage to building and weather tight mounting. Provide for thermal movement. Coordinate installation with flashings and other components. For Flashing and Trim, comply with performance requirements, manufacturer's written installation instructions, and the SMACNA "Architectural Sheet Metal Manual." Provide concealed fasteners where possible, and install units to true level. Install work with laps, joints, and seams that will be permanently watertight.

Field Quality Control

If requested by Owner, provide manufacturer’s field service consisting of product use recommendations and periodic site visits for inspection of product installation in accordance with manufacturer’s instructions.

Precautions, Cleaning and Protection

Touch-up paint is used to cover and protect unexpected scratches on the paint finish that may occur during installation of panel. Touch-up paint will not weather as well or at the same rate as the original system. Test in an area that will not be noticeable. Metallic paint colors are available at an additional charge. Minor differences in color and appearance are normal and to be expected.

To minimize possible differences in appearance, an entire project should be painted at one time, from one batch of paint, using the same application equipment. Additionally, fabricated panels, flat sheet, and flashings should be oriented in the same direction.

After installation remove temporary coverings and protection of adjacent work areas. Repair or replace any installed products that have been damaged. Clean installed panels in accordance with manufacturer’s instructions prior to Owner’s acceptance. Remove and lawfully dispose of construction debris from Project site. Protect installed product and finish surfaces from damage during construction.

Building Codes

Current data on building code requirements and product compliance may be obtained from Metal Sales Corporation technical support specialists. Installation must comply with the requirements of authority having jurisdiction.

6. AVAILABILITY AND COST

Availability

Metal Sales Manufacturing Corporation products are nationally distributed and supported from 21 convenient locations nationwide, including Alaska. Manufacturer has the ability to ship worldwide. Contact manufacturer for information on local availability.

Cost

Budget installed cost information may be obtained from a local Metal Sales distributor or through the manufacturer.

7. WARRANTY

Coordinate with Conditions of the Contract and with Division 01 Closeout Submittals (Warranty) Section. Product is available in two coatings, 70 percent fluorocarbon PVDF (polyvinylidene fluoride) and SMP (silicone-modified polyester).

Information below applies when PVDF coating warranty is required.

Special Exposed Panel Finish Warranty: Manufacturer's standard 70 percent polyvinylidene fluoride (Kynar 500®) PVDF limited warranty for film integrity, chalk rating and fade rating in which manufacturer agrees to repair or replace panels that show evidence of deterioration within specified warranty period.

Deterioration shall include but is not limited to:

a. Chipping, cracking, checking or peeling of paint.

b. Chalking in excess of a numerical 8 rating when tested according to ASTM D 4214.

c. Color fading of more than 5 E units when calculated according to ASTM D 2244.

Warranty Period: 30 years from date of Substantial Completion. Manufacturer’s warranty excludes surface deterioration due to physical damage, exposure to corrosive and aggressive atmospheres and salt air environments.

Information below applies when SMP coating warranty is required.

Special Exposed Panel Finish Warranty: Manufacturer's standard form silicone modified polyester (SMP) warranty for film integrity, chalk rating and fade rating in which manufacturer agrees to repair or replace panels that show evidence of deterioration within specified warranty period.

Deterioration shall include but is not limited to:

a. Chipping, cracking, checking or peeling of paint.

b. Chalking in excess of a numerical 8 rating when tested according to ASTM D 4214.

c. Color fading of more than 7 E units when calculated according to ASTM D 2244.

Warranty Period: 20 years from date of Substantial Completion. Manufacturer’s warranty excludes surface deterioration due to physical damage, exposure to corrosive and aggressive atmospheres and salt air environments.

8. MAINTENANCE

No specific maintenance is required for properly installed Metal Sales panel products. Periodic inspection to verify system integrity, drainage functionality and repair of storm damage is advised.

9. TECHNICAL SERVICES

Technical assistance, including more detailed information, product literature, test results, project lists, assistance in preparing project specifications and arrangements for application supervision, is available by contacting Metal Sales Manufacturing Corporation.

10. FILING SYSTEMS

Additional product information is available from the manufacturer upon request. Trademarks include IMPower Series™ owned by Metal Sales Manufacturing Corporation, and Kynar 500® owned by Arkema, Inc.