SUGGESTED MASTER SPECIFICATION

Square Dowel Basket Assembly

PART 1 - GENERAL

1.1 	SUMMARY

A.	Products supplied under this section:  Square dowel basket assemblies.

B.	Related sections:

1.	Section 03200 - Concrete reinforcement.

2.	Section 03300 - Cast-in-place concrete.

1.2	REFERENCES

A.	American Concrete Institute (ACI):

1.	ACI Detailing Manual SP-66.

2.	ACI 302.1R-04 Guide for Concrete Floor and Slab Construction.

3.	ACI 360R-06 Guide for Design of Slabs-on-Ground.

B.	American Society for Testing and Materials International (ASTM):

1.	A 36 Standard Specifications for Carbon Structural Steel.

2.	A 108 Standard Specifications for Steel Bar, Carbon and Alloy, Cold Finished.

1.3	SUBMITTALS

A.	Product data: Manufacturer’s product data with application and installation instructions for proprietary materials and items, including but not limited to square dowel basket assemblies.

B.	Shop drawings:  Indicate placement of square dowel basket assemblies.

1.	Indicate dimensions and spacings.

2.	Comply with ACI 302.1R-04, ACI 360R-06, ACI Detailing Manual (SP-66) and PNA installation guides indicating arrangement of square dowel basket assemblies.

1.4 	QUALITY ASSURANCE

A.	Pre-installation meeting:

1.	Convene a pre-installation meeting ** two (2) ** ____ ** weeks before installation of PNA products.  Require attendance of parties affecting the work of this section, including Contractor, Engineer, and installers.

2.	Review installation procedures and coordinate with other work.

1.5	DELIVERY, STORAGE AND HANDLING

A.	Deliver square dowel basket assemblies in manufacturer’s packaging.

B.	Deliver and handle materials to prevent damage or weakening to baskets.

C.	Prevent accumulation of rust, debris or deleterious materials on baskets during storing.  Store off ground and under cover.

D.	Do not cut shipping wires on baskets.

PART 2 - PRODUCTS

2.1	MANUFACTURER

A.	Acceptable manufacturer: PNA Construction Technologies, www.pna-inc.com; 800-542-0214.

2.2	SQUARE DOWEL BASKET ASSEMBLIES

A.	Acceptable product: PNA Square Basket Assembly.

B.	Material:

1.	Square dowels: Saw cut from hot rolled steel bar meeting ASTM A 36 to within 3/16" of specified length.

2.	Square dowel clip: Clip shall contain compressible foam on each vertical side to allow limited horizontal movement parallel and perpendicular to joint.  Length shall be +/- 3/16" of half total dowel length + 2”.

3.	Side frame supports: 1/4" diameter cold drawn wire meeting ASTM A 108, Grade 1010-1020.

B.	Fabrication:

1.	Weld dowels (on one end only) into side frames, with welded ends alternating along length of assembly.

2.	Weld eight gauge wires across side frames at 3'-0" on center to keep assembly stable during shipping and installation.

3.	Fit square dowel clips to free end of dowel and extend to at least 2" past center of dowel.

4.	Finished assembly shall hold dowels to within +/- 1/8" of slab depth.

SLAB DEPTH AND NUMBER OF BARS SHALL BE DETERMINED BASED ON JOINT SPACING PER SHOP DRAWINGS.

5.	Dowel cross section of **½” to **1 ¼” and length from **12” to **36”.  ** (Fill in appropriate dimensions.) **

PART 3 - EXECUTION

3.1  	EXAMINATION

A.	Notify Engineer at least 72 hours prior to placing concrete dowel systems to allow time for examination of baskets.  Place no concrete in forms without Engineer’s acceptance of concrete systems.

3.2 	INSTALLATION

A.	Install square dowel basket assemblies in accordance with manufacturer’s product data and in accordance with shop drawings.

B.	Square dowel basket assembly installation:

1.	Do not cut shipping wires.

2.	Measure and mark (notch or nail) saw-cut joint location on forms.

3.	Pull string lines.  Spray paint saw-cut locations on subgrade.

4.	Lay-out one row of baskets over painted marks on subgrade.  Keep plates a minimum of 12" away from intersection of two joints.

5.	Place concrete.  Maintain alignment of baskets over paint marks.

6.	Screed and finish concrete, using internal vibration as necessary, in accord with ACI guidelines.

7.	Use markings or nails on forms to snap chalk-lines and saw-cut joints as normal to specified depth.

3.3	PROTECTION

A.	Protect square dowel basket assemblies from displacement and from damage until concrete casting is complete.

END OF SECTION