Sure Klean® Asphalt & Tar Remover Cleaning Specification

Specifier Note:  The information provided below is intended to guide the Architect in developing specifications for products manufactured by PROSOCO, Inc. and should not be viewed as a complete source of information about the product(s).  The Architect should always refer to the Product Data Sheet and MSDS for additional recommendations and for safety information

Specifier Note:  Paragraph below is for PART 1 GENERAL, Quality Assurance.

# Test Area

Test a minimum 4 ft. by 4 ft. area on each type of masonry. Use manufacturer’s application instructions. Let the test panel dry 3 to 7 days before inspection. Keep test panels available for comparison throughout the cleaning project.

Specifier Note:  Paragraphs below are for PART 2 PRODUCTS, Manufacturers and Products.

# Manufacturer

PROSOCO, Inc., 3741 Greenway Circle, Lawrence, KS 66046.  Phone: (800) 255-4255; Fax: (785) 830-9797.  E-mail:  CustomerCare@prosoco.com

# Product Description

Sure Klean® Asphalt & Tar Remover is a highly efficient multipurpose solvent cleaner for masonry and concrete. It is easy to apply and water rinsable. Ideal for cleaning and surface preparation, Asphalt & Tar Remover will not damage most masonry or concrete and does not contain methylene chloride or methanol. It removes asphalt, tar (roofer’s stain), grease, hydraulic oil and motor oil.

# Typical Technical Data

FORM: Colorless, clear liquid

SPECIFIC GRAVITY: 0.84

pH: not applicable

WEIGHT/GALLON: 6.96 pounds

ACTIVE CONTENT: not applicable

TOTAL SOLIDS: not applicable

VOC CONTENT: not applicable

FLASH POINT: 130 degrees Fahrenheit (54 degrees Celsius) ASTM D 3278

FREEZE POINT.: less than -22 degrees Fahrenheit (less than -30 degrees Celsius)

SHELF LIFE: 2 years in tightly sealed, unopened container

# Limitations

- May damage some synthetic or reflective glass.

- Asphalt & Tar Remover requires thorough rinsing for effective use and results. It is most effective for exterior application where rinse water can be used freely and where adequate ventilation is ensured. For interior applications, use Enviro Klean® Safety Peel 1 or SafStrip® 8.

Specifier Note:  Paragraphs below are for PART 3 EXECUTION, Installation.

# Application

Before applying, read “Preparation” and “Safety Information” sections in the Manufacturer’s Product Data Sheet for Asphalt & Tar Remover.  Do not dilute.

- Apply diluted cleaning solution liberally to the dry surface using low-pressure (40 psi) spray, or a brush.

- Let the cleaner dwell for 3 to 10 minutes. Keep people away from treated surfaces.

- Reapply cleaner. Brush the stained area vigorously.

- Rinse thoroughly with water to remove residue. Keep the wall below wet and rinsed free of cleaner and residues. This helps avoid streaking.

- Repeat steps 1 through 4 if necessary to fully remove any remaining residues.

# Cleanup

Clean tools and equipment using fresh water.

SPECIFIER NOTE: The best combination of rinsing pressure and water volume is provided by masonry washing equipment generating 400 to 1000 psi with a water flow rate of 6 to 8 gallons per minute delivered through a 15 to 45 degree fan spray tip. Equipment should be adjustable to reduce water flow rate and rinsing pressure as required for controlled cleaning of more sensitive surfaces. See also “Equipment” section of the Product Data Sheet.

SPECIFIER NOTE: Pressure washing is the preferred method to minimize waste materials. Rinse from bottom to top.