PROSOCO® Oil & Grease Stain Remover Cleaning Specification

Specifier Note: The information provided below is intended to guide the Architect in developing specifications for products manufactured by PROSOCO, Inc. and should not be viewed as a complete source of information about the product(s). The Architect should always refer to the Product Data Sheet and MSDS for additional recommendations and for safety information.

Specifier Note: Paragraph below is for PART 1 GENERAL, Quality Assurance.

# Test Area

Test a minimum 4 ft. by 4 ft. area on each type of masonry. Use manufacturer’s application instructions. Let the test panel dry 3 to 7 days before inspection. Keep test panels available for comparison throughout the cleaning project.

Specifier Note: Paragraphs below are for PART 2 PRODUCTS, Manufacturers and Products.

# Manufacturer

PROSOCO, Inc., 3741 Greenway Circle, Lawrence, KS 66046. Phone: (800) 255-4255; Fax: (785) 830-9797. E-mail: CustomerCare@prosoco.com

# Product Description

PROSOCO® Oil & Grease Stain Remover is a poultice cleaner for pulling stubborn oil and grease stains out of pavers, concrete, stone and other porous surfaces. Pour Oil & Grease Stain Remover on the embedded stain, and it spreads out to make an instant poultice – no mixing, troweling, plastic covering or tape needed.

As the poultice dries, it pulls the contaminants out of the substrate, capturing them in its highly absorbent blend of batter-thick surfactants, solvents and powders. After a 5 to 8 hour dwell, simply sweep up the dried poultice and its absorbed contaminants for easy disposal.

# Typical Technical Data

FORM: Viscous, white liquid, slight citrus fragrance

SPECIFIC GRAVITY: 1.05

pH: 10.5

WEIGHT/GALLON: 8.75 pounds

ACTIVE CONTENT: not applicable

TOTAL SOLIDS: not applicable

VOC CONTENT: 4 percent, maximum

FLASH POINT: greater than 200 degrees Fahrenheit (greater than 93 degrees Celsius) ASTM D 3278

FREEZE POINT: 32 degrees Fahrenheit (0 degrees Celsius)

SHELF LIFE: 2 years in tightly sealed, unopened container

# Limitations

- Not recommended for asphaltic surfaces

Specifier Note: Paragraphs below are for PART 3 EXECUTION, Installation.

# Application

Before applying, read “Preparation” and “Safety Information” sections in the Manufacturer’s Product Data Sheet and the Safety Data Sheet for Oil & Grease Stain Remover. Use in concentrate. Do not dilute or alter, or use for applications other than as specified.

# Application Instructions

1.	Scrape debris and buildup from surface. Do not pre-wet.

2.	Shake product container well. Pour onto the stained surface until it completely covers the stain. Pour onto the stained surface until it exceeds the boundaries of the stain by at least one inch.

3.	Let the poultice dry for 5 to 8 hours overnight. Protect from pedestrian traffic and rain during dwell period. Dry times may vary with environmental conditions such as temperature, wind and humidity. The poultice surface develops cracks as it contracts and dries. A fully developed network of cracks indicates the poultice is completely dry.

4.	Sweep up the dry powder and dispose. On porous substrates, pressure water rinsing may facilitate removal of poultice residue.

SPECIFIER NOTE: The batter-thick poultice will automatically reach its required thickness as it spreads out across the stain.

SPECIFIER NOTE: Most oil and grease stains can be cleaned with one application. Severe, deeply-embedded stains may continue rising to the surface following cleaning. If stains reoccur, reapply.

SPECIFIER NOTE: Regarding rain protection, a plastic “tent” over the poultice will allow proper drying. Avoid using tightly adhered plastic covers.

# Clean-up

Brush-out brooms and wipe down dustpans with a damp cloth. High-pressure air may also be used to remove dust residues from brooms, brushes and dustpans.