STO GUIDE SPECIFICATION

SPECIFICATION CR512 Sto Clear Coat Sealer

Clear, Acrylic-based, Weatherproof Topcoat Sealer

SECTION 09980

CLEARCOAT SEALER

# PART 1 - GENERAL

## SUMMARY

##### Provide clear weatherproof sealer for architectural finishes and horizontal floor treatments.

##### Related Sections:  Other specification sections which relate directly to the work of this section include the following:

###### Section 03300, Cast-In-Place Concrete.

## SUBMITTALS

##### Product Data:  Submit manufacturer's product data and installation instructions for each material and product used.  Include manufacturer's Material Safety Data Sheets.

## QUALITY ASSURANCE

##### Manufacturer's Qualifications: The manufacturer shall be a company with at least five years experience and regularly engaged in the manufacture and marketing of products specified herein.  The manufacturer shall have an ISO 9001 certified quality system.

##### Installer's Qualifications: The contractor shall be qualified to perform the work specified by reason of experience.

## DELIVERY, STORAGE AND HANDLING

##### Deliver products in original packaging, labeled with product identification, manufacturer, batch number and shelf life.

##### Store products in a dry area with temperature maintained between 50 and 85 degrees F (10 and 29 degrees C).  Protect from direct sunlight.  Protect from freezing.

##### Handle products in accordance with manufacturer's printed recommendations.

# PART 2 - PRODUCTS

### MATERIALS

##### Acrylic-based clear topcoat sealer for use on horizontal and vertical surfaces.  Comply with the following:

###### Manufacturer:  CR512 Sto Clear Coat Sealer, as manufactured by Sto Corp., PO Box 44609, Atlanta, GA, 30336-5609; telephone 404-346-3666.

# PART 3 - EXECUTION

#### INSTALLATION OF STO CLEAR COAT SEALER

##### Examine substrates and conditions under which materials will be installed.  Do not proceed with installation until unsatisfactory conditions are corrected.

##### Coordinate installation with adjacent work to ensure proper sequence of construction.  Protect adjacent areas and landscaping from contact due to mixing and handling of materials.

###### Surface Preparation: Clean surface to be treated using a low pressure (<1000 psi (<6.9 MPa)) water wash.

##### Mixing:

###### Use material undiluted.

###### Mix thoroughly with a drill and paddle immediately before use.

##### Application:

###### Apply when ambient and surface temperatures are 50 degrees F (10 degrees C) and rising and will remain above 50 degrees F (10 degrees C) for 24 hours.

###### Do not apply in freezing conditions or during precipitation.

###### Apply using a low-pressure sprayer, roller, or brush.

###### Apply in two coats, 5 to 10 wet mils (0.13 to 0.25 mm) per coat.

###### Allow sealer to dry between coats (normally 1 to 2 hours at 70-75 degrees F (21-24 degrees C)).

###### Protect from rain until completely dry (24 to 48 hours, depending on temperature and humidity).

###### Protect from foot traffic for a minimum of 6 hours.

###### Protect from vehicular traffic until completely dry.

##### Cleaning:  Clean tools and equipment with water immediately after use.

END OF SECTION

Copyright 2000 Sto Corporation.  All Rights Reserved.

A copyright license to reproduce this specification is granted to specifying architects and engineers.

ATTENTION

Sto products are intended for use by qualified professional contractors, not consumers, as a component of a larger construction assembly as specified by a qualified design professional, general contractor or builder.  They should be installed in accordance with those specifications and Sto’s instructions.  Sto Corp. disclaims all, and assumes no, liability for on-site inspections, for its products applied improperly, or by unqualified persons or entities, or as part of an improperly designed or constructed building, for the nonperformance of adjacent building components or assemblies, or for other construction activities beyond Sto’s control.  Improper use of Sto products or use as part of an improperly designed or constructed larger assembly or building may result in serious damage to Sto products, and to the structure of the building or its components.  STO CORP. DISCLAIMS ALL WARRANTIES EXPRESS OR IMPLIED EXCEPT FOR EXPLICIT LIMITED WRITTEN WARRANTIES ISSUED TO AND ACCEPTED BY BUILDING OWNERS IN ACCORDANCE WITH STO’S WARRANTY PROGRAMS WHICH ARE SUBJECT TO CHANGE FROM TIME TO TIME.  For the fullest, most current information on proper application, clean-up, mixing and other specifications and warranties, cautions and disclaimers, please refer to the Sto Corp. website, www.stocorp.com.