SECTION 09 54 23

USG CEILINGS PLUS BARZ - OPEN PLENUM MODULAR ACOUSTIC SYSTEM

PART 1  GENERAL

SECTION INCLUDES

Linear metal ceilings.

Suspended metal support system and perimeter trim.

The paragraph below is optional text

Supplementary acoustical insulation over system units.

RELATED REQUIREMENTS

The paragraph below is optional text

Section 016116 - Volatile Organic Compound (VOC) Content Restrictions.

The paragraph below is optional text

Section 031000 - Concrete Forms and Accessories:  Execution requirements for placement of attachment anchors to structure above.

The paragraph below is optional text

Section 033000 - Cast-in-Place Concrete:  Execution requirements for placement of attachment anchors to structure above.

The paragraph below is optional text

Section 053100 - Steel Decking:  Execution requirements for placement of attachment anchors to structure above.

The paragraph below is optional text

Section 072100 - Thermal Insulation.

The paragraph below is optional text

Section 083100 - Access Doors and Panels:  Access panels.

The paragraph below is optional text

Section 092116 - Gypsum Board Assemblies - USG:  Gypsum board and metal framing products

The paragraph below is optional text

Section 095100 - Acoustical Ceilings - USG:  Metal suspension system.

The paragraph below is optional text

Section 211300 - Fire-Suppression Sprinkler Systems:  Sprinkler heads.

The paragraph below is optional text

Section 233700 - Air Outlets and Inlets:  Air diffusers.

The paragraph below is optional text

Section 265100 - Interior Lighting:  Luminaires.

The paragraph below is optional text

Section 265600 - Exterior Lighting:  Luminaires.

The paragraph below is optional text

Section 275116 - Public Address Systems:  Audio speakers.

The paragraph below is optional text

Section 284600 - Fire Detection and Alarm:  Fire detection and alarm components in ceiling.

REFERENCE STANDARDS

ASCE 7 - Minimum Design Loads and Associated Criteria for Buildings and Other Structures; Most Recent Edition Cited by Referring Code or Reference Standard.

ASTM A492 - Standard Specification for Stainless Steel Rope Wire; 1995 (Reapproved 2019).

ASTM A580/A580M - Standard Specification for Stainless Steel Wire; 2018.

ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process; 2022.

ASTM A666 - Standard Specification for Annealed or Cold-Worked Austenitic Stainless Steel Sheet, Strip, Plate, and Flat Bar; 2023.

ASTM B209/B209M - Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate; 2021a.

ASTM C423 - Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method; 2022.

ASTM C635/C635M - Standard Specification for Manufacture, Performance, and Testing of Metal Suspension Systems for Acoustical Tile and Lay-in Panel Ceilings; 2022.

ASTM C636/C636M - Standard Practice for Installation of Metal Ceiling Suspension Systems for Acoustical Tile and Lay-In Panels; 2019.

ASTM C665 - Standard Specification for Mineral-Fiber Blanket Thermal Insulation for Light Frame Construction and Manufactured Housing; 2017.

ASTM E84 - Standard Test Method for Surface Burning Characteristics of Building Materials; 2023.

ASTM E90 - Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements; 2009 (Reapproved 2016).

ASTM E413 - Classification for Rating Sound Insulation; 2022.

ASTM E580/E580M - Standard Practice for Installation of Ceiling Suspension Systems for Acoustical Tile and Lay-in Panels in Areas Subject to Earthquake Ground Motions; 2022.

ADMINISTRATIVE REQUIREMENTS

Coordination:  Coordinate work of this section with installation of mechanical and electrical components and with other construction activities affected by work of this section.

Preinstallation Meeting:  Convene one week before starting work of this section.

Sequencing:  Supply hanger clips during steel deck erection.  Supply additional hangers and inserts as required.

SUBMITTALS

Product Data:  Furnish for component profiles.

Shop Drawings:  Indicate reflected ceiling plan.

Seismic Design:  Include seal and signature of design professional on each drawing.

Samples:  Two samples [___] by [___] inch ([___] by [___] mm) in size showing color and finish of exposed to view components.

The paragraph below is optional text

Designer's qualification statement.

The paragraph below is optional text

Manufacturer's qualification statement.

Installer's qualification statement.

Maintenance Materials:  Furnish the following for Owner's use in maintenance of project.

See Section 016000 - Product Requirements for additional provisions.

Extra Linear Panels:  One, standard length.

QUALITY ASSURANCE

The paragraph below is optional text

Designer Qualifications for Seismic Design:  Perform under direct supervision of a Professional Structural Engineer experienced in design of this work and licensed at the State in which the Project is located.

Manufacturer Qualifications:  Company specializing in manufacturing the Products specified in this section with minimum three years documented experience.

Installer Qualifications:  Company specializing in performing the work of this section.

Minimum [__________] years documented experience.

Approved by metal ceiling manufacturer.

The paragraph below is optional text

Products Requiring Electrical Connection:  Listed and classified by Underwriters Laboratories Inc.

MOCK-UP

The paragraph below is optional text

Construct [________] mock-up, [____] feet ([____] m) long by [____] feet ([____] m) wide; include suspension system, panels, closures in mock-up.

The paragraph below is optional text

See Section 014000 - Quality Requirements for additional requirements.

The paragraph below is optional text

Locate mock-up where directed.

The paragraph below is optional text

Mock-up may remain as part of the Work.

DELIVERY, STORAGE, AND HANDLING

Accept factory-finished products on site in manufacturer's unopened factory packaging only; reject opened packages.

Protect factory-finished products from damage to appearance by storing products in manufacturer's unopened factory packaging in dry storage area.

WARRANTY

See Section 017800 - Closeout Submittals for additional warranty requirements.

Provide five year manufacturer warranty; include coverage for corrosion resistance and discoloration of surface finish.

PART 2  PRODUCTS

LINEAR METAL CEILING ASSEMBLIES

Linear Metal Baffle Assembly LMBA-1, Item No. [_____]:

Baffle Assemblies: Barz Design Solutions.

Layout:  As indicated on drawings.

Finish Type:

Saranté Colors:  [S11 Crème Ovang] [S21 Blond Teak] [S31 Golden Oak] [S12N Valley Maple] [S32 CP Maple] [S33N2 Honey Anigre] [S23N Golden Birch] [S13 Red Birch] [S25 Natural Ovang] [S34 Cherry Anigre] [S14N Cinnamon Cherry] [S22 Oak Line] [S24N Grey Cedar] [S35N2 Cherry Birch] [S15 Blond Pear] [S37 Dark Jatoba] [S36N European Cherry] [S16N Tan Sawn Oak] [S17 Dark Oak] [S27 Forest Walnut] [S26 Earth Rosewood] [S18 Sable Walnut] [S38 Natural Walnut].

Arboreal Veneers: [Maple] [VG Fir] [White Oak] [Cherry] [Mahogany] [Walnut].

Anodized – On Metal Colors: [Kryolite] [Grau] [Sateen]

Painted - On Metal Colors: [Standard Silver] [Blanco Mat] [Flat White] [Custom].

Standard Perforations Pattern:  Reference IC425 perforation guide for more information.

Parti Custom Multi-Panel Perforations:  On portions of linear metal ceilings indicated on drawings.

Suspension Grid:  Semi-concealed, with CP slotted main tees and DX cross tees.

DESIGN REQUIREMENTS

Design components to ensure light fixtures will not induce eccentric loads.  Where components may induce rotation of ceiling system components, provide stabilizing reinforcement.

PERFORMANCE REQUIREMENTS

Design for maximum deflection of 1/360 of span.

Design to support imposed loads of indicated elements without eccentric loading of supports. Where supported elements may induce rotation of ceiling system components, provide stabilizing reinforcement.

The paragraph below is optional text

Seismic Performance:  Ceiling systems designed to withstand the effects of earthquake motions determined according to ASCE 7 for Seismic Design Category D, E, or F and complying with the following:

The paragraph below is optional text

Local authorities having jurisdiction.

The paragraph below is optional text

ICC-ES Evaluation Report No. [__________].

Surface Burning Characteristics:  Flame spread index of  25, smoke developed index of  50, when tested in accordance with ASTM E84.

Acoustic Attenuation:  STC of [________] calculated in accordance with ASTM E413, based on tests conducted in accordance with ASTM E90, with insulation installed.

Sound Absorption Average (SAA):  [______], measured in accordance with ASTM C423 with insulation installed.

Noise Reduction Coefficient (NRC):  [______], measured in accordance with ASTM C423 with insulation installed.

The paragraph below is optional text

Systems Located Outside Building Envelope:

The paragraph below is optional text

Accommodate wind and suction loads and wind uplift without damage in accordance with applicable code.

The paragraph below is optional text

Accommodate wind and suction loads and wind uplift to resist [____] psf ([____] kPa) without damage.

The paragraph below is optional text

Thermal Resistance Value:  Total R-value of [____] (RSI-value of [____]) with insulation installed.

COMPONENT PRODUCTS

Linear Metal Baffle Assemblies:  Multiple baffle elements assembled into panels (cassettes) and attached to underside of suspension members.

Baffle Profiles:   [As indicated on drawings] [Manufacturer's standard].

Cassette Configuration:   [As indicated on drawings].

Spacing Between Cassettes:  [[_____] inches between cassettes ([_____] mm between cassettes)] [As indicated on drawings].

Material:  Aluminum sheet, ASTM B209/B209M.

The paragraph below is optional text

Perforations Pattern:  [__________].

Finishes:

Applied PVC-Free Laminate Finish:  Faux-Wood USG Ceilings Plus Saranté laminate.

The paragraph below is optional text

Color: S11 Crème Ovang

Color: S21 Blond Teak

Color: S31 Golden Oak

Color: S12N Valley Maple

Color: S32 CP Maple

Color: S33N2 Honey Anigre

Color: S23N Golden Birch

Color: S13 Red Birch

Color: S25 Natural Ovang

Color: S34 Cherry Anigre

Color: S14N Cinnamon Cherry

Color: S22 Oak Line

Color: S24N Grey Cedar

Color: S35N2 Cherry Birch

Color: S15 Blond Pear

Color: S37 Dark Jatoba

Color: S36N European Cherry

Color:S16N Tan Sawn Oak

Color: S17 Dark Oak

Color: S27 Forest Walnut

Color: S26 Earth Rosewood

Color: S18 Sable Walnut

Color: S38 Natural Walnut

Real Wood Veneer for interior only: USG Arboreal.

The paragraph below is optional text

Color: Maple

Color: VG Fir

Color: White Oak

Color: Cherry

Color: Mahogany

Color: Walnut

Anodized: On Metal: Manufacturer's standard colors.

The paragraph below is optional text

Color:  Kryolite

Color:  Grau

Color:  Sateen

Painted: On Metal: Manufacturer's standard colors.

The paragraph below is optional text

Color: Standard Silver

Color: Blanco Mat

Color: Flat White

Color:  Custom

Installation:  Design system to allow every cassette to provide access to ceiling plenum.  Panels designed for progressive access are not permitted.

Mounting Assemblies:  Manufacturer's standard backer channels attached to back of cassettes.

Mount heavy-duty torsion springs on backer channels to allow downward movement of baffles without potential for damage to baffle face or hinge assembly.  Do not attach springs directly to individual baffles.

Use the number of backer bars required to transfer the dead load of each cassette to the supporting grid within its structural capabilities.

The paragraph below is optional text

Sound-Absorptive Backer:  Manufacturer's standard “Ultrasorb” recycled cotton fiber material, factory-laminated to backside of the perforated panels in sufficient thickness to achieve specified NRC rating for the panels.

The paragraph below is optional text

Installation:  Fill-in; inside each baffle.

The paragraph below is optional text

Installation:  Lay-in; on top of each cassette.

The paragraph below is optional text

Thickness, Density, and Acoustical Performance:  [1 inch thick with density of 1.5 pcf, for NRC 0.75 (25.4 mm thick with density of 24 kg/cu m, for NRC 0.75)] [1 inch thick with density of 3.0 pcf, for NRC 0.80 (25.4 mm thick with density of 48 kg/cu m for NRC 0.80)] [1 inch thick with density of 6.0 pcf, for NRC 0.85 (2.54 mm thick with density of 96 kg/cu m, for NRC 0.85)] [1-1/2 inches thick with density of 1.5 pcf, for NRC 0.90 (38 mm thick with density of 24 kg/cu m, for NRC 0.90)] [2 inches thick with density of 4.0 pcf, for NRC 1.15 (51 mm thick with density of 48 kg/cu m, for NRC 1.15)] [[___] inches thick with density of [___] pcf for NRC [___] ([___] mm thick with density of [___] kg/cu m, for NRC [___])].

Products:

USG Corporation; Barz Design Solutions:  www.usg.com/ceilings/#sle.

Substitutions:  Not permitted.

Suspension Systems:

Metal Suspension Systems:  See Section 095100 - Acoustical Ceilings - USG.

Metal Suspension Systems - General:  Complying with ASTM C635/C635M; die cut and interlocking components, with [perimeter moldings] [hold down clips] [stabilizer bars] [seismic clips] [splices], as required.

Materials:

Steel Grid:  ASTM A653/A653M G30 coating, unless otherwise indicated.

Semi-Concealed Acoustical Suspension System:  Hot-dipped galvanized steel grid and cap.

The paragraph below is optional text

Application(s):  Seismic.

Structural Classification:  Heavy-duty, when tested in accordance with ASTM C635/C635M.

The paragraph below is optional text

Recycled Materials Content:  Classified as containing greater than 50% total recycled content.  Available for specific sizes and lengths.

Profile:  Tee; 15/16 inch (24 mm) face width.

Finish:  Baked enamel.

Color:  Black.

The paragraph below is optional text

Cassettes Installation:  Baffles installed from below by inserting torsion springs into slots in faces of main runners of ceiling grid.

Products:

USG Corporation;  15/16 Inch suspension system, with CP slotted main tees and DX cross tees:  www.usg.com/ceilings/#sle.

Substitutions:  Not permitted.

ACCESSORIES

Support Channels, Carriers, and Hangers:  [Galvanized] [Primed] steel; size and type to suit application[None - N/A] [, seismic requirements,] and ceiling system flatness requirement specified.

Suspension Wire[and Rope] [None - N/A]:  Size and type as required for application[, seismic requirements,] [None - N/A] and ceiling system flatness requirement specified.

Concealed Suspension:

Suspension Wire:  Steel, annealed, [galvanized] [plain] finish, [12 gage, 0.0808 (2.05 mm)] [9 gage, 0.1144 inch (2.91 mm)] [[___] gage, [___] inch ([___] mm)] diameter.

Exposed (To View) Suspension:

Suspension Wire:  Stainless steel, 18 gage, 0.0403 (1.02 mm) diameter, complying with ASTM A580/A580M.

Suspension Rope:  1/32 inch (0.8 mm) stainless steel rope wire complying with ASTM A492, with  [loop and crimp-end] [turnbuckle] [wire crimp] or  [loop and crimp-end] [turnbuckle] [wire crimp] connection.

The paragraph below is optional text

Seismic Clips:  Manufacturer's standard clips for seismic conditions and to suit application.

The paragraph below is optional text

Seismic Clips:  Galvanized Steel clips, furnish and install per Manufacturer's installation criteria to suit code compliant application.

Seismic Attachment Clip: Used to attach tees ends to perimeter angle for seismic design C D E F categories.

Available products: ACM7.

Molding Attachment Clip: 2 in. thick x 1/2 in. thick x 1-5/8 in.  Used to attach cross tees and main tees to walls/ wall molding.

Molding Attachment Clip: 9/16 in. thick for SQ panels or FL panels, Available products: MAC2.

Seismic Separation Joints: Galvanized Steel clips, furnish and install per Manufacturer's installation criteria to suit code compliant application.

Two Way Seismic Separation joint accessories: Tee sleeves to accommodate movement: Snap fit sleeve, prefinished to match suspension system components.

Seismic Grid Separation Clip: DH2 2-way seismic separation joint clip, attached to ceiling grid tees intersections [fixed with fasteners] [Unrestrained to accommodate grid tee movement] in a [directional] vector.

Four way Seismic Separation joint clip: Connects 4 way intersection: Snap fit sleeve with optional screw/fixing holes.

Seismic Grid Separation Clip: DH4 4-way seismic separation joint clip, attached to ceiling grid tees intersections [fixed with fasteners] [Unrestrained to accommodate grid tee movement] in a [directional] or [omnidirectional].

Miscellaneous Accessories:  Manufacturer's standard splice plates, filler strips, and perimeter trim required for complete installation of system.

The paragraph below is optional text

Edge Molding, Expansion Joints, and Splices:  Same material, thickness, and finish as linear panels.

The paragraph below is optional text

End Caps:  Formed metal; same color and finish as sight-exposed surfaces of linear panels.

The paragraph below is optional text

Metal Baffle Grid Attachment Clips:  Manufacturer's standard for attachment to main or cross tees.

Acoustical Insulation:  [Specified in Section{\\id\\#277}] [ASTM C665friction fit type, unfaced batts].

Thickness:  [2 inch (51 mm)] [6 inch (152 mm)] [[____] inch ([____] mm)].

Size:  To fit acoustical suspension system.

The paragraph below is optional text

Thermal Insulation:  Specified in Section 072100.

The paragraph below is optional text

Thermal Insulation:  ASTM C665,  preformed glass fiber batt; friction fit, complying with the following:

The paragraph below is optional text

Thermal Resistance:  R-value (RSI-value) of [____] ([____]).

The paragraph below is optional text

[Batt] [Roll] Size:  [___]  by [___] inch ([___]  by [___] mm).

The paragraph below is optional text

Facing:  Unfaced.

The paragraph below is optional text

Facing:  Faced on one side with [foil] [asphalt treated Kraft paper] [black plastic] [mesh reinforced Kraft paper] .

Gypsum Board and Framing Materials:  See Section 092116.

Touch-Up Paint for Exposed Surfaces:  Type and color to match linear panels and suspension system grid and trim elements.

Touch-Up Paint For Concealed Galvanized Items:  [Zinc rich] [Zinc oxide] type, as recommended by ceiling system manufacturer.

FABRICATION

Shop cut linear panels to accommodate mechanical and electrical items.

Factory-form internal and external corners of same material, thickness, finish, and profile to match exposed linear panels [None - N/A] [; back brace internal corners].

Fabricate components to allow access to ceiling plenum as required.

PART 3  EXECUTION

EXAMINATION

The paragraph below is optional text

Verify existing conditions before starting work.

The paragraph below is optional text

Verify that layout of hangers will not interfere with other work.

The paragraph below is optional text

Verify that field measurements are as indicated on shop drawings.

The paragraph below is optional text

Start of installation constitutes acceptance of project conditions.

PREPARATION

The paragraph below is optional text

Coordinate the location of hangers with other work.

The paragraph below is optional text

Provide hanger clips during steel deck erection.  Provide additional hangers and inserts as required.

The paragraph below is optional text

Install after major above-ceiling work is complete.

INSTALLATION - SUSPENSION SYSTEM

Install suspension system in accordance with ASTM C636/C636M and manufacturer's instructions and as supplemented by this section.

Install hangers and inserts coordinated with overhead work.  Provide additional hangers and supports as required.

Rigidly secure system, including integral mechanical and electrical components, for maximum deflection of 1:360.

Lay out system to a balanced grid design with edge units no less than 50 percent of acoustical unit size.

Locate system on room axis according to reflected ceiling plan.

Suspension System, Non-Seismic:  Hang suspension system independent of walls, columns, ducts, pipes and conduit.  Where carrying members are spliced, avoid visible displacement of face plane of adjacent members.

The paragraph below is optional text

Seismic Suspension System, Seismic Design Category C:  Hang suspension system independent of walls, columns, ducts, pipes and conduit.  Maintain a 3/8 inch (9 mm) clearance between grid ends and wall.

The paragraph below is optional text

Seismic Suspension System, Seismic Design Categories D, E, F:  Hang suspension system with grid ends attached to the perimeter molding on two adjacent walls; on opposite walls, maintain a 3/4 inch (19 mm) clearance between grid ends and wall.

Where ducts. facility services, or equipment prevent the regular spacing of hangers, reinforce the nearest affected hangers and related carrying channels to span the extra distance.

Do not support components on main runners or cross runners if weight causes total dead load to exceed deflection capability.

Support fixture loads using supplementary hangers located within 6 inches (152 mm) of each corner, or support components independently.

Do not eccentrically load system or induce rotation of runners.

Form expansion joints as detailed.  Form to accommodate plus or minus 1 inch (25 mm) movement.  Maintain visual closure.

Install unopposed tee attachment clips at appropriate locations to enable installation of acoustical units in an ashlar pattern.

Edge Moldings:  Install at intersection of ceiling and vertical surfaces and penetrations, using components of maximum length, set level. Provide edge moldings at junction with other ceiling finishes. Miter corners. Provide preformed edge closures to match bullnosed cornered partitions.

Install in bed of acoustical sealant.

Use longest practical lengths.

Overlap and rivet corners.

INSTALLATION - LINEAR METAL COMPONENTS

Install linear panels, baffles, and other system components in accordance with manufacturer's instructions.

Stagger end joints minimum 12 inches (300 mm).

Align end joints.

Butt interior end joints tight.

Set exterior end joints with 1/16 inch (2 mm) gap for expansion and contraction.

Provide expansion joints to accommodate plus or minus 1 inch (25 mm) movement and maintain visual closure.

Field miter corners at changes in panel direction.

Install filler strips between linear panels at interior locations.

Install edge moldings at junctions with other finishes and at vertical surfaces; use maximum piece lengths.

Where bullnose masonry units occur, install radiused closures to fit edge molding.

Install end caps at sight-exposed ends of linear panels.

Exercise care when site cutting sight-exposed finished components to ensure surface finish is not defaced.

Insulation:  Install above panel members; fit tight between grid members ; place insulation with facing side down.

TOLERANCES

Maximum Variation from Flat and Level Surface:  1/8 inch in 10 feet (3 mm in 3 m).

Maximum Variation from Plumb of Grid Members Caused by Eccentric Loads:  2 degrees.

Maximum Variation From Dimensioned Position:  1/4 inch (6 mm).

CLEANING

Clean polished surfaces.

Replace damaged or abraded components.

SCHEDULES

The paragraph below is optional text

Main Foyer:  Box beam design, no space closures, polished chrome finish; 9 feet (2.75 m) above finished floor; refer to reflected ceiling plan.

The paragraph below is optional text

Sloped Ceilings in Stair Wells:  Flat panel shape, bullnosed edge, acoustic insulation above, recessed black filler, flat white surface finish.

Disclaimer:  The USG Product Specifications contained herein are intended for use as product reference material by architects, engineers, other design professionals, contractors, building code officials, or other competent construction industry trade professionals having an interest in the selection, specification and use of products manufactured by the subsidiaries of USG Corporation. The specifications are intended solely as technical support incident to the sale and use of USG products and not intended to be a substitute for the design review and approval of the licensed design professionals for the project.  These materials may be printed and/or transferred electronically solely as needed by the user. Because electronic files can be modified by other parties, without notice or indication of such modifications, modification of USG Product Guide Specifications is the sole responsibility of the Design Professional.