SECTION 07 5X XX –USG Securock® Brand Glass-Mat Roof Board

Sustainability Notes:

- For Sustainability attributes, reference USG Securock® Brand Glass-Mat Roof Board Product Attribute Report (PAR) RF72.

Product Summary:

- Lightweight core; up to 18% lighter than competitive glass-mat roof boards.1 - Ideal for use as cover board in single-ply mechanically attached systems. - Provides protection to roof system from hail and foot traffic. - Fire-resistant for use as fire barrier and thermal barrier. Noncombustible Core per ASTM E136-12 (CAN/ULC-S114). - Fire Performance: Meets Factory Mutual (FM) Class 1 and Underwriters Laboratories (UL) Class A fire ratings for unlimited slope in fire barrier applications per UL 790. The 5/8" thick panel Meets the requirements of Type X per ASTM C1177 and may be used in P series designs as a thermal barrier. - UL Classified (Type SGMRX) as to Surface Burning Characteristics in accordance with ASTM E84 (CAN/ULC-S102). - Class A unlimited slope in accordance with UL790 (CAN/ULC-S107). - FM Approved, complies with requirements of FM 4450 and FM 4470 • Meets FM Class 1 - Scored a “10” for mold resistance on ASTM D3273. - For More information, reference product submittal sheet RF32 and roofing product information RF39. - <insert product representative here> or 800.874.4968 for technical questions.

Note to Specifiers:

- This document is not intended to function as a standalone specification. It is intended to assist the specifier in inserting the proper language into the following recommended specification sections: 07 5x xx – ROOFING, SUBSTRATE BOARD. - Refer to roof system manufacturer's written instructions, local code requirements and Factory Mutual Global (FMG) and/or Underwriters Laboratories (UL) requirements for proper installation techniques. - USG Securock® Glass-Mat Roof Board is engineered to perform within a properly designed roof system. The use of USG Securock® Glass-Mat Roof Board as a roofing component is the responsibility of the design professional. - Consult roofing manufacturers for specific instructions on the application of their products to USG Securock® Glass-Mat Roof Board. - Wind uplift (vertical pull) of the roof system as installed can be affected by many factors beyond USG's control, including moisture migrating into the roof assembly from inside or outside the building, proper fastener spacing, the quality of installation especially for fasteners and whether the framing has been properly designed and installed to meet strength and deflection criteria specified in the contract documents. For all these reasons, USG cannot guarantee the wind-uplift resistance (vertical pull) of any roof assembly or system containing USG roof boards.

2.2   SUBSTRATE BOARD, USG SECUROCK® GLASS MAT ROOF BOARD

        - Gypsum Board ASTM C 1177 “Standard Specification for Glass Mat Gypsum Water Resistant Gypsum Backing Panel”: Type X.

          - Basis of Design: Subject to compliance with project requirements, the design is based on the following: “USG Securock® Brand Glass-Mat Roof Board”.

          - UL Type Designation “SGMRX”.

          - ASTM D 3273: Mold Resistance: Score of 10 as rated according to ASTM D 3273.

          - ASTM E96: [16] [18].

          - ASTM E84 Surface-Burning Characteristics:

              - Flame Spread: 0.

              - Smoke Developed: 0.

          - ASTM E661 Flute Span-ability: [2 5/8” (66)] [5” (127)] [8” (203)].

          - ASTM C518 Permeance: [0.36] [0.53] [0.54].

          - Compressive Strength: 700-1000 psi.

          - Thickness: [1/4” (66.4)] [1/2” (12.7)] [5/8” (15.9)].

          - Length: [8’-0” (2438)].

          - Widths: 48" (1219).

          - Weight: [1.2] [1.7] [2.2] lb./ft2.

          - Bending Radius: [4’-0” (1219)] [6’-0” (1829)] [9’-0” (2743)].

          - Long Edges: Square.

Disclaimer:  The USG Product Specifications contained herein are intended for use as product reference material by architects, engineers, other design professionals, contractors, building code officials, or other competent construction industry trade professionals having an interest in the selection, specification and use of products manufactured by the subsidiaries of USG Corporation. The specifications are intended solely as technical support incident to the sale and use of USG products and not intended to be a substitute for the design review and approval of the licensed design professionals for the project.  These materials may be printed and/or transferred electronically solely as needed by the user. Because electronic files can be modified by other parties, without notice or indication of such modifications, modification of USG Product Guide Specifications is the sole responsibility of the Design Professional.