GUIDE SPECIFICATION FOR RED ROSIN PAPER:  MULTI-PURPOSE BUILDING PAPER

SECTION 07720

ROOF ACCESSORIES:  MULTI-PURPOSE BUILDING PAPER

Specifier Notes: This guide specification is written according to the Construction Specifications Institute (CSI) Format. The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project. Coordinate this section with other specification sections and the drawings.

Specifier Notes: W.R. Meadows SEALTIGHT RED ROSIN PAPER Multi-Purpose Building Paper is a single-ply sheathing paper manufactured from 100 percent recycled fibers set in alum to resist bleeding. RED ROSIN PAPER is useful in a wide variety of construction applications including roofing, flooring and as a general jobsite protective covering. When used as an inner liner in wall, roofing or wood flooring applications, RED ROSIN PAPER acts as a separation barrier, protecting against wind and dust infiltration while minimizing squeaks. It is compatible with coal tar and asphalt-based roofing systems.

PART 1 GENERAL

1.01	SECTION INCLUDES

A.	Application of multi-purpose building paper.

1.02	RELATED SECTIONS

Specifier Notes: Edit the list of related sections as required for the project. List other sections dealing with work directly related to this section.

A.	Section 07510 – Built-Up Bitiminous Roofing.

B.  	Section 09640 – Wood Flooring.

1.03	SUBMITTALS

A.	Comply with Section 01330 - Submittal Procedures.

B.	Submit manufacturer's product data and application instructions.

1.04	DELIVERY, STORAGE, AND HANDLING

A.	Deliver materials to site in manufacturer's original, unopened containers and packaging, with 	labels clearly identifying product name and manufacturer.

B.	Store materials in a clean dry area indoors in  accordance with manufacturer's instructions.

C.	Protect materials during handling and application to prevent damage.

PART 2	 PRODUCTS

2.01	MANUFACTURER

A.	W.R. Meadows, Inc., PO Box 338, Hampshire, Illinois 60140-0338. (800) 342-5976. (847) 		683-4500. Fax (847) 683-4544. Web Site www.wrmeadows.com.

2.02	MATERIALS

A.	Building Paper:  Multi-Purpose Building Paper, single ply sheathing paper manufactured from 100 percent recycled fibers set in alum to resist bleeding.

1.	Performance Based Specification:  Building paper shall have the 						following characteristics:

a.	Thickness: 9.0 to 11.5 mils.

b.	Weight: 3.0 to 3.4 pounds per 100 square feet (141 to 166 g/m2).

2.	Proprietary Based Specification:  Sealtight Red Rosin Paper multi-purpose building paper manufactured by W.R. Meadows.

PART 3 EXECUTION

3.01	EXAMINATION

A.	Examine surfaces to receive building paper. Notify Architect if surfaces are not acceptable. Do not begin surface preparation or application until unacceptable conditions have been corrected.

3.02	SURFACE PREPARATION

A.	Prepare surfaces to receive waterproofing in accordance with manufacturer's 				instructions.

B.	Clean surfaces of dirt, dust, and debris.

3.03	APPLICATION

A.	Install building paper in accordance with manufacturer's instructions at locations indicated on the drawings.

B.	Overlap each piece of building paper 2-1/2 inches (64 mm).

C.	Install subsequent applications once building paper is positioned.

3.04	PROTECTION

A.	Protect building paper from damage until covered with subsequent applications.

END OF SECTION